3GPP TSG-RAN WG4 Meeting #96-e 	R4-2011296

Online, 17 - 28 Aug 2020

Source: 	Huawei

Title: 	TP to TR 38.809: IAB Rx sensitivity

Agenda Item:	7.4.2.2.1

Document for:	Approval

# Introduction

The sub-clauses in the TS have been allocated to different companies to aid the drafting process, we have been allocated sensitivity and receiver dynamic range.

As per the new meeting rules as these are in the same agenda item they have been included in the same TP.

## 1.1	IAB-MT sensitivity

RX sensitivity for IAB-MT was mostly agreed in the last meeting WF (R4-2008785), the following agreements were made:

- FR2 EISREFSENS_50M declaration range for Local Area IAB-MT : -86 to -114dBm

- FR1 PREFSENS requirement for Local Area IAB-MT type 1-H : Reused from Local Area BS with NF =13dB, IM =2dB

- FR1 OTA sensitivity and OTA reference sensitivity level applicability for IAB-MT :

  - For IAB-MT type 1-H: OTA sensitivity requirement is specified for IAB-MT type 1-H

  - For IAB-MT type 1-O: Both OTA sensitivity and OTA REFSENS needs to be specified for 1-O

- FRC to be used by IAB-MT : Agreement: Selected UE FRC can be for IAB-MT based the same criteria as BS, companies are encouraged to bring FRC proposals in next RAN4 meeting.

The open issues around the FRC’s are discussed in discussion paper R4-200xxxx, in this TP the appropriated UL FRC’s from the UE specifications are renamed to match the BS criteria and defined in the Annex A. As there are many more types of FRC used in the UE specification they are not given unique designations in the same way as the BS it is difficult to reference them from the UE specifications, so for the IAB-MT FRC’s a similar designation has been given where the equivalent to the BS FRC G-FR1-A1-1 for the IAB-MT is G-FR1-A1-21. This numbering scheme can be changed if an alternative is deemed more suitable.

## 1.2	IAB-MT Rx dynamic range

There is currently no agreement on FR1 IAB-MT dynamic range.

It has been agreed (R4-2002483) that there will be no Rx dynamic range requirement for FR2 IAB-MT.

# TP to TR 38.809 v0.2.0

--- Start of changes ---

# References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.104: “NR; Base Station (BS) radio transmission and reception”

[3]	3GPP TS 38.101-1: “NR User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

[4]	3GPP TS 38.101-2: “NR User Equipment (UE) radio transmission and reception: Part 2: Range 2 Standalone”

[5]	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios "

[6]	3GPP TS 38.141-2: "NR; Base Station (BS) conformance testing; Part 2: Radiated conformance testing".

--- Next change ---

## 8.2	Reference sensitivity level

### 8.2.1	IAB-DU Reference sensitivity

The IAB-DU conducted reference sensitivity requirement is the same as the BS conducted sensitivity requirement.

## 8.2.2	IAB-MT Reference sensitivity

The IAB-MT uses similar assumptions for antenna architecture and gain as the IAB-DU, it is also assumed that the front end HW is similar and has the same NF.

As such the IAB-MT reference sensitivity will be derived using the same assumptions as the BS. The IAB-MT sensitivity is given by:

Where:

-	BW is the maximum transmission bandwidth for the FRC

-	NF is the noise figure

-	IM is the implementation margin.

-	SNR is the SNR value for which we reach 95% throughput. Each company provided simulation results, and average will be done for each BW.

The NF and the IM margin are hardware dependent and taken from the BS:

NF = 5dB for wide area IAB-MT and 13dB for local area IAB-MT

IM is 2dB

As the IAB-MT operated on the DL the FRC’s and the associated SNR requirements must be taken form the UE. Ther are many more FRC’s for the UE for each of the channel BW’s however it is sufficient to specify a limited number of FRC’s in the same way as the BS. For each BS FRC there is a UE FRC of the same transmission BW and hence these can be used for the IAB-MT as shown in table 8.2.2-1

Table 8.2.2-1: FRC’s for the FR1 IAB-MT

| BS Reference channel | equivalent UE reference channel (TS 38.101-1 [3], Annex A3.3) |
| --- | --- |
| G-FR1-A1-1 | Table A.3.3.2-1, 5MHz CBW |
| G-FR1-A1-2 | Table A.3.3.2-2, 5MHz CBW |
| G-FR1-A1-3 | Table A.3.3.2-3, 10MHz CBW |
| G-FR1-A1-4 | Table A.3.3.2-1, 20MHz CBW |
| G-FR1-A1-5 | Table A.3.3.2-2, 20MHz CBW |
| G-FR1-A1-6 | Table A.3.3.2-3, 20MHz CBW |

The SNR for the BS varies between -0.8dB to -1.2dB for each of the BS FRC’s, however the UE uses a figure of -1dB for all FRC’s. As the SNR is dependent on the modulation the UE figure is used.

Applying these number to the equation gives:

Table 8.2.2-2: NR Wide Area IAB-MT reference sensitivity levels

| IAB-MT channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel Ref TS 38.101-1 [3] Annex A | signal BW | IM | SNR | NF | IAB-MT reference sensitivity power level |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | dB | dB | dB | dBm |
| 5, 10, 15 | 15 | Table A.3.3.2-1, 5MHz CBW | 4.5 | 2 | -1 | 5 | -101.5 |
| 10, 15 | 30 | Table A.3.3.2-2, 5MHz CBW | 3.96 | 2 | -1 | 5 | -102.0 |
| 10, 15 | 60 | Table A.3.3.2-3, 10MHz CBW | 9.92 | 2 | -1 | 5 | -98.0 |
| 20, 25, 30, 40, 50 | 15 | Table A.3.3.2-1, 20MHz CBW | 19.08 | 2 | -1 | 5 | -95.2 |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 30 | Table A.3.3.2-2, 20MHz CBW | 18.36 | 2 | -1 | 5 | -95.4 |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 60 | Table A.3.3.2-3, 20MHz CBW | 17.28 | 2 | -1 | 5 | -95.6 |

Table 8.2.2-3: NR Local Area IAB-MT reference sensitivity levels

| IAB-MT channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel Ref TS 38.101-1 [3] Annex A | signal BW | IM | SNR | NF | IAB-MT reference sensitivity power level |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | dB | dB | dB | dBm |
| 5, 10, 15 | 15 | Table A.3.3.2-1, 5MHz CBW | 4.5 | 2 | -1 | 13 | -93.5 |
| 10, 15 | 30 | Table A.3.3.2-2, 5MHz CBW | 3.96 | 2 | -1 | 13 | -94.0 |
| 10, 15 | 60 | Table A.3.3.2-3, 10MHz CBW | 9.92 | 2 | -1 | 13 | -90.0 |
| 20, 25, 30, 40, 50 | 15 | Table A.3.3.2-1, 20MHz CBW | 19.08 | 2 | -1 | 13 | -87.2. |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 30 | Table A.3.3.2-2, 20MHz CBW | 18.36 | 2 | -1 | 13 | -87.4 |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 60 | Table A.3.3.2-3, 20MHz CBW | 17.28 | 2 | -1 | 13 | -87.6 |

--- Next change ---

## 10.2	OTA sensitivity

## 10.2.1	IAB-DU OTA sensitivity

The IAB-DU OTA sensitivity for IAB-DU type 1-H and 1-O is the same as the BS.

There is no OTA sensitivity requirement for IAB-DU type 2-O.

## 10.2.2	IAB-MT OTA sensitivity

As with the conducted reference sensitivity requirements as the IAB-MT antenna and receiver front end is similar to the BS the BS OTA sensitivity requirement is derived using the same assumptions as the BS.

The FR1 OTA sensitivity requirement is declared over a declared RoAoA without any boundaries and is used as both a minimum sensitivity requirement and  reference level for some of the interference requirements (ACS, in-band blocking, oob blocking, colocations blocking, RX IMD, in channel selectivity) as such the requirement is almost the same  as the BS requirement (with the same declarations required from TS 38.141-2 [6]). However as the IAB-MT receives the DL the FRC’s used are from the UE specification as described in table 8.2.2-1.

There is no OTA sensitivity requirement for IAB-MT type 2-O.

## 10.3	OTA reference sensitivity level

Detailed structure of the subclause is TBD.

## 10.3.1	IAB-DU OTA reference sensitivity

The IAB-DU OTA sensitivity for IAB-DU type 1-H, 2-O and 2-O is the same as the BS.

## 10.3.2	IAB-MT OTA reference sensitivity

#### 10.3.2.1	FR1

The IAB-MT antenna and front end is similar to that of a BS so the OTA reference sensitivity requirements will be based upon the same assumptions as the BS. As with the other IAB-MT sensitivity requirements however the FRC’s and the associated SNR requirement will be taken from the UE. The UE FRC’s will be of the same signal BW as the associated BS FRC’s for the requirement.

The FR1 OTA reference sensitivity level is calculated based on the required equivalent passive antenna gain if a receiver were to meet the conducted reference sensitivity requirements and cover a declared OTA reference sensitivity RoAoA (see TS 38.141-2 [6]).

The FR1 IAB-MT reference sensitivity therefore uses the same set of declarations as the BS, however the calculation is modified to correspond to the DL FRC’s and their associated SNR values. The OTA reference sensitivities are hence offset from the conducted reference sensitivity values given in tables 8.2.2-1 and 8.2.2-2 as shown in tables 10.3.2.1-1 and 10.3.2.1-2.

Table 10.3.2-1: Wide Area IAB-MT reference sensitivity levels

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel Ref TS 38.101-1 [3] Annex A | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 5, 10, 15 | 15 | Table A.3.3.2-1, 5MHz CBW | -101.5 - ΔOTAREFSENS |
| 10, 15 | 30 | Table A.3.3.2-2, 5MHz CBW | -102.0 - ΔOTAREFSENS |
| 10, 15 | 60 | Table A.3.3.2-3, 10MHz CBW | -98.0 - ΔOTAREFSENS |
| 20, 25, 30, 40, 50 | 15 | Table A.3.3.2-1, 20MHz CBW | -95.2 - ΔOTAREFSENS |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 30 | Table A.3.3.2-2, 20MHz CBW | -95.4 - ΔOTAREFSENS |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 60 | Table A.3.3.2-3, 20MHz CBW | -95.6 - ΔOTAREFSENS |

Table 10.3.2-2: Local Area IAB-MT reference sensitivity levels

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel Ref TS 38.101-1 [3] Annex A | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 5, 10, 15 | 15 | Table A.3.3.2-1, 5MHz CBW | -93.5 - ΔOTAREFSENS |
| 10, 15 | 30 | Table A.3.3.2-2, 5MHz CBW | -94.0 - ΔOTAREFSENS |
| 10, 15 | 60 | Table A.3.3.2-3, 10MHz CBW | -90.0 - ΔOTAREFSENS |
| 20, 25, 30, 40, 50 | 15 | Table A.3.3.2-1, 20MHz CBW | -87.2 - ΔOTAREFSENS |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 30 | Table A.3.3.2-2, 20MHz CBW | -87.4 - ΔOTAREFSENS |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 60 | Table A.3.3.2-3, 20MHz CBW | -87.6 - ΔOTAREFSENS |

#### 10.3.2.1	FR2

The FR2 IAB-MT antenna and front end is similar to that of a BS so the OTA reference sensitivity requirements will be based upon the same assumptions as the BS.

The BS FR2 OTA reference sensitivity level is based on a declared EIS value within a specified range, the range is calculated based on the useful range of antenna gains expected for the BS class. And is calculated as follows:

Hence for a wide area BS

Where: BW is the noise BW of the FRC, NF is the noise figure, IM is implantation margin not related to antenna array, SNR is the required SNR for demodulation and G is the antenna gain and RF losses.

The expected gain range is the same as the BS, however as there is no medium range IAB-MT the local area gain range is expanded to cover the medium range also:

Table 10.3.2.1-1: G assumptions for calculating FR2 WA and LA OTA REFSENS range

| IAB-MT class | G |  |
| --- | --- | --- |
|  | 30 GHz (24.25 – 33.4 GHz) | 45GHz (37 – 52.6 GHz) |
| WA | 10 to 33 dBi | 12 to 35 dBi |
| LA | 0 to 28 dBi | 2 to 30 dBi |

The FRC’s and the associated SNR values are also based on the Dl values used for the UE. There are UE FRC’s defined which have the same signal BW as the BS FRC’s as shown un table 10.3.2.1-2

Figure 10.3.2.1-2 FR2 equivalent IAB-MT FRC’s

| BS Reference channel | equivalent UE reference channel (TS 38.101-2, Annex A3.3) |
| --- | --- |
| G-FR2-A1-1 | Table A.3.3.2-1, 50MHz CBW |
| G-FR2-A1-2 | Table A.3.3.2-2, 50MHz CBW |
| G-FR2-A1-3 | Table A.3.3.2-3, 100MHz CBW |

The SNR for the BS FRCs range from -1.1 to -1.2 dB , the UE SNR assumption is -1dB, however as the declaration ranges and declared values are rounded to integer values this makes no difference to the final range. As such the FR2 IAB-MT range is the same as the BS, as follows:

For Wide Area IAB-MT, EISREFSENS_50M is an integer value in the range -96 to -119 dBm. The specific value is declared by the vendor.

For Local Area IAB-MT, EISREFSENS_50M is an integer value in the range -86 to -114 dBm. The specific value is declared by the vendor.

--- End of changes ---