3GPP TSG-RAN WG4 Meeting#96-e	R4- 2011424

Online, 17th Aug – 28th Aug, 2020

Source: 	Qualcomm Incorporated

Title: 	Views on FR2 DL 256QAM UE Demodulation Tests

Agenda item:	7.10.3.1

Document for:	Discussion

# Introduction

In WF [1], some issues were still open and different options were provided for open issues. In this paper, we discuss our views on open issues for FR2 DL 256QAM tests.

# Testable SNR for FR2

Using the Demod SNR range calculator spreadsheet in 38.810, we computed the testable SNR for Mode1 (with Noc) as presented in below observation assuming multi-band relaxation factor of 1.7dB in Cell K5 as per maximum value in Table 6.2.1.3-4 in 38.101-2.

Observation 1: Maximum testable SNR for FR2 band n260, 50MHz/120kHz under Mode 1 (used for fixed MCS, CQI reporting requirements) is 21.0 dB.

# Fixed MCS Requirements

In Table 1, we present the required SNR to achieve 70% of peak throughput for fixed MCS assuming 50MHz/120kHz. Requirement column is derived by adding 0.8dB IM (as agreed in Rel-15 FR1 256QAM tests) to impairment results.

Table 1: Required SNR for 50MHz/120kHz with 256QAM at 70% of peak throughput

| Case | Alignment SNR (dB) | Impairment SNR (dB) | Requirement SNR (dB) |
| --- | --- | --- | --- |
| AWGN, MCS20, Rank1 | 17.30 | 19.30 | 20.10 |
| TDLA30-300, MCS20, Rank1 | 19.54 | 21.54 | 22.34 |
| TDLD30-75, MCS20, Rank1 | 17.84 | 19.84 | 20.64 |
| TDLD30-35, MCS20, Rank1 | 17.85 | 19.85 | 20.65 |
| AWGN, MCS20, Rank2 | 20.59 | 22.59 | 23.39 |

For Rank 2, even AWGN channel is not testable. Therefore, considering the testable SNR limit, we propose the following.

Proposal 1: Define FR2 DL 256QAM fixed MCS requirements for TDLD30-75, MCS20, Rank1.

Proposal 2: Do not define FR2 DL 256QAM fixed MCS requirements for Rank 2.

# Conclusions

This paper discusses open issues for FR2 256QAM test cases. Following has been observed and proposed.

Observation 1: Maximum testable SNR for FR2 band n260, 50MHz/120kHz under Mode 1 (used for fixed MCS, CQI reporting requirements) is 21.0 dB.

Proposal 1: Define FR2 DL 256QAM fixed MCS requirements for TDLD30-75, MCS20, Rank1.

Proposal 2: Do not define FR2 DL 256QAM fixed MCS requirements for Rank 2.

# References

[1] R4-2008817, “WF on UE demodulation and CSI reporting requirements for FR2 DL 256QAM”, 3GPP TSG RAN WG4 Meeting #95-e, Online, May 25 - Jun 5, 2020.