3GPP TSG-RAN WG4 Meeting#96-e	R4- 2011436

Online, 17th Aug – 28th Aug, 2020

Source: 	Qualcomm Incorporated

Title: 	Views on NR CA PDSCH Demodulation Performance Tests

Agenda item:	7.16.1.1

Document for:	Discussion

# Introduction

In WF [1], there were still some open issues and different options were provided for different parameters. In this paper, we discuss our views on open issues in defining NR PDSCH demodulation performance tests for carrier aggregation.

# HARQ Timelines

In the previous meeting, different aspects were discussed to determine the HARQ timelines. In our opinion, initial transmission and retransmission should happen on the same type of slot. Otherwise, it will degrade the HARQ performance. Therefore, we propose the following.

Observation 1: Initial transmission and retransmission should happen on the same type of slot. Otherwise, it will degrade the HARQ performance.

Proposal 1: In case of TDD 15kHz + TDD 30kHz CA with TDD 15kHz as PCell, different RTTs (10 or 20 slots) are used for different HARQ processes, and initial transmission and retransmission are scheduled on the same type of TDD slot.

Proposal 2: In case of TDD 15kHz + TDD 30kHz CA with TDD 30kHz as PCell, use 8 HARQ processes.

If Pcell in both carriers are supported for TDD 15 kHz + TDD 30 kHz, our preference is to use TDD 30kHz as PCell since that is more widely deployed scenario. Also, performance does not change drastically with number of HARQ processes and RAN4 has already agreed to reuse single carrier performance for CA, so it should not matter from performance perspective. Therefore, we propose the following.

Proposal 3: If PCell in both carriers are supported, configure 30 kHz SCS cell as PCell in TDD 15kHz+30kHz SCS CA.

# Conclusions

This paper proposes parameters and configurations related to NR CA demodulation performance requirements. Following has been observed and proposed.

Observation 1: Initial transmission and retransmission should happen on the same type of slot. Otherwise, it will degrade the HARQ performance.

Proposal 1: In case of TDD 15kHz + TDD 30kHz CA with TDD 15kHz as PCell, different RTTs (10 or 20 slots) are used for different HARQ processes, and initial transmission and retransmission are scheduled on the same type of TDD slot.

Proposal 2: In case of TDD 15kHz + TDD 30kHz CA with TDD 30kHz as PCell, use 8 HARQ processes.

Proposal 3: If PCell in both carriers are supported, configure 30 kHz SCS cell as PCell in TDD 15kHz+30kHz SCS CA.

# References

[1] R4-2008838, “Way forward on PDSCH CA normal demodulation requirements”, 3GPP TSG RAN WG4 Meeting #95-e, Online, May 25 - Jun 5, 2020.