3GPP TSG-RAN WG4 Meeting #97-e                                                          R4-2014040

Electronic Meeting, 2 - 13 November 2020

Source:	Huawei, HiSilicon

Title:	TP to TR 37.717.31-11: Addition of DC configuration for DC_2-5-66_n7

Agenda item:	10.5.2

Document for:	Approval

# Background

This contribution provides text proposal on the EN-DC configuration DC_2-5-66_n7 as defined in WID on Dual Connectivity of 3 band LTE (3DL/1UL) and 1 NR band (1DL/1UL) [1].

# Text Proposal

##### ---Start of changes---

## 5.1.x	DC_2-5-66_n7

### 5.1.x.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band configurations EN-DC (four bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_2A-5A-66A_n7A DC_2A-5A-66A-66A_n7A | DC_2A_n7A DC_5A_n7A DC_66A_n7A |

### 5.1.x.2	∆TIB and ∆RIB values

For DC_2-5-66_n7, the TIB,c and RIB,c values are reused from the DC_2-7-13_n66, and are given in the tables below.

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5-66_n7 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 66 | 0.5 |
|  | n7 | 0.5 |

Table 7.3B.3.3.4-1: ΔRIB due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-5-66_n7 | 2 | 0.3 |
|  | 5 | 0 |
|  | 66 | 0.5 |
|  | n7 | 0.5 |

### 5.1.x.3	REFSENS requirements

No additional MSD requirement is needed.

##### ---End of changes---

# Reference

[1]	RP-20572, “Revised WID on Rel-17 Dual Connectivity (DC) of 3 bands LTE inter-band CA (3DL/1UL) and 1 NR band (1DL/1UL)”, Ericsson, RAN#89-e