3GPP TSG-RAN WG4 Meeting #97-e                                                    	R4-2014056

Online, 2nd Nov-13th Nov, 2020

Source:	Vodafone

Title:	TP for TR 37.717-21-11: DC_7-32_n78

Agenda item:			10.4

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.717-21-11 to update the reference sensitivity exceptions for DC_7-32_n78. Test points are proposed for B32 to account for the IMD3 and IMD4 impact of a DC_7_n78 UL configuration.

The IMD3 test point with 17.6 dB MSD is based on DC_3A-7A_n78A where the same IMD3 mixing product occurs with the same CBW. The IMD4 test point with MSD 4.9 dB is based on DC_3A-32A_n78A where a similar IMD4 impact is seen from DC_3_n78.

# 2. Reference

- RP-201369 DC of 2 bands LTE and 1 NR band

# 3. Text Proposal

<Start of Text Proposal>

### 5.9.4	Reference sensitivity exceptions

Table 5.x.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A-32A_n78A | n78 | 3560.5 | 10 | 50 | 3560.5 | N/A | N/A |
|  | 7 | 2517.5 | 5 | 25 | 2637.5 | N/A | N/A |
|  | 32 | N/A | 5 | N/A | 1474.5 | 17.6 | IMD3 |
|  | n78 | 3311 | 10 | 50 | 3311 | N/A | N/A |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | 32 | N/A | 5 | N/A | 1492 | 4.9 | IMD4 |

<End of Text Proposal>