3GPP TSG-RAN WG4 Meeting #97-e                                                         R4-2014106

Online, 2nd-13th Nov, 2020

Source:	Samsung, KDDI

Title:	TP for TR 37.717-21-11: DC_3-41_n3

Agenda item:			10.4.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.717-21-11 to include DC_3-41_n3 according to the request in [1].

# 2. Reference

- RP-201728, Revised WID on Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL).

# 3. Text Proposal

<Start of Text Proposal>

## 5.x	DC_3-41_n3

### 5.x.1	Configuration for DC

Table 5.x.1-1: Inter-band EN-DC configurations (three bands)

| DC configuration | Uplink configuration (NOTE 1) |
| --- | --- |
| DC_3A-41A_n3A DC_3A-41C_n3A | DC_3A_n3A2 DC_41A_n3A DC_41C_n3A |
| NOTE 2: Only single switched UL is supported |  |

### 5.x.2	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes. And it can be get:

- IMD4 of band 41 UL and band n3 UL falling to band 3 DL.

The MSD could reuse the value for DC_41A_n3A.

### 5.x.3	∆TIB and ∆RIB values

For DC_3-41_n3, the TIB,c and RIB,c values are given in the tables below. Numbers come from DC_41_n3.

Table 5.1.x.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-41_n3 | 3 | 0.5 |
|  | 41 | 0.31/0.82 |
|  | n3 | 0.5 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 5.1.x.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-41_n3 | 3 | 0 |
|  | 41 | 01/0.52 |
|  | n3 | 0 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

### 5.x.4	REFSENS requirements

Table 5.x.4-1 shows the required MSD levels for the DC configuration, its value can reuse the value of DC_3A_n41A.

Table 5.x.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-41A_n3A DC_3A-41C_n3A | 3 | 1770 | 5 | 25 | 1865 | 8.2 | IMD4 ¦2*fB41-2*fn3¦ |
|  | 41 | 2657.5 | 5 | 25 | 2657.5 | N/A | N/A |
|  | n3 | 1725 | 5 | 25 | 1820 | N/A | N/A |

<End of Text Proposal>