3GPP TSG-RAN WG4 Meeting #97-e					        R4-2014297

Electronic Meeting, 2-13 Nov., 2020

Title: 	Requirements for L1-SINR measurement accuracy

Source: 	Qualcomm CDMA Technologies

Agenda item:	7.9.3.2	L1-SINR measurement accuracy	[NR_eMIMO-Perf]

Document for:	Discussion

# Introduction

RAN4 agreed to simulation assumptions for evaluating the accuracy of L1-SINR during a previous meeting [1]. We present the simulation results in this contribution and propose L1-SINR accuracy values.

# L1-SINR Simulation Results

Figure 1: CDF of estimated SINR for SCS = 15 kHz

Figure 2: CDF of estimated SINR for SCS = 30 kHz

Figure 3: CDF of estimated SINR for SCS = 30 kHz

Observation 1: Table 1 summarizes the statistics of L1-SINR simulation results in different scenarios. *

* Note that, the results of CMR only scenarios have been updated slightly based on our recent simulations that were not reflected in figure 1, 2 and 3.

Table 1: Statistics of L1-SINR simulation results in different scenarios

| SCS | # samples | Mean [dB] |  |  |  |  | CMR only |  | SSB/NZP IMR |  | SSB/ZP IMR |  | CSI-RS/NZP IMR |  | CSI-RS/ZP IMR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | CMR only | SSB/ NZP | SSB/ ZP | CSI-RS/ NZP | CSI-RS/ ZP | 5-perc. | 95-perc. | 5-perc. | 95-perc. | 5-perc. | 95-perc. | 5-perc. | 95-perc. | 5-perc. | 95-perc. |
| 15 | 1 | -2.12 | -3.3 | -3.35 | -2.9 | -3 | -3.72 | -0.23 | -4.1 | -2.6 | -4.34 | -2.45 | -3.62 | -2.19 | -3.87 | -2.17 |
|  | 3 | -2.04 | -3.31 | -3.35 | -2.9 | -3 | -3 | -0.88 | -3.77 | -2.83 | -3.95 | -2.79 | -3.31 | -2.47 | -3.48 | -2.53 |
|  | 5 | -2.03 | -3.31 | -3.36 | -2.91 | -3.01 | -2.84 | -1.05 | -3.64 | -3 | -3.79 | -2.97 | -3.24 | -2.6 | -3.37 | -2.66 |
| 30 | 1 | -2.17 | -2.76 | -3.17 | -2.82 | -3.18 | -3.72 | -0.34 | -3.77 | -1.89 | -4.15 | -2.29 | -3.48 | -2.15 | -4.01 | -2.36 |
|  | 3 | -2.1 | -2.76 | -3.18 | -2.82 | -3.18 | -2.97 | -1.01 | -3.32 | -2.26 | -3.73 | -2.58 | -3.22 | -2.43 | -3.66 | -2.71 |
|  | 5 | -2.1 | -2.76 | -3.18 | -2.83 | -3.19 | -2.8 | -1.17 | -3.18 | -2.38 | -3.65 | -2.76 | -3.12 | -2.51 | -3.55 | -2.83 |
| 120 | 1 | -2.14 | -2.89 | -3.48 | -2.55 | -3.23 | -3.74 | -0.31 | -3.94 | -1.91 | -5.03 | -2.16 | -3.22 | -1.84 | -4.17 | -2.41 |
|  | 3 | -2.08 | -2.89 | -3.48 | -2.55 | -3.24 | -3.03 | -0.95 | -3.49 | -2.34 | -4.43 | -2.72 | -3.33 | -2.09 | -4.04 | -2.71 |
|  | 5 | -2.09 | -2.9 | -3.48 | -2.55 | -3.24 | -2.91 | -1.21 | -3.37 | -2.45 | -4.15 | -2.84 | -3.08 | -2.18 | -3.79 | -2.82 |

Observation 2: Simulation results show +- 1.5 dB accuracy for all CMR + IMR scenarios and roughly +- 3 dB accuracy for CMR only scenarios.

Based on above results, we find the achievable accuracy for each scenario. We add 2 dB and 3.5 dB margins for FR1 and FR2 respectively. Thereafter, we convert the required accuracy into the nearest ceiling that is an integer multiple of 0.5 dB.

Note that, the L1-RSRP accuracy in FR1 and FR2 are +- 5 dB and +- 6.5 dB. The margin for FR2 is 1.5 dB higher than that of FR1.

Observation 3: The implementation margin for L1-RSRP measurement accuracy in FR2 is 1.5 dB higher than that in FR1.

Proposal 1: RAN4 uses following table to define the estimation accuracy requirements of L1-SINR.

|  | FR1 | FR2 |
| --- | --- | --- |
| CMR only | +- 5 dB | +- 6.5 dB |
| CMR + IMR | +- 3.5 dB | +- 5 dB |

# Conclusion

Observation 1: Table 1 shows the statistics of L1-SINR simulation results in different scenarios.

Observation 2: Simulation results show +- 1.5 dB accuracy for all CMR + IMR scenarios and roughly +- 3 dB accuracy for CMR only scenarios.

Observation 3: The implementation margin for L1-RSRP measurement accuracy in FR2 is 1.5 dB higher than that in FR1.

Proposal 1: RAN4 uses following table to define the estimation accuracy requirements of L1-SINR.

|  | FR1 | FR2 |
| --- | --- | --- |
| CMR only | +- 5 dB | +- 6.5 dB |
| CMR + IMR | +- 3.5 dB | +- 5 dB |

# Reference

- R4-1915850, “Way forward on NR eMIMO RRM”, 3GPP TSG RAN WG4 #93.

# Appendix – Simulation Assumptions For L1-SINR Accuracy

Agreements - L1-SINR accuracy evaluation simulation assumption for CMR only: Parameters; Values. SCS; 15kHz (FR1), 30kHz (FR1), 120kHz (FR2). Channel measurement resource (CMR); CSI-RS. Interference measurement resource (IMR) configuration; N/A. Side condition (SNR) on CMR; -3dB. Density (D); 3. Number of samples (M); 1, 3, 5. Number of PRBs; 48. Propagation condition; AWGN. - L1-SINR accuracy evaluation simulation assumption for SSB-based CMR + NZP-IMR: Parameters; Values. SCS; 15kHz (FR1), 30kHz (FR1), 120kHz (FR2). Channel measurement resource (CMR); SSB. Interference measurement resource (IMR) configuration; CSI-RS. Periodicity; CMR Periodicity = IMR Periodicity. Ideal SINR; -3dB. Side condition (SNR) on CMR; option-1: 0dB, other options not excluded. Side condition (SNR) on IMR; option-1: 0dB, other options not excluded. Density (D) for IMR; 3. Number of samples (M) for IMR/CMR; 1, 3, 5. Number of PRBs for IMR; 48. Propagation condition; AWGN. - L1-SINR accuracy evaluation simulation assumption for SSB-based CMR + ZP-IMR: Parameters; Values. SCS; 15kHz (FR1), 30kHz (FR1), 120kHz (FR2). Channel measurement resource (CMR); SSB. Interference measurement resource (IMR) configuration; CSI-RS. Periodicity; CMR Periodicity = IMR Periodicity. Ideal SINR; -3dB. Side condition (SNR) on CMR; -3dB. Side condition (SNR) on IMR; N/A (only AWGN noise). Density (D) for IMR; 3. Number of samples (M) for IMR/CMR; 1, 3, 5. Number of PRBs for IMR; 48. Propagation condition; AWGN. - L1-SINR accuracy evaluation simulation assumption for CSI-RS-based CMR + NZP-IMR: Parameters; Values. SCS; 15kHz (FR1), 30kHz (FR1), 120kHz (FR2). Channel measurement resource (CMR); CSI-RS. Interference measurement resource (IMR) configuration; CSI-RS. Periodicity; CMR Periodicity = IMR Periodicity. Ideal SINR; -3dB. Side condition (SNR) on CMR; option-1: 0dB, other options not excluded. Side condition (SNR) on IMR; option-1: 0dB, other options not excluded. Density (D) for CMR/IMR; 3. Number of samples (M) for CMR/IMR; 1, 3, 5. Number of PRBs for CMR/IMR; 48. Propagation condition; AWGN. - L1-SINR accuracy evaluation simulation assumption for CSI-RS-based CMR + ZP-IMR: Parameters; Values. SCS; 15kHz (FR1), 30kHz (FR1), 120kHz (FR2). Channel measurement resource (CMR); CSI-RS. Interference measurement resource (IMR) configuration; CSI-RS. Periodicity; CMR Periodicity = IMR Periodicity. Ideal SINR; -3dB. Side condition (SNR) on CMR; -3dB. Side condition (SNR) on IMR; N/A (only AWGN noise). Density (D) for CMR/IMR; 3. Number of samples (M) for CMR/IMR; 1, 3, 5. Number of PRBs for CMR/IMR; 48. Propagation condition; AWGN.