3GPP TSG RAN WG4 Meeting #97-e                   	R4-2014321

Electronic meeting, 02nd – 13th November 2020

Agenda Item:	7.3.3.1, 7.3.4.1 & 7.3.4.2

Source:	LG Electronics

Title:	UE-to-UE coexistence and other remaining issues for NR V2X UE RF requirements in TR38.886

Document for:	Approval

- Introduction

In the last RAN4 #95-e meetings, RAN4 completed 5G V2X WI in rel-17. However, there are still open issues in TR38.886 as follow

- UE-to-UE coexistence requirements

  - Single CC operation in n38, n47

  - NR V2X Con-current operation in V2X_n71-n47

  - Inter-RAT Con-current operation in V2X_20_n38, V2X_n71_47

- Position of Switching period between LTE SL and NR SL at n47 [1]

  - RAN4 do not make consensus of switching period and switch position duraing almost 1 year

  - If, there is not agreement in RF session, then only specify RRM requirements for TDM operation.

- Delta Tib/Rib for V2X_20A_n38A

  - It will be updated based on R4-2014322 [2]

Based on the above remaining issues, we propose updated TP contents to finalize the open issues.

- UE-to-UE coexistence

Currently, single CC V2X operation at n47 or n38, the misaligned TR38.886 and TS38.101-1.

Basically, RAN4 agreed to add Band 47 and n47 to protect for V2X n47 UE. Also some additional new band n77 and n78 also deployed in region1 and region2. So it will be added in TR 38.886 and TS38.101-1.

For V2X con-current operation, RAN4 can follow the principle how to specify the co-existence protect band lists in 2UL CA TR36.860 in Rel-12 as follow.

To define the protected band lists for inter-region issues, we used option 5 in [6].

-    Option5) Choose a subset of protected frequency ranges/bands to protect based on spectrum allocations in the countries where the two bands are deployed

Based on the analysis, we can propose Text Proposal as follow

<Start of TP for TR38.886>

8.1.13	Spurious emission band UE co-existence for V2X UE

This clause specifies the spurious emission requirements for the specified NR V2X band, for coexistence with protected bands

Table 8.1.13-1: Requirements

| NR Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| … |  |  |  |  |  |  |  |
| n38 | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 12, 13, 14, 17, 20, 22, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 50, 51, 52, 65, 66, 67, 68, 72, 74, 75, 76, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2620 | - | 2645 | -15.5 | 5 | 4,5,6 |
|  | Frequency range | 2645 | - | 2690 | -40 | 1 | 4,5 |
| n47 | E-UTRA Band 1, 3, 5, 7, 8, 22, 26, 28, 34, 39, 40, 41, 42, 44, 45, 65, 68, 72, 73 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n47, n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 5925 | - | 5950 | -30 EIRP | 1 | 1,2,3 |
|  | Frequency range | 5815 | - | 5855 | -30 EIRP | 1 | 1,3,7 |
| NOTE 1: Applicable when NS_33 or NS_34 is configured by the pre-configured radio parameters. NOTE 2: In the frequency range x-5950MHz, SE requirement of -30dBm/MHz should be applied; where x = max (5925, fc + 15), where fc is the channel centre frequency. NOTE 3: The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 in TS36.101 from the edge of the channel bandwidth. NOTE 5: This requirement is applicable for power class 3 UE for any channel bandwidths within the range 2570 - 2615 MHz with the following restriction: for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. For power class 3 UE for carriers with channel bandwidth overlapping the frequency range 2615 - 2620 MHz the requirement applies with the maximum output power configured to +19 dBm in the IE P-Max. NOTE 6: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 7: Resolution BW is 10% of the measurement BW and the result should be integrated to achieve the measurement bandwidth. The sweep time shall be set larger than (symbol length)*(number of points in sweep) to improve the measurement accuracy. |  |  |  |  |  |  |  |

In Note 2, the frequency range with -30dBm/MHz will be further discussed.

When "NS_33" is configured from pre-configured radio parameters or the cell and the indication from upper layers has indicated that the UE is within the protection zone of CEN DSRC devices or HDR DSRC devices, the power of any V2X UE emission shall fulfil either one of the two set of conditions.

|  | Maximum Transmission Power (dBm EIRP) | Emission Limit in Frequency Range 5795-5815 (dBm/MHz EIRP) |
| --- | --- | --- |
| Condition 1 | 10 | -65 |
| Condition 2 | 10 | -45 |

<Unchanged sections are omitted>

10	Tx/Rx characteristics of concurrent operation

10.1	NR V2X SL operation in 5.9GHz or other potential ITS spectrum and LTE/NR Uu operation in licensed band for FR1

<Unchanged sections are omitted>

      - Spurious emission band UE co-existence

This clause specifies the spurious emission requirements of the inter-band con-current V2X operation, for UE-to-UE coexistence to protect legacy protected bands

Table 10.1.1.13-1: Requirements

| NR V2X con-current operating band cofiguration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| V2X_n71A-n47A | E-UTRA Band 5, 26, 53 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 NR Band n47,n71, n77 | FDL_low | - | FDL_high | -50 | 1 | 1 |
|  | Frequency range | 5925 | - | 5950 | -30 | 1 | 3, 4 |
|  | Frequency range | 5815 | - | 5855 | -30 | 1 | 3 |
| NOTE 1: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. In case the exceptions are allowed due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3 or 4 for the 2nd, 3rd or 4th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 2: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the aggregated channel bandwidth. NOTE 3: Applicable when NS_33 is configured by the pre-configured radio parameters for power class 3 V2X UE. NOTE 4: In the frequency range x-5950MHz, SE requirement of -30dBm/MHz should be applied; where x = max (5925, fc + 15), where fc is the channel centre frequency. |  |  |  |  |  |  |  |

<Unchanged sections are omitted>

10.2.1.13 Spurious emission band UE co-existence

This clause specifies the spurious emission requirements of the inter-band con-current V2X operation, for UE-to-UE coexistence to protect legacy protected bands

Table 10.2.1.13-1: Requirements

| V2X con-current operating band cofiguration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| V2X_20A_n38A | E-UTRA Band 1, 3, 8, 22, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 38, 42, 52 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 1 |
| NOTE 1: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. In case the exceptions are allowed due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3 or 4 for the 2nd, 3rd or 4th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 2: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 3: This requirement is applicable for power class 3 UE for any channel bandwidths within the range 2570 - 2615 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. For power class 2 UE for any channel bandwidths within the range 2570 - 2615 MHz, NS_44 shall apply. For power class 2 or 3 UE for carriers with channel bandwidth overlapping the frequency range 2615 - 2620 MHz the requirement applies with the maximum output power configured to +19 dBm in the IE P-Max. NOTE 4 For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |  |

<End of TP>

- Position of Switching period between LTE SL and NR SL at n47

RAN4 studied during 1 year for switching period and position for TDM operation between LTE SL and NR SL at ITS spectrum.

They are some candidate options as follow in [1]

- Option 1: Switching period is placed at NR slot and the guard symbol at LTE side can also be utilized.

- Option1a: The whole switching time including transient period should be placed at NR slot.  In last RAN4 meeting, it was revised from option1 due to resource waste. Supporting the option 1a.

- Option 2: The whole switching time including switching period shall be placed at the previous E-UTRA sub-frame or NR slot.  Not aligned in WID description. Need to protect the high priority RAT.

- Option 3: Do not specify switching time RF requirement, use RRM requirement in 38.133 clause 12.9.1 to verify the LTE-NR Tx switching requirement.  If RAN4 do not make progress, then option3 is recommended for switching period for V2X TDM operation. In RF session just make consensus on the switching position.

- Option 4: LTE-NR Tx Switching time requirement is specified as a package in the following:

    - Switching time requirement in RF spec (38.101) is 150us

    - If UE supports this feature: “Support of fewer than 14 consecutive sidelink symbols in a slot”, the switching time requirement can be verified by configuring smaller number of consecutive symbols in a slot, and UE has to satisfy the 150us switching time requirement.

    - If UE doesn’t support this feature: “Support of fewer than 14 consecutive sidelink symbols in a slot”, the switching time requirement is considered as satisfied when UE satisfied the corresponding scheduling restriction requirement in 38.133 clause 12.9.1.

- Option 5: UE implementation + Option 3  UE implementation is quite difficult to decide the scheduling restriction requirement in TS38.133

Our first Priority is still option 1a as shown in candidate options.

Even though, we can compromise not to specify the RF requirements as option3 and just decide the switching position in RF session. Also the switching position shall be aligned slot boundary to reduce resource waste.

This is can be updated the candidate list as Option 6.

- Option 6:  Decide switching position in RF session to inform to RRM session + Option 3

Based on this compromised proposal we can propose Text proposal as follow

<Start of TP for TR38.886>

8	Transmitter characteristics

<Unchanged sections are omitted>

<End of TP>

- Additional ILs and MSD levels for V2X_20A_n38A

Based on the related analysis paper [2], we propose as follow in TR38.886

<Start of TP>

10.2	NR V2X SL operation and LTE/NR Uu operation both in licensed band for FR1

<Unchanged sections are omitted>

10.2.1.4 Configured transmitted power

For the inter-band con-current V2X operation, the configured transmitted power requirements for V2X UE shall be applied per each component carrier. The legacy LTE or NR configured transmitted power for inter-band EN DC shall be decided by MPRc and A-MPRc of each CC.

If the total transmitted power is over the power class of the V2X UE or given PEMAX,c for NR V2X service, the UE shall compare the priority between the legacy LTE service on licensed band and NR SL V2X service at licensed band. Based on the priority, the configured transmitted power will be decided which transmission will be applied on power scale down or dropping between the legacy LTE Uu transmission and NR V2X SL transmission at licensed band.

The ΔTIB,V2X of PCMAX,c,Uu is considered to determine maximum output power for corresponding V2X con-current operating bands

Table 10.2.1.4-1: ΔTIB,V2X for inter-band con-current V2X operation (two bands)

| V2X con-current operating band Configuration | Operating Band | ΔTIB,V2X [dB] |
| --- | --- | --- |
| V2X_20_n38 | 20 | 0.01 |
| Note 1: The ΔTIB,V2X is applied on top of ΔTIB,c of DC_20_n38 UE that is considered harmonic trap filter to reduce 3rd harmonic impact from Band 20. |  |  |

<Unchanged sections are omitted>

    - Rx requirements for inter-band con-current NR V2X operation

      - REFSENS

For the V2X UE RF receiver requirements, RAN4 can refer the 2DL inter-band CA to define general UE RF Rx requirements for inter-band con-current V2X UE.

The legacy REFSENS requirement will be applied on each CC of NR licensed bands if there was no self-interference problems in own receiver frequency band by own uplink and sidelink transmission.

Table 10.2.2.1-1 and Table 10.2.2.1-2 propose the uplink test configurations for inter-band con-current V2X REFSENS requirements. For the uplink configuration, RAN4 consider 10MHz Channel bandwidth.

Table 10.2.2.1-1: Uplink configuration for reference sensitivity of V2X UE (PC5)

| Inter-band V2X con-current band configuration |  |  | LTE or NR UL band / Channel BW / NRB / Duplex mode |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| V2X band (PC5) | LTE or NR band (Uu) | LTE or NR UL band |  | Channel Bandwidth (MHz) | SCS (kHz) | NRB | Duplex Mode |
| n38 | B20 | B20 |  | 10 | 15 | 50 | FDD |

Table 10.2.2.1-2: SL Tx configuration for reference sensitivity of V2X UE (Uu)

| Inter-band V2X con-current band configuration |  |  | LTE or NR UL band / Channel BW / NRB / Duplex mode |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| V2X band (PC5) | LTE or NR band (Uu) | NR V2X band (PC5) |  | Channel Bandwidth (MHz) | SCS (kHz) | NRB | Duplex Mode |
| n38 | B20 | n38 |  | 10 | 15 | 50 | HD |
|  |  |  |  |  | 30 | 24 |  |
|  |  |  |  |  | 60 | 10 |  |

Table 10.2.2.1-3 is proposed the REFSENS requirements with inter-band con-current V2X UE reception without any self-interference problem.

Table 10.2.2.1-3: Reference sensitivity for V2X QPSK PREFSENS

| Inter-band V2X reception |  | Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V2X Band | LTE or NR V2X band (Uu) | LTE or NR Band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | Duplex Mode |
| n38 | B20 | B20 | 15 | -97 | -94 | -91.2 | -90 |  |  | FDD |
|  |  | n38 | 15 |  | -96.5 |  | -93.2 | -91.4 | -90.1 | HD |
|  |  |  | 30 |  | -96.1 |  | -93.4 | -91.7 | -90.2 |  |
|  |  |  | 60 |  | -96.9 |  | -93.1 | -91.9 | -90.4 |  |

Table 10.1.2.2-4 is proposed additional Rx insertion loss according to harmonic trap filter to reduce the harmonic problem based on specific self desense analysis according to specific NR V2X inter-band con-current operation.

Table 10.2.2.1-4: ΔRIB,V2X (two bands)

| V2X inter-band con-current band Combination | V2X operating Band | ΔRIB,V2X [dB] | Note |
| --- | --- | --- | --- |
| V2X_20_n38 | 20 | 0.01 | 3rd harmonic from B20 impact into n38 |
| Note 1: The ΔRIB,V2X is applied on top of ΔRIB,c of DC_20_n38 UE that is considered harmonic trap filter to reduce 3rd harmonic impact from Band 20. |  |  |  |

10.2.2.1a Reference sensitivity exception due to UL harmonic problem

Sensitivity degradation is allowed for a band if it is impacted by UL harmonic interference from another band part of the inter-band con-current V2X UE. Reference sensitivity exceptions (MSD) for the victim band (high) are specified in Table 10.2.2.1a-1 with uplink configuration of the aggressor band (low) specified in Table 10.2.2.1a-2.

Table 10.2.2.1a-1: Reference sensitivity exceptions (MSD) due to UL harmonic for inter-band con-current operation

| V2X inter-band con-current band combinations | Operating Bands / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| V2X_20_n38 | UL band | SL operation | 10 MHz (dB) | 20 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) |
|  | 20 | n38 | 10.7 | 7.7 | 5.8 | 4.7 |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) for which the 3rd transmitter harmonic is within the sidelink transmission bandwidth of a victim (higher) band. NOTE 2: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB such that in MHz and with the carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the low band. NOTE 3: The MSD level applied to all supported SCSs in victim band. |  |  |  |  |  |  |

Table 10.2.2.1a-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for inter-band con-current V2X in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| UL band | SL operation | 10 MHz (LCRB) | 20 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) |
| 20 | n38 | 25 | 50 | 50 | 50 |
| NOTE 1: The UL configuration applies regardless of the channel bandwidth of the UL band unless the UL resource blocks exceed that specified in Table 7.3.1-2 in TS 36.101 [4] or Table 7.3.2-3 in TS 38.101-1 [2] for the uplink bandwidth in which case the allocation according to Table 7.3.1-2 in TS 36.101 [4] or Table 7.3.2-3 in TS 38.101-1 [2] applies |  |  |  |  |  |

<End of Changes>

Reference

- R4-2011714, ‘WF on NR-V2X switching period” CATT

- R4-2014322, “MSD Analysis results and harmonic reduction filter for V2X_20A_n38A,” LG Electronics

- TR 36.860 v12.0.0

- TS38.101-1 v16.5.0

- TS38.101-3 v16.5.0