3GPP TSG-RAN WG4 Meeting # 97-e                                                              R4-2014456

Electronic Meeting, 2-13 Nov., 2020

Source: 	CATT

Title: 	UE parameters for the frequency range 6.425-7.125GHz, 7.025-7.125GHz and 10.0-10.5GHz

Agenda Item:	11.1.1

Document for:	Discussion

- Introduction

In RAN#87-e meeting, a SI on IMT parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz was approved in [1]. There were extensive discussions on this topic and a WF was approved in [2, 3]. Companies are encouraged to have further inputs on how to reply the parameters regarding the open issue in [3]. No progress has been made during RAN4#96e meeting due to misalignment between companies’ simulations.

This paper presents our views on UE parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz based on updated simulation results in [4, 5].

- Discussion

According to the agreed WF [3], the following UE parameters are still open for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz.

- ACLR/ACS/SEM: based on simulations

- Blocking

- Option 1: 38.101

- Option 2: FFS

ACLR

According to the simulation results for uncoordinated deployment (100% shift) for 6GHz in [4], both the average throughput loss and cell edge user throughput loss goes below 5% at 27dB. Since UE ACLR is the dominating factor for uplink ACIR, it is proposed to define UE ACLR as 27dB for 6.425-7.025GHz, 7.025-7.125GHz.

According to the simulation results for uncoordinated deployment (100% shift) for 10GHz in [4], both the average throughput loss and cell edge user throughput loss goes below 5% at 23dB. Since UE ACLR is the dominating factor for uplink ACIR, it is proposed to define UE ACLR as 23dB for 10-10.5GHz.

ACS

According to the simulation results for uncoordinated deployment (100% shift) for 6GHz in [5], both the average throughput loss and the cell edge user throughput loss goes below 5% at 28dB. Since UE ACS is the dominating factor for uplink ACIR, it is enough to define UE ACS as 28dB for 6.425-7.025GHz, 7.025-7.125GHz.

According to the simulation results for uncoordinated deployment (100% shift) for 10GHz in [5], both the average throughput loss and the cell edge user throughput loss goes below 5% at 26dB. Since UE ACS is the dominating factor for uplink ACIR, it is enough to define UE ACS as 26dB for 10-10.5GHz.

SEM

Calculations can be done for SEM once ACLR is determined from the company simulation results for ACLR.

Blocking

It is proposed to adopt the similar blocking requirement as those in 38.101-1 for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz.

- Conclusion

This contribution further discussed the open UE parameters for the frequency range 6.425-7.125GHz and 10.0-10.5GHz.

References

[1]	RP-200513,	New SI proposal: Study on IMT parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz	Ericsson, Huawei, HiSilicon

[2] R4-2005733, WF on BS and UE IMT parameters for the SI on 6.425-7.125GHz and 10.0-10.5GHz, CATT

[3] R4-2009151, WF on BS and UE IMT parameters for the SI on 6.425-7.125GHz and 10.0-10.5GHz, CATT

[4] R4-2014459, Simulation results for 6425-7125MHz and 10-10.5GHz-uplink, CATT

[5] R4-2014458, Simulation results for 6425-7125MHz and 10-10.5GHz-downlink, CATT