| 3GPP TR 37.880 V0.1.0 (2020-10) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; High-power UE operation for fixed-wireless/vehicle-mounted use cases in Band 12, Band 5, and Band n71 (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	6

4	Background	7

4.1	Justification	7

4.2	Objective	7

5	Co-existence studies	8

5.1	Simulation assumptions	9

5.2	Simulation results	9

6	UE operation	9

6.1	UE transmitter third harmonic	9

6.2	BS receiver blocking	9

6.3	UE self-dense	9

6.4	Modem SW changes	9

7	UE hardware	9

7.1	New RF components	9

7.2	Components size and heat	9

7.3	Power supply	9

8	UE requirements	9

8.1	UE Output Power	9

8.2	Transmitter requirements	9

8.3	Receiver requirements	9

Annex A (informative): Change history	10

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document captures the findings from the study item "Study on High-power UE operation for fixed-wireless/vehicle-mounted use cases in Band 12, Band 5, and Band n71".

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP RP-201261: "New SID on high-power UE operation for fixed-wireless/vehicle-mounted use cases in Band 12, Band 5, and Band n71", U.S. Cellular.

[3]	3GPP TR 36.837: "Public safety broadband high power User Equipment (UE)".

[4]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[5]	3GPP R4-2008924: " LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-23 (below 5 GHz)", Ericsson.

[6]	3GPP TR 36.802: "Evolved Universal Terrestrial Radio Access (E-UTRA); NB-IOT; Technical Report for BS and UE radio transmission and reception".

[7]	3GPP TR 36.942: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Frequency (RF) system scenarios".

[8]	3GPP TS 36.101: " Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

CLx-ile	x-percentile CL value

Pmax	UE maximum transmit power

Pt	UE transmit power

Rmin 	Minimum power reduction ratio

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

3GPP	3rd Generation Partnership Project

BS	Base Station

CL	Coupling Loss

DL	Downlink

HPUE	High Power User Equipment

ITU	International Telecommunication Union

LTE	Long Term Evolution

MTC	Machine-Type Communications

NB	Narrowband

NR	New Radio

OOBE	Out Of Band Emissions

PA	Power Amplifier

PC	Power Class

RF	Radio Frequency

Rx	Receiver

SW	Software

TR	Technical Report

Tx	Transmitter

UE	User Equipment

# 4	Background

## 4.1	Justification

Support for fixed wireless and vehicle mounted user equipment usage scenarios, with broader rural coverage and higher data rates is envisioned as part of deployment configurations in LTE band 12 and band 5, and in NR band n71.  Improvements in coverage, availability, and throughput performance to meet the market demands associated with fixed wireless and vehicle mounted usage would be enabled with user equipment specified with a power class 1 (31dBm) up-link transmission capability.

The fixed wireless access scenario provides a variety of benefits consisting of rapid deployment, and a reduction of costs associated with the transport to a customer premise, relative to wireline types of transport. The enablement of fixed wireless user equipment provides backhauling services to an appropriate base station in a serving network, for any other equipment at the customer premise. Similarly, vehicle mounted access scenario provides both direct access to other devices and indirect access to other devices, via the network.

## 4.2	Objective

The objective is to study RF requirements that are applicable for high power UE operation in LTE band 12 and band 5, and in NR band n71 for fixed wireless and vehicle-mounted use cases, in ITU Region 2.

a)	Since the OOBE (Out Of Band Emissions) from the Band 12, Band 5, and Band n71 for high power user equipment (Power Class 1) into a neighbouring band is higher than Power class 3, co-existence studies are anticipated to evaluate the throughput OOBE impact on a victim band, from a high power aggressor in Band 12, Band 5, and Band n71.

1)	Coexistence studies for B5 to include at a minimum the impact on the B26 DL and the public safety downlink in 851-861 MHz.

2)	Coexistence studies for B5 include at a minimum the impact to the B5/n5/B26/n26 uplink including LTE, NR and MTC1 and 2, NB1 and 2 including the A* and B* blocks of the US cellular band configuration as well as the public safety uplink in 806-816 MHz.

3)	Coexistence studies for B12 to include at a minimum the impact to the B29 DL.

4)	Coexistence studies for B12 to include at a minimum the impact to the B12/n12/B71/n71/B85 uplink including LTE, NR and MTC1 and 2, NB1 and 2.

5)	Coexistence studies for n71 to include at a minimum the impact to the B12/n12/B71/n71/B85 uplink including LTE, NR and MTC1 and 2, NB1 and 2.

6)	Coexistence with B66.

b)	Considerations for high power UE operation in Band 12, Band 5, and n71:

1)	B12 UE Tx 3rd harmonic falls into B4 UE Rx band – will require more filters on Tx path if Carrier Aggregation is used:

-	Harmonic exceptions are usually granted, but harmonics from PC1 could be much higher.

2)	B5 UE Tx 3rd harmonic falls into 2.4GHz band – may need more filters on Tx path:

-	Harmonic exceptions are usually granted, but harmonics from PC1 could be much higher.

3)	n71 UE Tx 3rd harmonic falls into 3GPP victim bands – may need more filters on Tx path:

-	Harmonic exceptions are usually granted, but harmonics from PC1 could be much higher.

4)	B5/n5/B26/n26/B27 eNB Rx blocking from UE Tx power in B5.

5)	B85/B12/n12/B17/B66/n66/B71/n71 eNB Rx blocking from UE Tx power in B12.

6)	B85/B12/n12/B17/B2/n2/B25/n25/B71/n71 eNB Rx blocking from UE Tx power in n71.

7)	Self-desense of B5, B12, and n71 Rx due to PC1 UL Tx power.

c)	Considerations in terms of high power user equipment hardware in Band 12, Band 5, and n71:

1)	Identify all new RF components needed for B12, B5, and n71, including Power Amplifier, large ceramic duplex filter, switches, possibly antenna tuner.

2)	New components will be much larger and produce more heat.

3)	Battery power most likely not an option – will need power supply due to high PA current.

4)	Modem SW changes needed to support broader power control range, headroom reporting, and higher output power.

d)	UE requirements.

The scope of these considered requirements in table 4.2-1 is limited to fixed wireless and vehicle-mounted broadband applications.

Table 4.2-1: Considered UE requirements

| Requirement | LTE Band 12 | LTE Band 5 | NR Band n71 |
| --- | --- | --- | --- |
| UE Output Power (dBm) | +31dBm +2/-3dB | +31dBm +2/-3dB | +31dBm +2/-3dB |
| 3GPP 36.101 [8] clause 6 Transmitter requirements | Existing Power Class 1 specification | Existing Power Class 1 specification | Existing Power Class 1 specification |
| 3GPP 36.101 clause 7 Receiver requirements | Existing Power Class 1 specification | Existing Power Class 1 specification | Existing Power Class 1 specification |

High power user equipment (power class 1) would require a different RF specification relative to the typical handheld (power class 3) user equipment. The proposal is to study relevant requirements for high power user equipment operation in Band 12, Band 5, and n71, using a framework similar to that specified for high power user equipment operation in Band 14 (700MHz spectrum) in ITU Region 2. These requirements will be captured in the present document.

# 5	Co-existence studies

Editor note: This clause the first objective of the study item.

## 5.1	Simulation assumptions

### 5.1.1	General

The band plan within the 600 – 900 MHz frequency range in the USA and the corresponding 3GPP operating bands are shown in figure 5.1.1-1 below.

Figure 5.1.1-1: Band plan within the 600 – 900 MHz frequency range in the USA

It can be seen from figure 5.1-1 that the coexistence scenarios between a high-power UE aggressor in LTE band 12 and band 5, and in NR band n71 to the adjacent victim bands are much alike to the scenarios between a high-power UE aggressor in band 14 to the adjacent victim bands. Therefore, the simulation assumptions in TR 36.837 [3] for coexistence study of public safety broadband high power UE can be reused in this study.

Moreover, the frequency range of band 14 is approximately in the middle of LTE band 12 and band 5, and in NR band n71, hence the 790 MHz carrier frequency in TR 36.837 can be used for this study and the LTE simulation results in TR 36.837 can be used for reference directly, as the slight change in the carrier frequency should have minor impacts on the simulation results.

For NR operation, it can be seen in TS 38.101-1 [4] that neither band n71 nor its adjacent bands have larger than 20 MHz UE channel bandwidth. Furthermore, it has been agreed in the reply LS to ITU-R WP5D [5] that there is no beamforming assumed for BS and UE below 1.7 GHz. Considering these two factors, the differences between the LTE and NR simulation results will be minor.

For MTC operation, since it is a narrowband LTE operation, the simulation results will be very similar to the LTE results, considering the narrower interference bandwidth (which means less of the adjacent interference will fall into the BS receiver) and noise bandwidth (which means the BS receiver noise floor will be lower). Therefore, the LTE simulation results in TR 36.837 can be used for reference directly for MTC operation.

For NB-IoT operation, the performance metric is SINR degradation instead of throughput loss as recorded in TR 36.802 [6], and the NB-IoT ACIR model in TR 36.802 should be used for simulation of NB-IoT standalone operation. The UE ACLR is the dominant factor here, and it will be scaled down by a factor of 50 (=10/0.2) due to the channel bandwidth difference between the HPUE aggressor and the NB-IoT victim. Here, the standalone NB-IoT operation will be a worse case compared to in-band or guard-band NB-IoT operation considering the internal gap within the 10 MHz LTE channel bandwidth.

Considering the target completion date of this study item, the simulation results are prioritized according to table 5.1.1-1 according to the discussion above.

Table 5.1.1-1: Priorities of simulation results

| Scenario | Aggressor | Victim | Priority |
| --- | --- | --- | --- |
| UL HPUE Vs NB-IoT standalone operation | HPUE | NB-IoT | High |
| UL HPUE Vs NR / NB-IoT guard band operation (note) | HPUE | NR | Medium |
| UL HPUE Vs LTE / MTC / NB-IoT in-band operation | HPUE | LTE / MTC | Low |
| Note: NB-IoT guard band impact should be checked with the HPUE ACLR agreed from simulation results. |  |  |  |

#### 5.1.2	Macro cell Propagation model - Rural Area

The HPUE will be mainly used for extending the coverage in rural areas. The following Hata rural model [7] is used,

L (R)=  69.55 +26.16log10(f)–13.82log10(Hb)+[44.9-6.55log10(Hb)]log(R)  – 4.78(log10 (f))2+18.33 log10 (f) -40.94

where:

R is the base station-UE separation in kilometres

f is the carrier frequency in MHz

Hb is the base station antenna height above ground in metres

For carrier frequency of 790MHz and base station antenna height of 45m above the ground, the propagation model becomes:

L=94.5+34log10(R)                        	 (1)

Also, per [7], in rural area, Macro cell MCL is 80dB as show in the table 5.1.2-1.

Table 5.1.2-1: Minimal Coupling Loss

| Environment | Scenario | MCL |
| --- | --- | --- |
| Macro cell Urban Area | BS UE | 70 dB |
| Macro cell Rural Area | BS UE | 80 dB |

#### 5.1.3	Power control modelling

In [7], the following power control equation is used for the uplink coexistence simulations:

Pt=Pmax×min1,maxRmin,CLCLx-ileγ	(2)

where Pmax is the maximum transmit power, Rmin is the minimum power reduction ratio to prevent UEs with good channels to transmit at very low power level, CL is the coupling loss defined as max{path loss-G_Tx-G_Rx, MCL}, where path loss is propagation loss plus shadow fading, G_TX is the transmitter antenna gain in the direction of the receiver, G_RX is the receiver antenna gain in the direction of the transmitter and CLx-ile is the x-percentile CL value. With this power control equation, the x percent of UEs that have the highest coupling loss will transmit at Pmax. Finally, 0<<=1 is the balancing factor for UEs with bad channel and UEs with good channel.

For HPUEs, it is assumed that Rmin has 10 dB more dynamic range. The parameter sets for power control are also specified in [7] and showed here in table 5.1.3-1.

Table 5.1.3-1: Power control algorithm parameter for 2GHz band

| Parameter set | Gamma | CLx-ile (10MHz bandwidth) |
| --- | --- | --- |
|  |  | 0.5km cell range |
| Set 1 | 1 | 112 |
| Set 2 | 0.8 | 129 |

However, the power control parameters were specified for carrier frequency of 2 GHz with 500 m cell range. The path loss model was based on Urban Hata model below:

L=128.1+37.6log10(R)	(3)

where:

R is the base station-UE separation in kilometres

Based on equations (1), and (3), the power control parameters can be modified to account for different propagation model, carrier frequency, BS antenna height and cell range. For example, in Table 5.1.3-2, CLx-ile = 112 dB for set 1, which corresponds to R = 0.373 km according to equation (3) assuming total antenna gain of 0 dBi. For 4 km cell range at 700 MHz band, using equation (1) will get CLx-ile = 94.5 + 34×log10(4/0.5×0.373) = 111 dB. Similarly, the modified power control parameters for other cases can be obtained as showed in Table 5.1.3-2.

Table 5.1.3-2: Power control algorithm parameters for 23 dBm UE at 700 MHz band

| Parameter set | Gamma | CLx-ile (10 MHz bandwidth, 45 m antenna height) |
| --- | --- | --- |
|  |  | 4 km cell range |
| Set 1 | 1 | 111 |
| Set 2 | 0.8 | 126 |

For NB-IoT operation, the power control parameters can be modified to account for different bandwidth with bandwidth scale of 17=10×log10(9000/180), as described in TR 36.802 [6]. The modified power control parameters for NB-IoT operation are shown in Table 5.1.3-3.

Table 5.1.3-3: Power control algorithm parameters for NB-IoT UE at 700 MHz band

| Parameter set | Gamma | CLx-ile (10 MHz bandwidth, 45 m antenna height) |
| --- | --- | --- |
|  |  | 4 km cell range |
| Set 1 | 1 | 128 |

#### 5.1.4	Practical considerations for HPUE deployment

The fractional power control formula (2) can be simplified into the following

Pt=Pmax+γCL-γCLx-ile  for CL<CLx-ile	 (4)

Note that the unit in the formula above is dB. If a UE is close to its serving eNodeB so that CL<CLx-ile, whether it is a 23 dBm UE or a 31 dBm UE, it should transmit similar power and the transmitted power should be less than 23 dBm. The 31 dBm UE extends the cell range by being able to transmit more than 23 dBm power at areas where the 23 dBm UE can only transmit its maximum power of 23 dBm.

Using equation (4) will get Pt1=Pmax1+γCL-γCLx-ile1 and Pt2=Pmax2+γCL-γCLx-ile2, where Pt1 denotes the transmit power of a 23 dBm UE at a location with coupling loss CL, and Pt2 denotes the transmit power of a 31 dBm UE at a location with the path loss of PL. Based on the reasoning above, Pt1=Pt2. Hence

CLx-ile2=CLx-ile1+Pmax2-Pmax1γ	(5)

where:

Pmax2=31 dBm and Pmax1=23 dBm

Based on equation (5) and table 5.1.3-2, the new power control parameters for HPUE is obtained in table 5.1.4-1.

Table 5.1.4-1: Power control algorithm parameters for HPUE at 700 MHz band

| Parameter set | Gamma | CLx-ile (10 MHz bandwidth, 45 m antenna height) |
| --- | --- | --- |
|  |  | 8 km cell range |
| Set 1 | 1 | 119 |
| Set 2 | 0.8 | 136 |

The power control parameters for both aggressor system and victim system are summarized in table 5.1.4-2.

Table 5.1.4-2: Power control algorithm parameters for UE at 700 MHz band

| Parameter set | Gamma | CLx-ile (10 MHz bandwidth, 45 m antenna height) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 4 km cell range 23 dBm UE | 4 km cell range NB-IoT UE | 8km cell range HPUE (31 dBm) |
| Set 1A | 1 | 111 | 128 | 119 |
| Set 2A | 0.8 | 126 | N/A | 136 |

#### 5.1.5	Cell layout

For aggressor system with HPUE, the cell size should be bigger than a victim system with 23 dBm UEs. Figure 5.1.5-1 shows overlay of two systems with different cell ranges. In particular, the cell range of aggressor system (in green) is double the cell range of victim (in blue). Figure 5.1.5-1 shows the worst case in the sense that some of the aggressor sites are located at the cell edge of victim system.

Figure 5.1.5-1 Multi system cell layout with different cell radius

#### 5.1.6	Other simulation assumptions

Other simulation assumptions are summarized in the following tables:

Table 5.1.6-1: Simulation parameters for aggressor or victim system with 23dBm UE

| Parameter | Base Station | UE |
| --- | --- | --- |
| Carrier frequency | 790 MHz |  |
| Channel bandwidth | 10 MHz |  |
| Cell range | 4km |  |
| Cell layout | Wrap-around 19 tri-sector cells, uncoordinated |  |
| Frequency reuse | 1x3x1 |  |
| Pathloss model | 94.5+34 log(R), R in km |  |
| Lognormal fading | 10 dB |  |
| Antenna gain and horizontal antenna pattern | A(θ)=-min12θθ3dB2,Am 15 dBi, = 65 degrees, Am = 20 dB | Omni-directional antenna with -6 dBi. |
| Noise figure | 5 dB | 9 dB |
| Transmit power | 46 dBm | 23 dBm |
| Antenna height | 45 m | 1.5 m |
| ACLR | 45 dB | ACLR1: 30+X, ACLR2: 43+X |
| ACS | 45 dB | 33 dB |

Table 5.1.6-2: Simulation parameters for victim system with NB-IoT UE

| Parameter | Base Station | UE |
| --- | --- | --- |
| Carrier frequency | 790 MHz |  |
| Channel bandwidth | 10 MHz |  |
| Cell range | 4km |  |
| Cell layout | Wrap-around 19 tri-sector cells, uncoordinated |  |
| Frequency reuse | 1x3x1 |  |
| Pathloss model | 94.5+34 log(R), R in km |  |
| Lognormal fading | 10 dB |  |
| Antenna gain and horizontal antenna pattern | A(θ)=-min12θθ3dB2,Am 15 dBi, = 65 degrees, Am = 20 dB | Omni-directional antenna with -6 dBi. |
| Noise figure | 5 dB | 9 dB |
| Transmit power | 46 dBm | 23 dBm |
| Antenna height | 45 m | 1.5 m |
| ACLR | 45 dB | ACLR1: 37+X, ACLR2: 50+X |
| ACS | 45 dB | 35 dB |

Table 5.1.6-3: Simulation parameters for aggressor system with HPUE

| Parameter | Base Station | HPUE |
| --- | --- | --- |
| Carrier frequency | 790 MHz |  |
| Channel bandwidth | 10 MHz |  |
| Cell range | 8km |  |
| Cell layout | Wrap-around 7 tri-sector cells, uncoordinated (see Figure 5.1.5-1) |  |
| Frequency reuse | 1x3x1 |  |
| Pathloss model | 94.5+34log(R), R in km |  |
| Lognormal fading | 10 dB |  |
| Antenna gain and horizontal antenna pattern | A(θ)=-min12θθ3dB2,Am 15 dBi, = 65 degrees, Am = 20 dB | Omni-directional antenna -1dBi for vehicle mounted mobile. |
| Noise figure | 5 dB | 9 dB |
| Transmit power | 46 dBm | 33 dBm/31 dBm |
| Antenna height | 45 m | 1.5 m |
| ACLR | 45 dB | ACLR1: 30+X, ACLR2: 43+X |
| ACS | 45 dB | 33 dB |

#### 5.1.7		Simulation procedure

For the co-existence study, the following should be performed:

1)	Run the aggressor UL to victim UL coexistence study assuming parameters of both systems are according to Table 5.1.6-1 and with victim UL for NB-IoT UE operation using parameters in Table 5.1.6-2. Power control parameters in Table 5.1.4-2 are used. This corresponds to the coexistence of two commercial 23dBm UEs networks operating in adjacent bands and with similar deployment parameters. This is used as the reference. Victim system performance degradation results in this scenario are used as the baseline.

2)	Run the aggressor UL to victim UL coexistence study assuming +31dBm power class UE is deployed in the aggressor system using parameters in Table 5.1.6-3, obtain the victim system performance degradation results. The power control parameters in Table 5.1.4-2 are used.

3)	Compare the victim system performance degradation in 2) and 1), choose ACLR value for the HPUE so that the victim performance degradation due to HPUE in 2) is comparable to 1).

## 5.2	Simulation results

## 5.3	Coexistence with Public Safety

For Public Safety operation, it is uncertain whether coexistence simulation would be carried out within the time frame of this study item, as RAN4 has not carried out coexistence simulation with Public Safety system and thus the simulation assumptions and methodology will need to be investigated from inception. The impact on Public Safety downlink at 851-861 MHz can be accessed using analysis.

# 6	UE operation

Editor note: This clause the second objective of the study item.

## 6.1	UE transmitter third harmonic

## 6.2	BS receiver blocking

## 6.3	UE self-dense

## 6.4	Modem SW changes

# 7	UE hardware

Editor note: This clause the third objective of the study item.

## 7.1	New RF components

## 7.2	Components size and heat

## 7.3	Power supply

# 8	UE requirements

Editor note: This clause the fourth objective of the study item.

## 8.1	UE Output Power

## 8.2	Transmitter requirements

## 8.3	Receiver requirements

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2020-08 | RAN4#96-e | R4-2011219 |  |  |  | Skeleton TR | 0.0.1 |
| 2020-10 | RAN4#97-e | R4-201xxxx |  |  |  | Implement approved TP in RAN4#96-e: R4-2011833, and editorial suggestions from ETSI editHelp. | 0.1.0 |