3GPP TSG-RAN4#97e	R4-2014525

17th Aug – 28th Aug 2020

Source: 	Nokia, T-Mobile USA

Title: 					TP for TR 38.717.02-01: CA_n71A-n77A

Agenda Item: 			10.2.2

Document for:	Approval

# 1	Introduction

This is a text proposal to introduce CA_n71A-n77A into TR 38.717.02-01 Rel-17 NR inter-band Carrier Aggregation/Dual connectivity for 2 bands DL with x bands UL (x=1, 2).

# 2 	Text proposal

************************************Start of TP********************************************

## 6.x		CA_n71-n77

### 6.x.1	Common for 1 band UL and 2 bands UL CA

#### 6.x.1.1 Operating bands for CA

Table 6.x.1-1: CA band combination of band n5 + n25

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n71-n77 | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

#### 6.x.1.2	Channel bandwidths per operating band for CA

Table 6.x.2-1: Supported bandwidths per CA band combination of band n71+n77

| NR CA configuration / Bandwidth combination set [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | UL configuration | NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n71A-n77A | CA_n71A-n77A | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

#### 6.x.1.3	Co-existence studies

Table 6.x.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n41-n77.

Table 6.x.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 71 | 663 | 698 | 617 | 652 | 1326 | 1396 | 1989 | 2094 | 2652 | 2792 |
| 77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 |

Based on above table no harmonics occur.

Table 6.x.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 71 | 663 | 698 | 617 | 652 | 1234 | 1304 | 1851 | 1956 | 2468 | 2608 |
| 77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 |

Based on above table no harmonic mixing occur.

#### 6.x.1.4	∆TIB and ∆RIB values

For CA_n41-n77, it is proposed to re-use the TIB,c and RIB values from CA_n41-n71.

Table 6.x.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n71-n77 | n71 | 0.3 |
|  | n77 | 0.6 |

Table 6.x.1.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n71-n77 | n71 | 0.2 |

#### 6.x.1.5	REFSENs requirements

No additional requirements are necessary.

#### 6.x.1.6	OOB blocking exception requirements

There is no OOB exception for the CA combination.

### 6.x.2		Specific for 2 bands UL CA

#### 6.x.2.1	Maximum output power for inter-band CA

Table 6.x.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n71A-n77A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

#### 6.x.2.2		UE co-existence

Table 6.x.2.2-1 gives IMD interference analysis for CA_ n41-n77 with 2 ULs.

Table 6.x.2.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 663 | 698 | 3300 | 4200 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2602 | 3537 | 3963 | 4898 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2874 | 1904 | 5902 | 7737 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4626 | 5596 | 7263 | 9098 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 2211 | 1206 | 9202 | 11937 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5289 | 6294 | 10563 | 13298 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 7074 | 5204 | 7926 | 9796 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 16137 | 12502 | 508 | 1548 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13863 | 17498 | 5952 | 6992 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 11274 | 8504 | 4506 | 6411 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11226 | 13996 | 8589 | 10494 |

Based on the table 6.x.2.2-1, there is 2nd and 5th order IMD are an issue for n77, however IMD2 cannot interfere own downlink channel even it hits the band.

Table 6.x.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.x.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band |  | Frequency range (MHz) |  |  |  | Maximum Level (dBm) |  |  | MBW (MHz) | NOTE |  |
| CA_n71-n77 |  | E-UTRA Band 1, 3, 4, 5, 7, 8, 10, 11, 12, 13, 14, 17, 18, 19, 20, 21, 24, 26, 27, 28, 29, 30, 34, 39, 40, 42, 44, 45, 48, 50, 51, 52, 53, 65, 66, 71, 73, 74, 85, |  | FDL_low | - | FDL_high |  | -50 | 1 |  |  |  |
|  |  | Frequency range |  | 1884.5 |  | 1915.7 |  | -41 | 0.3 |  |  | 8 |
|  |  | E-UTRA Band 2, 25, 41, 70 |  | FDL_low | - | FDL_high |  | -50 | 1 |  |  | 2 |
|  |  | E-UTRA Band 29 |  | FDL_low | - | FDL_high |  | -38 | 1 |  |  | 15 |
|  |  | E-UTRA Band 71 |  | FDL_low | - | FDL_high |  | -50 | 1 |  |  | 15 |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.2-1 in TS 38.101-1 or Table 5.5-1 in TS 36.101 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 8: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |  |  |  |  |  |

#### 6.x.2.3		REFSENS requirements

It is proposed to use IMD5 test point in Table 6.x.2.3-1 which is based on similar CA_n28A-n77A.

Table 6.x.2.3-1

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41A-n77A | n71 | 671 | 5 | 25 | 625 | 5.5 | TDD | IMD5 |
|  | n77 | 3300 | 10 | 50 | 3300 | N/A | TDD | N/A |

************************************End of TP********************************************