3GPP TSG-RAN WG4 Meeting #97-e		R4-2014548

E-meeting, 2 – 13 November, 2020

Agenda item:	7.10.1.2

Source:	Intel Corporation

Title:	Discussion on SDR requirements for FR2 DL 256QAM

Document for:	Discussion

# Introduction

In the previous meeting WF on UE demodulation and CSI reporting requirements for FR2 DL 256QAM was agreed [1].

In this paper we provide our view on UE SDR requirements for FR2 256QAM.

# Discussion

In the previous meeting the following agreements were reached

- Whether to define SDR requirements for FR2 256QAM - Option 1: Add 256QAM (modulation format of 8) to FR2 SDR requirements - Option 2: Not to define FR2 SDR requirements for 256QAM - If it is agreed to define SDR requirements, consider the following test parameters: - MCS and rank - Option 1:Add MCS indexes 26, 21, 20 and 11 in MCS table 2 for both 1 and 2 MIMO layers in the Table 7.5A.1-3. Run simulations to derive the required SNR at 85% throughput for MCS 20 to MCS 26 in MCS table 2, with both 1 layer and 2 layers. - Other options are not precluded.

Taking into account that FR2 requirements are limited by testable SNR, we suggest to identify scenarios which can be executed under existing testing methodology.

TR 38.810 [2] provides information on testable SNR for demodulation requirements. Same time, testable SNR for Normal and SDR requirements is different because generation of noise is not needed for SDR and more power can be used for generation of useful signal. Recently, in our paper [3], we analysed the testable SNR for SDR requirements. In Table 1 we provide estimations on testable SNR and MCS (using impairments results from Table 3) for different aggregation factors and different bands for DFF method.

Table 1. Testable SNR and MCS for SDR requirements

| Band |  | Aggregated channel bandwidth, [MHz] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 50 | 100 | 200 | 400 | 500 | 600 | 700 | 800 | 1000 |
| n257, 258, 261 | TE SNR, [dB] | 32.1 | 29.3 | 26.3 | 23.2 | 22.3 | 21.5 | 20.8 | 20.2 | 18.8 |
|  | Feasible MCS (Rank 1) | MCS27 | MCS27 | MCS26 | MCS23 | MCS22 | MCS22 | MCS21 | MCS21 | N/A |
|  | Feasible MCS (Rank 2) | MCS27 | MCS26 | MCS23 | MCS21 | N/A | N/A | N/A | N/A | N/A |
| n260 | TE SNR, [dB] | 29.5 | 26.7 | 23.7 | 20.6 | 19.7 | 18.9 | 18.2 | 17.6 | 16.2 |
|  | Feasible MCS (Rank 1) | MCS27 | MCS26 | MCS22 | MCS21 | MCS20 | N/A | N/A | N/A | N/A |
|  | Feasible MCS (Rank 2) | MCS26 | MCS23 | MCS21 | N/A | N/A | N/A | N/A | N/A | N/A |

Table 2. PDSCH ideal results

| Rank configuration | MCS 20 | MCS 21 | MCS 22 | MCS 23 | MCS 24 | MCS 25 | MCS 26 | MCS 27 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rank 1 | 16.5 | 17.1 | 18.5 | 19.7 | 20.4 | 21.6 | 22.7 | 24.2 |
| Rank 2 | 19.5 | 20.0 | 21.4 | 22.7 | 23.3 | 24.4 | 25.5 | 27.2 |

Table 3. PDSCH impairments results

| Rank configuration | MCS 20 | MCS 21 | MCS 22 | MCS 23 | MCS 24 | MCS 25 | MCS 26 | MCS 27 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rank 1 | 19.5 | 20.1 | 21.5 | 22.7 | 23.4 | 24.6 | 25.7 | 27.2 |
| Rank 2 | 22.5 | 23.0 | 24.4 | 25.7 | 26.3 | 27.4 | 28.5 | 30.2 |
| Note: Based on simulation results from [4], there is about 3 dB difference between average ideal and average impairment results for 64QAM modulation. We expect that SNR difference for 256QAM will be not lower and assume 3 dB difference for these results. |  |  |  |  |  |  |  |  |

From results in Table 1, we can observe that 256QAM MCS and Rank 2 can be tested for channel bandwidth up to 400 MHz for bands n257, n258, n261 and up to 200 MHz for band n260. Also, we can observe that rather high MCSs (i.e. MCS 24-27) can be tested mainly for 50 and 100 MHz aggregated channel bandwidth, which is not the case for SDR requirements.

In Table 4 we provide our estimations on tested Data Rate for different MCS Tables (without and with 256QAM). 150 MHz aggregated channel bandwidth is considered is the minimum value for FR2 CA (50 + 100 MHz).

Table 4. Data rate for SDR tests

| Band |  |  | Aggregated channel bandwidth, [MHz] |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 150 | 200 | 400 | 500 | 600 | 700 | 800 |
| n257, 258, 261 | Rank 1 | Data Rate MSC Table 1 | 5777 | 7701 | 15409 | 19260 | 23114 | 26965 | 29507 |
|  |  | Data Rate MSC Table 2 | 7870 | 10494 | 18038 | 21317 | 24096 | 28112 | 32129 |
|  |  | Data Rate gain, % | 36% | 36% | 17% | 11% | 4% | 4% | 9% |
|  | Rank 2 | Data Rate MSC Table 1 | 11555 | 15409 | 29509 | 35243 | 42292 | 45905 | 52462 |
|  |  | Data Rate MSC Table 2 | 14266 | 18038 | 32131 | 35243 | 42292 | 45905 | 52462 |
|  |  | Data Rate gain, % | 23% | 17% | 9% | 0% | 0% | 0% | 0% |
| n260 | Rank 1 | Data Rate MSC Table 1 | 5777 | 7701 | 15409 | 18441 | 21144 | 22956 | 26236 |
|  |  | Data Rate MSC Table 2 | 7500 | 9511 | 16064 | 19260 | 21144 | 22956 | 26236 |
|  |  | Data Rate gain, % | 30% | 24% | 4% | 0% | 0% | 0% | 0% |
|  | Rank 2 | Data Rate MSC Table 1 | 11555 | 15409 | 26231 | 30323 | 36388 | 40173 | 41980 |
|  |  | Data Rate MSC Table 2 | 12790 | 16064 | 26231 | 30323 | 36388 | 40173 | 41980 |
|  |  | Data Rate gain, % | 11% | 4% | 0% | 0% | 0% | 0% | 0% |

From this table we can observe that significant tested data rate increasing can be observed only for scenarios with small aggregated channel bandwidth (up to 500 MHz for n257, 258, 261 and up to 200 MHz for n260). Same time, we can observe if UE supports Rank 2 transmission and aggregated CBW 500 MHz for bands n257, 258, 261, then such UE will be tested with 64QAM modulation. For band n260, similar situation will be observed in case UE supports Rank2 and aggregated CBW 400 MHz

Based on all above analysis, we suggest not to define FR2 SDR requirements for 256QAM.

Proposal 1:	Do not define SDR requirements for FR2 256QAM.

# Conclusion

In this paper we provided view on FR2 256QAM SDR requirements definition and made the following proposals:

Proposal 1:	Do not define SDR requirements for FR2 256QAM.

# References

- R4-2012666 “WF on UE demodulation and CSI reporting requirements for FR2 DL 256QAM”, China Telecom, RAN4 #96-e, August 2020.

- TR 38.810 “Study on test methods”

- R4-1902881 “NR SDR performance requirements”, Intel, RAN4 #90bis, April 2019.

- R4-1905736 “Summary of simulation results of NR FR2 SDR requirements”, Intel, RAN4 #91, May 2019.