3GPP TSG-RAN WG4 Meeting #97-e		R4-2014549

E-meeting, 2 – 13 Nobember, 2020

Agenda item:	7.16.1.1

Source:	Intel Corporation

Title:	Discussion on NR CA UE demodulation requirements

Document for:	Discussion

# Introduction

In RAN4 #96-e WF on PDSCH CA normal demodulation requirements was approved [1]. In this paper we provide view on remaining open issues for NR CA PDSCH normal demodulation requirements.

# Discussion

## Testing of different CA capabilities

In the previous RAN4 meeting the following agreements were reached on NR CA applicability rules:

- Option 1: Test intra-band contiguous CA, intra-band non-contiguous CA and inter-band CA with the largest number of bands. - Option 2: Test all the supported CA capabilities, including intra-band contiguous CA, intra-band non-contiguous CA and inter-band CA with different numbers of bands. - Option 3: - Intra-band CA: test intra-band contiguous CA, and intra-band non-contiguous CA - Inter-band CA: test inter-band CA with the largest number of bands and inter-band CA with the largest aggregated CBW - The details are to be discussed - If the selection of “inter-band CA with the largest number of bands” and “inter-band CA with the largest aggregated CBW” results in the same CA configuration(s), only one inter-band CA configuration will be tested; otherwise, two inter-band CA configurations will be tested. - Other options are not precluded

Depending on scenarios, LTE UE is tested for each or any one supported CA capability. We think that it is rather important to defined CA requirements with testing on all CA scenarios, i.e. contiguous Intra-band, non-contiguous Intra-band and Inter-band. Testing of UE for multiple Inter-band CA scenarios with different number of bands looks rather redundant. Therefore, in the previous meeting, several options were considered to avoid such redundancy. The simplest option is just to test the inter-band CA with the largest number of bands. Same time, there was concern that scenario with the largest number of bands is not the scenario with the largest aggregated bandwidth. Therefore, it was suggested to test inter-band CA with the largest number of bands and inter-band CA with the largest aggregated CBW. However, based on our understanding, due to flexible UE capability, scenario with the largest aggregated CBW may not correspond to scenarios with the largest supported data rate and, potently, such scenario can not be selected for testing because UE does not support Rank 2 with 16QAM transmission for such aggregated channel bandwidth.

Another possible option is just checking of the maximum supported data rate for each supported CA bandwidth combination for all supported Inter-band CA scenarios and select CA scenario which contains the CA bandwidth combination with the largest data rate (using similar procedure as for SDR requirements).

Proposal 1:	Consider the following CA capabilities for NR Normal CA testing: Intra-band contiguous CA, Intra-band non-contiguous CA and Inter-band CA with CA bandwidth combination the largest data rate.

## Selection of CA configuration for testing

In the previous RAN4 meeting the following agreements were reached on methodology for selection of CA configuration for testing:

- Options for further discussion - Option 1: Proposal 7 in R4-2009579 - Option 2: Proposal 6 in R4-2011010 - Option 3: Proposals 5 and 6 in R4-2009730 - Other options are not precluded - Way forward - Companies are encouraged to provide the comments on the following questions: - Whether to consider maxNumberMIMO-LayersPDSCH, supportedModulationOrderDL and scalingFactor, since only 16QAM and rank 2 are used? - If yes for the above bullet, align the understanding of these capability based on 38.306, such as, applied per CC, per band or in the final data rate calculation? - Which option is better to accommodate the FR2 testability?

Based on our understanding, due to very flexible NR UE capability signalling (maxNumberMIMO-LayersPDSCH, supportedModulationOrderDL and scalingFactor), this signalling should be taken into account in procedure for selection of tested CA configuration. Previously, it was agreed that Rank 2 and MCS13 will be used for NR CA requirements definition. For all considered channel bandwidth combinations these parameters can be converted in certain data rate. Same time, NR UE may have limitation on maximum supported Data Rate. Therefore, potentially, some UE may not be tested with that Rank 2 and MCS13 for scenarios with the certain aggregated channel bandwidths.

Also, we would like to clarify our understanding on these NR UE capabilities:

- maxNumberMIMO-LayersPDSCH is used for limitation of maximum supported number of MIMO layers per CC

- supportedModulationOrderDL and scalingFactor are used for limitation of maximum supported Data Rate based on the following information from 38.306

supportedModulationOrderDL Indicates the maximum supported modulation order to be applied for downlink in the carrier in the max data rate calculation as defined in 4.1.2. If included, the network may use a modulation order on this serving cell which is higher than the value indicated in this field as long as UE supports the modulation of higher value for downlink. If not included: - for FR1, the network uses the modulation order signalled in pdsch-256QAM FR1. - for FR2, the network uses the modulation order signalled per band i.e. pdsch- 256QAM-FR2 if signalled. If not signalled in a given band, the network shall use the modulation order 64QAM. In all the cases, it shall be ensured that the data rate does not exceed the max data rate (DataRate) and max data rate per CC (DataRateCC) according to TS 38.214

As the first step of CA configuration selection procedure, we can select CA configurations which correspond to tested CA capability (i.e. Intra-band FDD, Inter-band FDD-TDD etc.)

In the next steps we need to take into account that requirements are defined for fixed FRC. First, we need to select CA configurations which contain CCs with support of number of MIMO layer specified for requirements. Second, we need to take into account that data rate during the test should not exceed maximum supported data rate (calculated based on equation from TS 38.306 [3] Section 4.1.2).

After selection of CA configurations where UE supports required Number of MIMO layers and Data Rate, further downselection of CA configurations can be done based on data rate or aggregated channel bandwidth calculations.

Based on suggestions above, we propose the following methodology for selection of CA configuration for testing for each CA capability:

- Step 1: Select CA configurations with maximum number of CCs, on which UE capability field maxNumberMIMO-LayersPDSCH is higher or equal to νLayersreq (number of MIMO layers for requirements definition), among all supported CA configurations

- Step 2: Select any one of CA configurations, which contain CBW combinations with the largest data rate not exceeding DataRatereq, among all the selected CA configurations from Step 1.

or

For FR2 CA testing, in the first step it is rather beneficial to select CA configurations which contain CBW combinations with maximum achievable SNR (SNRTEmax) higher or equal to SNR used for requirements definition (SNRreq). Such approach allows to avoid selection of CA configuration and CBW combination which cannot be tested due to limitation at TE side.

Proposal 2:	Use the following approach for selection of CA configuration for NR FR1 Normal CA testing for each CA capability:

- Step 1: Select CA configurations with maximum number of CCs, on which UE capability field maxNumberMIMO-LayersPDSCH is higher or equal to νLayersreq, among all supported CA configurations

- Step 2: Select any one of CA configurations, which contain CBW combination with the largest data rate not lower than DataRatereq, among all the selected CA configurations from Step 1.

Proposal 3:	Use the following approach for selection of CA configuration for NR FR2 Normal CA testing for each CA capability:

- Step 1: Select CA configurations, which contain CBW combinations with SNRTEmax higher or equal to SNRreq, among all supported CA configurations

- Step 2: Select CA configurations with maximum number of CCs, on which UE capability field maxNumberMIMO-LayersPDSCH is higher or equal to νLayersreq, among all the selected CA configurations from Step 1

- Step 3: Select any one of CA configurations, which contain CBW combination with the largest data rate not lower than DataRatereq and aggregated bandwidth with SNRTEmax higher or equal to SNRreq, among all the selected CA configurations from Step 2.

# Conclusion

In this paper we provided view on test methodology and simulation assumptions for NR CA PDSCH normal demodulation requirements and made the following proposals:

Proposal 1:	Consider the following CA capabilities for NR Normal CA testing: Intra-band contiguous CA, Intra-band non-contiguous CA and Inter-band CA with CA bandwidth combination the largest data rate.

Proposal 2:	Use the following approach for selection of CA configuration for NR FR1 Normal CA testing for each CA capability:

- Step 1: Select CA configurations with maximum number of CCs, on which UE capability field maxNumberMIMO-LayersPDSCH is higher or equal to νLayersreq, among all supported CA configurations

- Step 2: Select any one of CA configurations, which contain CBW combination with the largest data rate not lower than DataRatereq, among all the selected CA configurations from Step 1.

Proposal 3:	Use the following approach for selection of CA configuration for NR FR2 Normal CA testing for each CA capability:

- Step 1: Select CA configurations, which contain CBW combinations with SNRTEmax higher or equal to SNRreq, among all supported CA configurations

- Step 2: Select CA configurations with maximum number of CCs, on which UE capability field maxNumberMIMO-LayersPDSCH is higher or equal to νLayersreq, among all the selected CA configurations from Step 1

- Step 3: Select any one of CA configurations, which contain CBW combination with the largest data rate not lower than DataRatereq and aggregated bandwidth with SNRTEmax higher or equal to SNRreq, among all the selected CA configurations from Step 2.

# References

- R4-2012688 “Way forward on PDSCH CA normal demodulation requirements”, Intel, RAN4 #96-e, August 2020.

- R4-2003185 “Discussion on NR CA UE demodulation requirements”, Intel, RAN4 #94-e-Bis, April 2020.

- TS 38.306 “User Equipment (UE) radio access capabilities”.