3GPP TSG-RAN WG4 Meeting # 97-e	R4-2014598

Electronic Meeting, 2-13 Nov., 2020

Source: 	Nokia, Nokia Shanghai Bell

Title: 					More on an alternative to creating new BCSs

Agenda Item: 			17.1

Document for:	Approval

# 1	Introduction

Though RAN4#96-e discussed an alternative to creating new BCSs based on [1], no conclusion was made due to mainly the following two concerns.

- More IODT efforts: The following concern was captured in [2].

  - With this BCS4 proposal, if UE for example report to support n1 with maximum BW of 50MHz, then UE should automatically support all the possible band combos which are less than 50MHz regardless of whether there is specified BCS request from companies.

- Unclarity on how to handle cases that new channel bandwidth(s) is introduced into an existing band

This contribution addresses the above raised concerns.

# 2	Discussion

On concern on more IODT efforts

Suppose that if an alternative proposed in [1] is applied to a UE supporting new channel bandwidths for n1 and n3 as well as CA_n1A-n3A and if the UE reports supported maximum CBW of 50 MHz for n1 and that of 40 MHz for n3 via supportedBandwidthDL, respectively. The supported channel bandwidth (CBW) combinations by the UE must look like Table 2.2. Note that Table 2.1 shows currently specified BCS 0 table for CA_n1A-n3A.

Table 2.1: Currently specified BCS 0

| NR CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |
|  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |

Table 2.2: CBW combinations with [1] with supportedBandwidthDL of 50 & 40 MHz for n1 & n3

| NR CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
|  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

Note: Cells coloured in yellow shows difference between Table 2.1 and 2.2.

If we understand the concern correctly, with the alternative in [1], the UE shall meet all the requirements for the captured CBW combinations in Table 2.2. If we adopt the conventional method, i.e., the introduction of a new BCS, the following CBW combination set shown in Table 2.3 is possible and this allows UE and network to save IODT efforts.

Table 2.3: A possible CBW combinations for CA_n1A-n3A whose CBW less than 20 MHz

| NR CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
|  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

The demand for CBW combinations for CA_n1A-n3A in Table 2.3 is real possibility, since it is likely that up to 15 MHz channel bandwidth would not be used in a region and/or country where up to 50 MHz and 40 MHz channel bandwidth available for n1 and n3 respectively. The reason would be that if an operator holds 50 MHz for n1/LTE Band 1 and 40 MHz for n3/LTE Band 3, the operator would not use merely 5MHz for NR in their commercial service. Thus, they would not need smaller channel bandwidths. In fact, if we look back at LTE specification, this has happened as shown in Table 2.4. It is likely that when larger channel bandwidths are used for some of the bands within a band combination, smaller channel bandwidths are omitted, though it is not always.

Table 2.4: E-UTRA CA configurations and bandwidth combination sets defined for CA_4A-12A

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations (NOTE 4) | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_4A-12A | CA_4A-12A | 4 | Yes | Yes | Yes | Yes |  |  | 20 | 0 |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
|  |  | 4 | Yes | Yes | Yes | Yes | Yes | Yes | 30 | 1 |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes | 30 | 2 |
|  |  | 12 |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 4 |  |  | Yes | Yes |  |  | 20 | 3 |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes | 30 | 4 |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
|  |  | 4 |  |  | Yes | Yes | Yes |  | 20 | 5 |
|  |  | 12 |  |  | Yes |  |  |  |  |  |

Hence, it is understandable that the 1st concern was raised. In order to address the 1st concern, one possible way would be defining a minimum CBW supported per SCS per NR band within a band combination as UE capability in addition to supportedBandwidthDL that indicates maximum CBW per SCS per NR band within a band combination.

For example, if a UE report a minimum CBW = 20 MHz for both n1 and n3 within CA_n1A-n3A together with supportedBandwidthDL = 50 MHz for n1 and that for 40 MHz for n3 within CA_n1A-n3A, it is possible for UE to support CBW combinations captured in Table 2.3 with simple UE capabilities.

Finally, we would like to emphasis on that the alternative to creating new BCSs provides advantage that RAN4 does not need to update the existing NR CA CBW combination tables in specifications anymore as well as there is no need for gNodeB to import these tables to software. We believe that the benefit is significant. Hence, we propose the following.

Proposal 1: Introduce an alternative to creating new BCSs in the following ways.

- For a given SCS, a UE reports

  - supported CBW(s) for each NR band as a single band operation if any.

  - supported maximum CBW for each NR band within a band combination via supportedBandwidthDL

  - supported minimum CBW for each NR band within the band combination via UE capability

- The supported CBW combinations consist of CBWs which satisfy the above three conditions per band. A fixed BCS number such as BCSx, e.g., x=4, common to any band combinations to inform BS that the UE is utilizing the alternative to creating new BCSs.

Proposal 2: Introduce a new capability for a UE to indicate the supported minimum CBW per SCS per NR band within a band combination and send an LS to RAN2 to inform RAN2 of its necessity.

On concern on MSD due to the introduction of new CBW(s) for a band

MSD requirements for all the possible CBW combinations for band combinations including bands which introduced new channel bandwidths should be specified, when the new CBWs are introduced. Specifying MSD for these band combinations are not always required and even if it is required, the agreeable MSD values can be derived with ease by referring to those for the existing CBWs, as far as sizes of newly introduced CBWs are smaller than that of the exiting maximum CBW. In addition, even if we kept using the conventional BCS method, in any case, new MSD due to the introduction of the new CBWs itself is necessary. Hence, additional work for new MSD itself should not be a bottleneck to introduce the alternative to creating new BCS. Only what we need to clarify is that when and in which WI RAN4 is going to specify MSD. In summary, we obtain the following observation and propose the followings.

Observation: The amount of work for MSD for CBW combinations due to the introduction of new CBWs for existing band combinations would not be affected by the introduction of the new approach in R4-2010062.

Proposal 3: MSD for all the possible CBW combinations due to the introduction of new CBWs for existing band combinations should be specified. RAN4 further discusses the detail on when and in which WI RAN4 specifies MSD in the next meeting.

# 4	Conclusion

In this contribution, we addressed concerns raised for [1]. In order to address them, we propose the followings.

Proposal 1: Introduce an alternative to creating new BCSs in the following ways.

- For a given SCS, a UE reports

  - supported CBW(s) for each NR band as a single band operation if any.

  - supported maximum CBW for each NR band within a band combination via supportedBandwidthDL

  - supported minimum CBW for each NR band within the band combination via UE capability

- The supported CBW combinations consist of CBWs which satisfy the above three conditions per band. A fixed BCS number such as BCSx, e.g., x=4, common to any band combinations to inform BS that the UE is utilizing the alternative to creating new BCSs.

Proposal 2: Introduce a new capability for a UE to indicate the supported minimum CBW per SCS for each NR band within the band combination and send an LS to RAN2 to inform RAN2 of its necessity.

Observation: The amount of work for MSD for CBW combinations due to the introduction of new CBWs for existing band combinations would not be affected by the introduction of the new approach in R4-2010062.

Proposal 3: MSD for all the possible CBW combinations due to the introduction of new CBWs for existing band combinations should be specified. RAN4 further discusses the detail on when and in which WI RAN4 specifies MSD in the next meeting.

The draft LS is captured in the Annex in this contribution.

# 5	References

[1] R4-2010062, An alternative to creating new BCSs, T-Mobile USA, Deutsche Telekom, AT&T, Telus, Bell Canada, Telstra, Telecom Italia, Ericsson

[2] R4-2011881, Email discussion summary for [96e][144] BC_simplification, NTT DOCOMO, INC.

# 6	Annex

Title:	draft LS on bandwidth combination set handling

Response to:

Release:	Release 16

Work Item:	TEI16

Source:	3GPP TSG RAN WG4

To:	RAN2

Cc:	NONE

Contact Person:

- Name:	Hiromasa Umeda

- E-mail Address:	hiromasa.umeda@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

RAN4 agreed to stop introducing new BCS tables into RAN4 specifications anymore and adopt an alternative to creating new BCSs in the following manner by combining an existing signalling mechanism as well as introducing one new UE capability(Hereafter referred to as supportedMinBandwidthDL) to indicate supported minimum CBW per SCS for each NR band within a band combination via supportedMinBandwidthDL.

- For a given SCS, a UE reports

  - supported CBW(s) for each NR band as a single band operation if any.

  - supported maximum CBW for each NR band within a band combination via supportedBandwidthDL

  - supported minimum CBW for each NR band within the band combination via supportedMinBandwidthDL

- The supported CBW combinations consist of CBWs which satisfy the above three conditions per band. A fixed BCS number such as BCSx, e.g., x=4, common to any band combinations to inform BS that the UE is utilizing the alternative to creating new BCSs.

2. Actions:

To: 3GPP TSG RAN WG2.

ACTION: RAN4 respectfully asks RAN2 to take the above information into account to reflect the RAN4 agreements into relevant RAN2 specification(s).

3. Date of Next TSG-RAN WG4 Meetings:

TSG RAN4 Meeting #98-e		25th Jan – 5th Feb 2021		               Electronic meeting