3GPP TSG RAN WG4 Meeting #97-e	R4-2014614

Electronic Meeting, 2-13 Nov., 2020

Agenda item:	10.4.2

Source:	SoftBank Corp., LG Electronics

Title:	TP update for TR 37.717-21-11: EN-DC_1-11_n28

Document for:	Approval

1	Introduction

EN-DC of 2B LTE and 1B NR of DC_1-11_n28 was approved in RAN4#96[1] but REFSENS exceptions have TBD parts. This TP is to update the REFSENS exception results for 2nd IMD and reflect the REFSENS exception.

2	Text Proposal

[Unchanged Parts Skipped]

5.x	DC_1-11_n28

### 5.x.1	Configurations for DC_1-11_n28

Table 5.x.1-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-11A_n28A | DC_1A_n28A DC_11A_n28A | CA_1A-11A | n28A |

### 5.x.2	Co-existence studies

When Uplink EN-DC configuration is DC_11A_n28A, IMD2 and IMD3 of (B11 - n28) will fall into Rx band of Band 1. But considering the current situation that Band 11 is operated only by a certain operators in Japan, the frequency ranges can be limited as follows.

- Operator X: B11 (UL:1427.9-1437.9 / DL:1475.9-1485.9 MHz), B1 (UL:1960-1980 / DL:2150-2170 MHz), n28 (UL:738-748 / DL:793-803 MHz)

- Opeartor Y: B11 (UL:1437.9-1447.9 / DL:1485.9-1495.9 MHz), B1 (UL:1920-1940 / DL:2110-2130 MHz), n28 (UL:718-728 / DL:773-783 MHz)

Then the own Rx impact can be simplified as below.

- IMD2 of (B11 - n28) will fall into Rx band of Band 1

### 5.x.3	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.x.3-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-11_n28 | 1 | 0.3 |
|  | 11 | 0.4 |
|  | n28 | 0.6 |

Table 5.x.3-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-11_n28 | 1 | 0 |
|  | 11 | 0 |
|  | n28 | 0.2 |

### 5.x.4	Reference sensitivity exceptions

As mentioned above, IMD2 of B11 and n28 to Band 1 Rx need to be addressed for REFSENS relaxation. The following values are proposed:

Table 5.x.4-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_1A-11A_n28A | 11 | 1440 | 5 | 25 | 1488 | N/A | FDD | N/A |
|  | n28 | 710 | 5 | 25 | 765 | N/A | FDD | N/A |
|  | 1 | 1960 | 5 | 25 | 2150 | 28.3 | FDD | IMD2 |

[Unchanged Parts Skipped]

7.  Reference

[1]  R4-2009992  TP for TR 37.717-21-11: EN-DC_1-11_n28, SoftBank Corp.