3GPP TSG RAN WG4 Meeting #97-e	R4-2014647

Electronic Meeting, 2-13 Nov., 2020

Agenda item:	10.7.2

Source:	SoftBank Corp.

Title:	TP for TR 37.717-11-21: EN-DC_11_n3-n77

Document for:	Approval

1	Introduction

EN-DC of 1B LTE and 2B NR of DC_11_n3-n77 was approved in RAN#89 [1]. This TP is to capture the basic aspects for the EN-DC. REFSENS exception results are needed to complete the combo.

2	Text Proposal

[Unchanged Parts Skipped]

6.X	DC_11_n3-n77

6.X.1	Operating bands for DC

Table 6.X.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_11_n3-n77 | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  |  | n3 |  | 1710 MHz |  | – |  | 1785 MHz |  | 1805 MHz |  | – |  | 1880 MHz |  | FDD |  |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.X.2	Channel bandwidths per operating band for DC

Table 6.X.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_11A_n3A-n77A | DC_11A_n3A DC_11A_n77A | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 140 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_11A_n3A-n77(2A) | DC_11A_n3A DC_11A_n77A | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 240 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.X.3	Co-existence studies

Based on the co-existence studies of DC_11A-n3A and DC_11A-n77A captured in TR 37.716-11-11 and 37.863-01-01,

- 4th and 5th order IMD generated by dual uplink of Band 11 + Band n3 may also fall into own Rx of Band n77

- 2nd and 5th order IMD generated by dual uplink of Band 11 + Band n77 may also fall into own Rx of Band n3.

6.X.4	∆TIB and ∆RIB values

For DC_11_n3-n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.X.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_11_n3-n77 | 11 | 0.8 |
|  | n3 | 0.9 |
|  | n77 | 0.8 |

Table 6.X.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_11_n3-n77 | 11 | 0.3 |
|  | n3 | 0.5 |
|  | n77 | 0.5 |

6.X.5	MSD

As mentioned in 6.X.3, IMD4 of B11 and n3 to n77 Rx and IMD2 of B11 and n77 to n3 Rx need to be addressed for REFSENS relaxation. The following values are proposed:

Table 5.1.x.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_11A-n3A_n77A DC_11A-n3A_n77(2A) | 11 | TBD | TBD | TBD | TBD | N/A | FDD | N/A |
|  | n3 | TBD | TBD | TBD | TBD | N/A | FDD | N/A |
|  | n77 | TBD | TBD | TBD | TBD | TBD | TDD | IMD4 |
|  |  | 11 |  | TBD |  | TBD |  | TBD |  | TBD |  | N/A |  | FDD |  | N/A |  |
|  |  | n77 |  | TBD |  | TBD |  | TBD |  | TBD |  | N/A |  | TDD |  | N/A |  |
|  |  | n3 |  | TBD |  | TBD |  | TBD |  | TBD |  | TBD |  | FDD |  | IMD5 |  |

[Unchanged Parts Skipped]

7.  Reference

[1]  RP-201477  Revised WID on Rel-17 Dual Connectivity (DC) of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA(2DL/1UL), LG Electronics