3GPP TSG-RAN WG4 Meeting #97-e	R4-2014712

Nov 2020

Title:	Tx diversity changes for Rel-16

Source:	Qualcomm Incorporated

Agenda item:	7.19.2.1

Release:	Rel-16

Work Item:

Responsible WG:

Document for:	Approval

# 1.	Introduction

In RAN4#96e an agreement was made and recorded into the chairman minutes:

“Chair: For PC2 and PC3, MPR issues related to 2TX, including UL-MIMO, uplink full power transmission, and TxD, will be further discussed in TEI16.”

There was also WF [1] agreed in the same meeting but many open items still remain. This paper discusses open items and how to conclude the work and introduce TxD for rel-16 specifications.

# 2. 	Discussion

## 2.1	Introduction

WF agrees two things: 1) is to use terminology “requirements are defined as the sum of powers from both connectors” and 2) “MPR defined for TxD is applied to the total output power rather than at each antenna connector”. Earlier WF also agrees the method to define UE level ACLR [2].

WF has options or FFS’s for EVM, Declaration of default connector, UE behaviour in conformance testing, power splitting behaviour, “signalling” which means the capability for TxD, applicability of requirements meaning how to write the requirements and how to handle CDD.   We will not discuss all options here but indicate what is our view on closing the open items.

## 2.2	EVM handling

EVM details were agreed to be put in the Annex F and an already agreed method with the formula in the earlier WF [2] or new method proposed in [3]. While we do not have a strong opinion on which method RAN4 will adopt and we share our view that we can leave details on EVM definition to implemented later for Annex F. This is because in both cases the EVM needs to be measured from each connector separately for 1-layer signal when coming out of more than one antenna connector. The core specification can be written with this information.

## 2.3	Declaration of default connector

We do not have a strong preference on this issue. Option 1b and 2 are feasible. If UE declares a default connector or not is more ran5 issue and ran4 could leave the discussion to testing details. Ran4 could indicate to ran5 that declaring one default tx connector is feasible if needed but the option 1b also works from UE requirements point of view that TE detects all TX antenna connectors.

## 2.4	UE behaviour in conformance testing

This issue has relevance in UE behaviour. Ran4 can three way to solve this issue:

- Test mode when UE does not change tx div status i.e. UE keeps transmitting from same antenna connector(s) and keep power imbalance same

- TE needs to adapt to UE changes in TxDiv status i.e. TE can not make any assumption for any individual transmission but needs to sum every (declared) antenna port

- Leave this to test details i.e. UE vendors ensure that any test will not make UE change TxDiv state. This maybe easier than it seems since our expectation is that UE will use Dl channel information on choosing to turn on and off TxD and in Tx tests, Dl is assumed to be stable

Our preference is mildly to option 2 but we can accommodate others too.

## 2.5	Power spitting behaviour

How UE splits the power between the tx branches needs some discussion. The are two distinct  two issues: 1) Error in power between branches that maybe in the order few dB’s 2) Intentional power imbalance that maybe much more, in the order of 10 dB’s

For issue 1, UE should be allowed a certain mismatch that may be caused by UE internal routing or part to part differences.

Issue 2 implies advanced TxD behaviour such as detecting antenna blockage and increasing power for the blocked antenna. Or dropping the other branch to save power. This kind of behaviour has not been discussed much but also it has not been justified why it needs to be ruled out in core requirement level. The agreements for power summing [1], ACLR [2] and EVM [2] so far accommodate any intentional power difference and unless there is a good reason, this behaviour should not be precluded.

Proposal 1: RAN4 core requirements for TxD should enable intentionally set power difference between the tx connectors

## 2.6	TxD feature i.e. “signalling”

This issue should have high priority in Ran4 discussion since it has a big impact on how we will write requirements and without the agreement on this, drafting a CR is very difficult.

In our view, we would need to distinguish UE’s with and without TxD somehow. That seems to be also general view.

The modified MPR behaviour can be made to work but it will then reserve a bit from every band and will be highly confusing since there are other requirements than MPR impacted. Using modified MPR behaviour is not our preference.

Our preference is to define a way to distinguish TxD UE’s from non-TxD UE’s. Two remaining ways in the WF [2] are defining new power class and dedicated capability. The capability is the most straight forward way and is our first preference. New power class is more cumbersome way but can be made to work with some extra effort in fulfilling the power class tables in addition to writing requirements that apply to that power class.

WF did not leave room for other approaches but there is one more way that would address all issues: to define a dedicated section, suffix or merely write separate sub-clauses for TxD UE’s and then in testing phase declare if UE implement TX diversity or not. This would allow RAN5 test procedures to be dedicated for TxD UE’s and not set back their progress in test validation for existing types of UE’s.

Proposal 2: Distinguish requirements for TX Diversity UE’s in some way from single Tx UE’s in RAN4 requirements.

## 2.7	CDD requirements

S-CDD was in RAN1 mind when they made the agreement on transparent Tx Diversity thus RAN4 should not preclude it in requirement setting. Inserting a cyclic delay to one of the branches improves the field operation significantly CDD impact should be analysed but it may not impact the setting of the requirements but more the test procedure. UE with and without CDD would still need to meet same output power and emission requirements so requirements themselves should not differ but there may need to some further discussion on how to accommodate S-CDD in verification of requirements so that the test procedure does not fail a UE because it implements S-CDD.

S-CDD idea is to create some time varying change in the transmissions between the antennas that smoothens the received signal. Without it, there can be a static null at the receiver antenna when the signals cancel each other. The issues with the CDD discussed in the FR2 context are related to the OTA nature of the conformance testing since OTA the signals combine. With Fr1 and conducted setup, the problem only comes if there is an analogue domain power combiner in the TE. Measuring everything per connector and then mathematically merging the result will avoid any problems with S-CDD.

Observation: Measuring power and emission per connector and then merging the result will enable S-CDD implementation to meet same requirements than an implementation without S-CDD.

# Conclusion

We discussed TxD handling based on previous agreements and made two proposals:

Proposal 1: RAN4 core requirements for TxD should enable intentionally set power difference between the tx connectors

Proposal 2: Distinguish requirements for TX Diversity UE’s in some way from single Tx UE’s in RAN4 requirements.

For the CDD issues, we made one observation

Observation: Measuring power and emission per connector and then merging the result will enable S-CDD implementation to meet same requirements than an implementation without S-CDD.

# References

[1]	R4-2011768, “WF on Rel-16 TxD”, Samsung, 3GPP TSG-RAN WG4 Meeting # 96-e, Electronic Meeting, 17 – 28 Aug, 2020

[2]	R4-2008465, ‘WF on Enabling Transparent TxD in Rel-16’, Qualcomm, 3GPP TSG-RAN WG4 Meeting #95-e

[3]	R4-2011519, Further Considerations on the EVM Definition for Antenna Ports Including Transparent Transmit Diversity,	Lenovo, Motorola Mobility, 3GPP TSG-RAN WG4 Meeting # 96-e, Electronic Meeting, 17 – 28 Aug, 2020