3GPP TSG-RAN WG4 meeting #97-e	R4-2014745

Electronic Meeting, 2 – 13 November 2020

Agenda Item:	7.9.4.2.3

Source:	Samsung

Title:	Simulation results for Single-DCI URLLC schemes

Document for:	Discussion

1	Introduction

RAN1 defined 5 single-DCI based transmission schemes, in which ‘Single-DCI based SDM scheme’ is introduced for eMBB transmission, meanwhile, the other 4 schemes are applied for URLLC transmission, which are FDM scheme A, FDM scheme B, TDM scheme A and inter-slot TDM respectively.

In RAN4 #96-e meeting, only FDM scheme A and inter-slot TDM scheme are determined as URLLC PDSCH performance requirements:

- FDM scheme A (2a)

  - Single PDSCH transmission occasion for one transport block scheduled

  - Single redundancy version

  - The PDSCH resources are non-overlapped in frequency domain, but overlapped in time domain

- Inter-slot TDM scheme (4) transmission schemes

  - Slot-based TDM, PDSCH from two TRPs are non-overlapped in time domain but overlapped in frequency domain

  - The number of repetitions is configured by RepNumR16

  - Multiple PDSCH transmission occasions for one transport block scheduled

  - Up to four redundancy versions

  - UE can combine PDSCH symbols from multiple PDSCH transmission occasions

In this contribution, we focus on FDM scheme A (2a), and provide corresponding performance based on the agreed WF in [1].

2	Test assumptions for Single-DCI URLLC scheme

RAN4 defined performance requirements for FDM A with two cases per duplex mode, and reuse parameters from single-DCI based SCM Tx scheme test case as shown in [2].

The principle parameters are presented as following.

Table 2.1 Simulation parameters for Single-DCI FDM A scheme

| Parameters | Value |
| --- | --- |
| MCS | 13/19 |
| Antenna configuration | 2x2 ULA Low for each TRP |
| DMRS configuration | Resource allocation Type 1, 1+1 Mapping Type: Type A One CDM group |
| Antenna port index | TCI #1: {1000} TCI #2: {1001} |
| PDSCH resource start symbol and time duration | PDSCH #1: 2,12 PDSCH #2: 2,12 |
| PDSCH resource allocation | - FDD - PDSCH #1: PRB #0 ~ PRB #25 - PDSCH #2: PRB #26 ~ PRB #51 - TDD - PDSCH #1: PRB #0 ~ PRB #52 - PDSCH #2: PRB #53 ~ PRB #105 |
| Redundancy version | Single RV in a slot |

The test metric for requirements is defined as:

- Option 1: 1% BLER which is more suitable for URLLC service

- Option 2: 70% SNR point at max throughput

3	Simulation results

- FDD MCS 19

Fig. 1: URLLC BLER for FDD MCS 19

Fig. 2: Throughput for FDD MCS 19

- FDD MCS13

Fig. 3: URLLC BLER for FDD MCS 13

Fig. 4: Throughput for FDD MCS 13

- TDD MCS 19

Fig. 5: URLLC BLER for TDD MCS 19

Fig. 6: Throughput for TDD MCS 19

- TDD MCS 13

Fig. 7: URLLC BLER for TDD MCS 13

Fig. 8: Throughput for TDD MCS 13

Table 3.1  SNR point with different cases

| Duplex mode | BW / SCS | MCS | Propagation condition | Frequency offset / Time offset | 70% of max Throughput (dB) | 1% BLER (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| FDD | 10MHz / 15kHz | 19 | [TDLA30-10] for both TRP1 and TRP2 | Test 1a: 200Hz /2us | 9.0 | 11.5 |
|  |  |  |  | Test 1a: 200Hz/-0.5us | 9.3 | 12 |
| FDD | 10MHz / 15kHz | 13 | [TDLA30-10] for both TRP1 and TRP2 | Test 1a: 200Hz / 2us | 4.1 | 7.5 |
|  |  |  |  | Test 1a: 200Hz / -0.5us | 4.3 | 7.8 |
| TDD | 40MHz / 30kHz | 19 | [TDLA30-10] for both TRP1 and TRP2 | Test 1a: 300Hz /1us | 10.5 | 12.5 |
|  |  |  |  | Test 1a: 300Hz/-0.25us | 11.1 | 13 |
| TDD | 40MHz / 30kHz | 13 | [TDLA30-10] for both TRP1 and TRP2 | Test 1a: 300Hz /1us | 3.5 | 5.6 |
|  |  |  |  | Test 1a: 300Hz/-0.25us | 3.7 | 5. |

The SNR point on condition that 70% of max throughput and 1% BLER are collected in table 3.1.

4	Summary

Observation 1: The SNR point following two metric options are summarized as in table 3.1.

References

- R4-2012663, “WF for PDSCH requirements with Single-DCI based multi-TRP/Panel transmission schemes (URLLC)”, Intel Corporation, RAN4 #96-e Electronic meeting.

- R4-2012664, “Simulation assumption for PDSCH requirements with Single-DCI SDM scheme and Multi-DCI transmission schemes”, Ericsson, RAN4 #96-e Electronic meeting

- R4-2010140, “Test case design for PDSCH requirements with Multi-TRP/Panel transmission”, Samsung, RAN4 #96-e Electronic Meeting

- R4-2010481, “PDSCH requirements for Multi-DCI/Single-DCI based transmission”, Ericsson, RAN4 #96-e Electronic Meeting