3GPP TSG-RAN4 Meeting #97-e	R4-2014902

# Online, , 2nd Nov 2020 - 13th Nov 2020

Agenda item:	10.25.1

Source:	Apple Inc.

Title:	A-MPR Proposal for n13

WI/SI:	NR_n13

Release:	Rel-17

Document for:	Decision

# 1	Introduction

A way forward was agreed in RAN4#96-e [1] to define n13 A-MPR based on contributions from [2] and [3]. This paper presents A-MPR values for NS_07 requirements.

# 2  Discussion

##### 2.1 Simulation assumptions

The following assumptions for the simulations were used:

- Power Class 3

- PA calibration point of 1dB MPR to meet general requirement for fully allocated DFT-s-OFDM QPSK waveform

- Carrier Leakage: 28dBc

- Image: 28dBc

- CIM3: 60dBc

- No filter rejection for 769-775MHz

##### 2.2 NS_07 requirements

Table 1: Additional SEM requirements NS_07

| ΔfOOB (MHz) | 5 MHz | 10 MHz | Measurement bandwidth |
| --- | --- | --- | --- |
| ± 0 – 0.1 | -15 | -18 | 30 kHz |
| ± 0.1 – 1 | -13 | -13 | 100 kHz |
| ± 1 – 6 | -13 | -13 | 1 MHz |
| ± 6 – 10 | -25 |  |  |
| ± 10 – 15 |  | -25 |  |

Table 2: Additional requirements NS_07

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit- (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5 MHz, 10 MHz |  |
| 769 ≤ f ≤ 775 | -57 | 6.25 kHz |
| NOTE 1: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. |  |  |

###### 2.3 A-MPR for NS_07

The simulations were done for special requirements of NS_07. Due to the nature of NS_07 some allocations require large A-MPR. Typically, a model can run into limitations in high backoff situations and may present too ideal and low values. In order to counter this issue a second model was used, designed for high backoff ranges. The results are provided in chapter 3 for selected allocations. Most simulations are in line with previous proposals. The second model shows slightly increased values for A1, compared to WF [1].

Table 3: Contiguous allocation A-MPR for NS_07

| Channel BW (MHz) | Carrier Frequency (MHz) | RBstart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5 | 782 ≤ fc ≤ 784.5 | > 0 | ≥ 1.8 | A3 |
| 5 | 779.5 ≤ fc ≤ 782 | ≤ 0.9 | ≥ 0 | A1 |
|  |  | > 0.9, ≤ 1.26 | ≥ 1.26 | A2 |
|  |  | > 1.26, ≤ 3.42 | ≥ 1.8 | A3 |
|  |  | > 3.42 | ≤ 0.36 | A4 |
| 10 | Fc = 782 | ≤ 2.34 | ≥ 0 | A1 |
|  |  | > 2.34, ≤ 3.24 | ≥ 1.44 | A2 |
|  |  | > 3.24 ≤ 6.48 | ≥ 3.24 | A3 |
|  |  | > 6.48 | ≤ 0.36 | A4 |

Table 4: A-MPR for modulation and waveform type for NS_07

| Waveform | Modulation | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- | --- |
|  |  | Outer / Inner | Outer / Inner | Outer / Inner | Outer / Inner |
| DFT-s-OFDM | PI/2 BPSK | 14 | 9 | 6 | 3 |
|  | QPSK | 14 | 9 | 6 | 3 |
|  | 16QAM | 14 | 9 | 6 | 3 |
|  | 64QAM | 14 | 9 | 6 | 3 |
|  | 256QAM | 14 | 9 | 6 | 3 |
| CP-OFDM | QPSK | 15 | 10 | 7 | 3 |
|  | 16QAM | 15 | 10 | 7 | 3 |
|  | 64QAM | 15 | 10 | 7 | 3 |
|  | 256QAM | 15 | 10 | 7 | 3 |

Proposal: Use values found in Table 3 and Table 4 to define n13 A-MPR for NS_07.

# 3  Simulations

###### 3.1 Simulations results for BW = 5MHz

| Low backoff model | High backoff model (only selected allocations) |
| --- | --- |

| Low backoff model | High backoff model (only selected allocations) |
| --- | --- |

| Low backoff model | Low backoff model |
| --- | --- |

###### 3.2 Simulation results for BW = 10MHz

| Low backoff model | High backoff model (only selected allocations) |
| --- | --- |

| Low backoff model | High backoff model (only selected allocations) |
| --- | --- |

# 4	Conclusions

Proposal: Use values found in Table 3 and Table 4 to define n13 A-MPR for NS_07.

# 4	References

- R4-2011801, “WF on n13 A-MPR “, Qualcomm, Huawei,  3GPP TSG-RAN WG4 Meeting RAN#96-e.

- R4-2010490, “A-MPR for n13”, Huawei, HiSilicon, 3GPP TSG-RAN WG4 Meeting # 96-e

- R4-2008209, “n13 AMPR”, Qualcomm Incorporated, 3GPP TSG-RAN WG4 #95-e