3GPP RAN-WG4 Meeting #97-e		R4-2014921

Electronic meeting, November 2 – 13, 2020

Agenda item:	13.1.3

Source:	Apple Inc.

Title:	Impact of AoA offset on inter-band CA PSD difference

WI/SI:	FS_FR2_enhTestMethods

Release:	Rel-17

Document for:	Discussion

# 1	Introduction

The concept of utilizing offset antennas to perform blocking and inter-band CA tests in the FR2 RF measurement setup was proposed in [1] and [2].  During the RAN4 #96e meeting a set of simulation assumptions was agreed to investigate the impact of offset antennas on the spatially filtered PSD difference at the DUT [3].

This contribution provides our analysis of the topic.

# 2	Discussion

Following a similar simulation methodology as described in [4], an analysis of the impact of an AoA offset ranging from 0 to 7 degrees was analyzed.  The gain difference between the CCs is a test case parameter informed by the recent agreement on the PSD difference in the REFSENS and EIS spherical coverage requirement for DL inter-band CA within FR2 [38.101-2].  The simulation assumptions are shown in Table 1 below.

Table 1: Simulation assumptions for PSD imbalance

| Simulation Parameters |  |
| --- | --- |
| Antenna array size | 4x1 |
| Element spacing | 5mm |
| Element pattern | See TR38.803 |
| Antenna impairments | Not considered |
| Phase shifter impairments | See TR38.817-01 |
| Transmission line impairments | Modeled TL length and loss per element |
| AoA offset | {0, 2, 5, 7} deg |
| Beam management assumptions | Independent beam management on each CC |
| Center frequencies of component carriers | f1 = 27.9 GHz, f2 = 38.5 GHz |
| Gain difference between CC1 and CC2 | 15.2 dB |

For each AoA offset the PSD difference between CC1 and CC2 was calculated after spatially filtering the signal with the array response pattern corresponding to each CC.  Figure 1 below illustrates the simulation results.

Figure 1: Spatially filtered PSD difference vs. angular separation between CCs

Table 2 below summarizes the simulation results.

Table 2: Simulation results for spatially filtered PSD difference vs. angular separation between CCs

| Max excess PSD difference due to AoA offset (dB) |  |
| --- | --- |
| AoA offset (º) | PSD diff (dB) |
| 2 | 0.4 |
| 5 | 1.1 |
| 7 | 1.7 |

As shown in [4], and eventually captured in the specification, the requirement for inter-band CA between bands n261 and n260 is defined assuming a non-zero PSD difference between the component carriers.  According to our understandng, this assumption holds for the class of inter-band CA configurations to which the independent beam management (IBM) type is applicale.

Observation 1:	At least for IBM inter-band CA requirements, AoA offsets of up to 7 degrees between two FR2 CA component carriers do not significantly impact the PSD difference assumpiton taken for the core requirement.

The above observation should be double-checked after the band n262 requirements are finalized and the scope of CA configurations with n262 are understood.

We further note that as part of the Rel-17 FR2 RF enhancement work item, RAN4 is discussing the potential definition of requirements assuming common beam management (CBM) for combination of certain band groups, such as 28+28 GHz.  An analysis of the impact of AoA offsets in the test setup for inter-band CA with CBM is needed after the scope of CBM requirements and associated agreements are better understood.

Proposal 1:	An OTA test system configuration with AoA offsets can be considered a permitted test system for IBM FR2 inter-band CA requirements: at least for CA between n261 and n260.

Proposal 2:	The applicability of a test system configuration with AoA offsets to band n262 test cases should be checked after band n262 requirements and scope of CA configurations with n262 are better understood.

Proposal 3:	The applicability of a test system configuration with AoA offsets to CBM test cases should be checked after the scope of CBM requirements for FR2 CA is better understood.

# 3	Conclusions

This contribution provides our view on the impact of a test environment with AoA offsets on the spatially filtered PSD perceived by the UE configured with CA in FR2.  We have made the following observation and proposals:

Observation 1:	At least for IBM inter-band CA requirements, AoA offsets of up to 7 degrees between two FR2 CA component carriers do not significantly impact the PSD difference assumpiton taken for the core requirement.

Proposal 1:	An OTA test system configuration with AoA offsets can be considered a permitted test system for IBM FR2 inter-band CA requirements: at least for CA between n261 and n260.

Proposal 2:	The applicability of a test system configuration with AoA offsets should be checked after band n262 requirements and scope of CA configurations with n262 are better understood.

Proposal 3:	The applicability of a test system configuration with AoA offsets should be checked after the scope of CBM requirements for FR2 CA is better understood.

# 4	References

- R4-2000445, “Consideration on capability of multi signal transmission from single AoA in FR2 OTA test system”, Anritsu Corporation, RAN4 #94e, February 2020

- R4-2003332, “Consideration on FR2 OTA test system with multi signal transmission,” Anritsu Corporation, RAN4 #94bis-e, April 2020

- R4-2012715, “WF on the inter-band CA within FR2 objective,” Anritsu Corporation, 3GPP RAN4 #97e, August 2020

- R4-2004700, “Views on PSD difference in FR2,” Apple Inc., 3GPP RAN4 #94bis-e, April 2020