3GPP TSG-RAN WG4 Meeting # 97-e                	R4-2014942

Electronic Meeting, 2-13 Nov., 2020

Source: 	Nokia, Nokia Shanghai Bell

Title: 	PUCCH Demodulation performance requirements for operation in unlicensed bands

Agenda item: 	7.1.8.4.2

Document for: 	Discussion

Introduction

This paper extends the discussion with Nokia’s views on PUCCH BS  demodulation requirements on [4] considering the outcome of RAN4 #96-e meeting on that topic [2][3]. Among the topics discussed on this paper, are which PUCCH formats to include in the performance requirements and the number of interlaces which should be used for testing.

PUCCH BS demodulation performance requirements

PUCCH formats

An overview on the PUCCH formats is shown in Table 1. From the 5 formats used in Rel. 15 PUCCH, 4 of those have interlaced designs in order to conform with regional OCB regulatory requirements [7], formats 0 to 3. On RAN4 demodulation requirements, the applicability of the tests depend on manufacturer declaration D.102.

Table 1 PUCCH formats (TS 38.211 [6])

| PUCCH format | Length in OFDM symbols | Number of bits |
| --- | --- | --- |
| 0 | 1 – 2 | ≤2 |
| 1 | 4 – 14 | ≤2 |
| 2 | 1 – 2 | >2 |
| 3 | 4 – 14 | >2 |
| 4 | 4 – 14 | >2 |

Demodulation requirements for Rel. 15 PUCCH formats are applicable depending on manufacturer declaration D.102.

RAN4 to define demodulation requirements to all interlaced PUCCH formats (i.e. formats 0, 1, 2, and 3), with NR-U specific applicability rule for the new formats.

Number of interlaces for PUCCH resource allocation

The number of PRBs on existing PUCCH performance requirements is described in Table 1.

Table 2 Summary of parameters used for Rel. 15 PUCCH performance requirements

| PUCCH format | Number PRBs | Number symbols | Information bits |
| --- | --- | --- | --- |
| 0 | 1 | 1 | 1 |
|  | 1 | 2 | 1 |
| 1 | 1 | 14 | 2 |
| 2 | 4 | 1 | 4 |
|  | 9 | 2 | 22 |
| 3 | 1 | 14 | 16 |
|  | 3 | 4 | 16 |
| 4 | 1 | 14 | 22 |

The maximum number of PRBs used on Rel. 15 PUCCH performance requirements is 9 for PUCCH format 2.

When using interlaced PUCCH in NR-U, the minimum allocation of 1 interlace consists of 10/11 PRBs.

When considering interlaced PUCCH in NR-U, the minimum allocation of 1 interlace includes more PRBs than any of the Rel. 15 PUCCH performance requirements in 38.104.

RAN4 to define performance requirements only for 1 interlace PUCCH.

Test scenario

In order to progress with the discussion on the test environment, we propose using Rel. 15 as a baseline for the discussion as in Table 3.

RAN4 to consider NR-U PUCCH performance requirements without frequency hopping.

RAN4 to consider QPSK modulation order tor NR-U PUCCH formats 2 and 3.

RAN4 to consider Rel.15 PUCCH requirements as a baseline for the discussion of the NR-U PUCCH test scenarios as in the table below:

Table 3 Proposed parameters for PUCCH testing

| PUCCH format | Number Interlaces | Number symbols | Information bits |
| --- | --- | --- | --- |
| 0 | 1 | 1 | 1 |
|  | 1 | 2 | 1 |
| 1 | 1 | 14 | 2 |
| 2 | 1 | 1 | 4 |
|  | 1 | 2 | 22 |
| 3 | 1 | 14 | 16 |
|  | 1 | 4 | 16 |

Conclusion

In this contribution we provided Nokia’s views on the BS demodulation requirements of PUCCH formats for operation on unlicensed bands. As a result of this discussion the following observations and conclusions are derived:

Observation 1: Demodulation requirements for Rel. 15 PUCCH formats are applicable depending on manufacturer declaration D.102.

Proposal 1: RAN4 to define demodulation requirements to all interlaced PUCCH formats (i.e. formats 0, 1, 2, and 3), with NR-U specific applicability rule for the new formats.

Observation 2: The maximum number of PRBs used on Rel. 15 PUCCH performance requirements is 9 for PUCCH format 2.

Observation 3: When using interlaced PUCCH in NR-U, the minimum allocation of 1 interlace consists of 10/11 PRBs.

Observation 4: When considering interlaced PUCCH in NR-U, the minimum allocation of 1 interlace includes more PRBs than any of the Rel. 15 PUCCH performance requirements in 38.104.

Proposal 2: RAN4 to define performance requirements only for 1 interlace PUCCH.

Proposal 3: RAN4 to consider NR-U PUCCH performance requirements without frequency hopping.

Proposal 4: RAN4 to consider QPSK modulation order tor NR-U PUCCH formats 2 and 3.

Proposal 5: RAN4 to consider Rel.15 PUCCH requirements as a baseline for the discussion of the NR-U PUCCH test scenarios as in the table below:

Proposed parameters for PUCCH testing

| PUCCH format | Number Interlaces | Number symbols | Information bits |
| --- | --- | --- | --- |
| 0 | 1 | 1 | 1 |
|  | 1 | 2 | 1 |
| 1 | 1 | 14 | 2 |
| 2 | 1 | 1 | 4 |
|  | 1 | 2 | 22 |
| 3 | 1 | 14 | 16 |
|  | 1 | 4 | 16 |

References

- RP-192926, Revised WID for Rel-16 NR-U WI

- R4-2012611, Way Forward on NR-U BS demodulation requirements, 3GPP TSG-RAN WG4 Meeting #96-e

- R4-2012542	Email discussion summary for [96e][328] NR_unlic_Demod, 3GPP TSG-RAN WG4 Meeting #96-e

- R4-2010905, Discussion on NR-U BS demodulation requirements, 3GPP TSG-RAN WG4 Meeting #96-e

- TS 38.104, V15.11.0, NR; Base Station (BS) radio transmission and reception

- TS 38.211, V16.3.0, NR; Physical channels and modulation

- ETSI EN 301 893 V2.1.1, 5 GHz RLAN; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU