3GPP TSG-RAN WG4 Meeting #97-e	R4-2015019

Electronic Meeting, 02th – 13th Nov., 2020

Agenda item:		7.10.1.1

Source:				ZTE

Title:		Propagation Condition for FR2 DL 256QAM

Document for:	Discussion

1. Introduction

In the WF from RAN4 #96e meeting[1], some aggrements we reached for 256QAM UE requirements but there are still some open issues.

This contribution presents our views on 256QAM UE requirements for reamining open issues from last meeting.

2. Discussion

Propagation condition was discussed in last meeting and a consensus which using fading channel and  further decide whether to use TDLA30-300 or TDLD30-75 has been reached. We provide the simulation result of TDLA and TDLD in table2.

Table 1: simulation parameters

| Parameters | Vaule |
| --- | --- |
| CBW and PRB Allocation | 50 MHz and Full allocation |
| SCS | 120 kHz |
| Propagation | TDL-D 30-75 TDL-A 30-300 |
| MCS | MCS 20 |
| Antenna configuration | 2x2 ULA Low |
| Rank | 1 |
| PDSCH configuration | DL slots: Start symbol 1, Duration 13 |
| PDSCH mapping type | Type A |
| DMRS configuration | Type 1 single symbol, 1 additional DMRS |
| PTRS configuration | every 2 PRB in frequency domain every 1 symbol in time domain |

Table 2: simulation result

| Channel Model | Ideal SNR( dB) @70%TP | Impairment SNR(dB)@70%TP |
| --- | --- | --- |
| TDLA30-300 | 18.4 | 21.4 |
| TDLD30-75 | 17.1 | 20.1 |

Because there is serious path loss and reflecting property in high frequency signal, in our opinion, a LOS scenario could more close to the actual application scenario. In the result of simulation, the performance of TDLD is better than TDLA. So we think TDLD could be suitable for propagation condition.

Proposal 1: Use TDLD30-75 fading channel.

3. Conclusion

In this contribution, we proivd our simulation result  on FR2 DL 256QAM  requirements. In summary, we make the following observation:

Table 2: simulation result

| Channel Model | Ideal SNR( dB) @70%TP | Impairment SNR(dB)@70%TP |
| --- | --- | --- |
| TDLA | 18.4 | 21.4 |
| TDLD | 17.3 | 20.1 |

Proposal 1: Use TDLD30-75 fading channel.

References

- R4-2012666, China Telecom, WF on FR2 DL 256QAM demod, RAN4 #96e