3GPP TSG-RAN WG4 Meeting #97-e	R4- 2015068

# Electronic Meeting, 2nd to 13th Nov, 2020

Agenda Item:	10.11.2

Source: 	MediaTek Inc.,

Title: 	MSD evaluation for CA 3DL2UL n1-n77-n79 for TR 38.717-03-02

Document for:	Approval

- 1. Introduction

The contribution is to characterize MSD for the CA 3DL2UL n1-n77-n79 in [1]

- 2. Discussion

5.1.x.4			REFSENS requirements

-	3rd, 4th and 5th  order IMD generated by dual uplink of CA_n1-n77 may fall into part of own band n79.

=>Same MSD apply with DC_1_n78-n79 specified in TS 38.101-3. Note that MSD for 4th and 5th order IMD are not specified in TS 38.101-3.

-	5th order IMD generated by dual uplink of CA_n1-n79 may fall into part of own band n77.

=>Same MSD apply with DC_1_n78-n79 specified in TS 38.101-3.

-	3rd, 4th and 5th order IMD generated by dual uplink of CA_n77-n79 may fall into part of own band n1.

Figure 1. FE architecture assumption

Figure 1 shows architecture assumption for the CA combination. Since there’s only 200MHz separate between n77 and n79, it is difficult for diplexer design with good enough isolation. And n77, n79 band filter also has almost no rejection on the other band. It is suggested to adopt architecture assumption as Figure 1. Calculation assumptions are listed in Table 1:

| Parameter | Value | Unit |
| --- | --- | --- |
| Antenna isolation | 10 | dB |
| Diplexer isolation | 17 | dB |
| n1 filter rejection at n77 range | 30 | dB |
| n1 filter rejection at n79 range | 30 | dB |
| n77 filter rejection at n1 range | 37 | dB |
| n77 filter rejection at n79 range | 5 | dB |
| n79 filter rejection at n1 range | 25 | dB |
| n79 filter rejection at n77 (4000-4200MHz) range | 0 | dB |
| n79 filter rejection at n77 (3300-4000MHz) range | 10 | dB |
| Front-end loss | 4 | dB |
| Thermal noise at n77/n79 RX ANT port | -165.8 | dBm/Hz |
| Transceiver effective phase noise | -150 | dBc/Hz |
| SNR requirement for QPSK | -1 | dB |

Table 1 Receiver performance parameters for MSD analysis

For MSD on n79 due to UL_CA_n1-n77, with the architecture assumption in Figure 1, considering n79 filter has very poor rejection to n77 UL, even the UL power is 20dBm (3dB lower for equal power sharing), the TX power into n79 receiver would be higher or than or equal to 0dBm that would saturate RX LNA. We are not sure if it is meaningful to specify such uplink configuration (UL_CA_n1-n77). This is same when uplink is UL_CA_n1-n79.

Observation 1: UE cannot support uplink configuration with UL_CA_n1-n77 and UL_CA_n1-n79

Since the n77 and n79 filter has almost no rejection for each other, the MSD would be dominated by RIMD mechanism that would need to be characterized by measurement bench. For MSD due to IMD3 when uplink is UL_CA_n77-n79 would be dominated by RIMD3 that requires measured data. Measured RIMD3 level is -37.9dBm at n77 PA output and -42.8dBm at n79 PA output. However due to n1 filter insufficient isolation to n77, calculated MSD is 13.3 dB (with considering BW scaling factor). For MSD due to IMD4, following same way, measured spur level are -50dBm and -60dBm and calculated MSD is 9.2dB. For MSD due to IMD5, following same way, measured spur level are -61dBm and -97dBm and calculated MSD is 8.4dB. Below Table 2 shows measured results and calculation:

|  | NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL CA Configuration |  | Victim NR band | Spur at PA1 | Spur at PA2 | Thermal noise | IMD BW correction factor | Total noise (P LNA) | Total noise (D LNA) | MSD | IMD Order |
| n77A-n79A |  | n1 | -42.8 | -37.9 | -99 | 10.77 | -86.7 | -86.7 | 14.8 | IMD3 |
| n77A-n79A |  | n1 | -50 | -60 | -99 | 13 | -90.8 | -90.8 | 13.8 | IMD4 |
| n77A-n79A |  | n1 | -61 | -97 | -99 | 13.4 | -98.6 | -91.6 | 13.7 | IMD5 |

Table 2 MSD due to IMD analysis

The Table one shows MSD for the CA configuration need to be characterized:

Proposal 1: MSD due to IMD is proposed as below table and corresponding TP is in [2]:

| NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| CA_n1A-n77A-n79A | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n77 | 3410 | 10 | 50 | 3410 | N/A | N/A |
|  | n79 | 4870 | 40 | 216 | 4870 | 15.9 (NA) | IMD3 |
|  | n1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n77 | 4160 | 10 | 50 | 4160 | N/A | N/A |
|  | n79 | 4460 | 40 | 216 | 4460 | (NA) | IMD4 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n77 | 3310 | 10 | 50 | 3310 | N/A | N/A |
|  | n79 | 4490 | 40 | 216 | 4490 | (NA) | IMD5 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n77 | 3490 | 10 | 50 | 3490 | (NA) | IMD5 |
|  | n79 | 4670 | 40 | 216 | 4670 | N/A | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | 14.8 | IMD3 |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | N/A |
|  | n79 | 4660 | 40 | 216 | 4660 | N/A | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | 13.8 | IMD4 |
|  | n77 | 3600 | 10 | 50 | 3600 | N/A | N/A |
|  | n79 | 4670 | 40 | 216 | 4670 | N/A | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | 13.7 | IMD5 |
|  | n77 | 3660 | 10 | 50 | 3660 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |

Table 3: MSD for the CA configuration

Corresponding TP is in [2]

- 3. Conclusion

- From the discussion above, we have following observations and proposal:

Observation 1: UE cannot support uplink configuration with UL_CA_n1-n77 and UL_CA_n1-n79

Proposal 1: MSD due to IMD is proposed as below table and corresponding TP is in [2]:

| NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| CA_n1A-n77A-n79A | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n77 | 3410 | 10 | 50 | 3410 | N/A | N/A |
|  | n79 | 4870 | 40 | 216 | 4870 | 15.9 (NA) | IMD3 |
|  | n1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n77 | 4160 | 10 | 50 | 4160 | N/A | N/A |
|  | n79 | 4460 | 40 | 216 | 4460 | (NA) | IMD4 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n77 | 3310 | 10 | 50 | 3310 | N/A | N/A |
|  | n79 | 4490 | 40 | 216 | 4490 | (NA) | IMD5 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n77 | 3490 | 10 | 50 | 3490 | (NA) | IMD5 |
|  | n79 | 4670 | 40 | 216 | 4670 | N/A | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | 14.8 | IMD3 |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | N/A |
|  | n79 | 4660 | 40 | 216 | 4660 | N/A | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | 13.8 | IMD4 |
|  | n77 | 3600 | 10 | 50 | 3600 | N/A | N/A |
|  | n79 | 4670 | 40 | 216 | 4670 | N/A | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | 13.7 | IMD5 |
|  | n77 | 3660 | 10 | 50 | 3660 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |

Reference:

- R4-2010580, TP for CA 3DL2UL n1-n77-n79 for TR 38.717-03-02. RAN4#96-e

- R4-2014595 TP for CA 3DL2UL n1-n77-n79 for TR 38.717-03-02, RAN4#97-e