3GPP TSG-RAN WG4 Meeting #97-e              	R4-2015118

Electronic Meeting, 2nd Nov– 13th Nov, 2020

Agenda item:	7.15.3.2.1

Source:	Samsung

Title:	Simulation results for NR HST PUSCH

Document for:	Discussion

# 1	Introduction

During the last RAN4 meeting, the remaining issues for NR HST BS PUSCH demodulation requirement were discussed and test parameters for related requirement were captured in the WF [1].

In this contribution, the updated simulation results are provided for performance requirement derived.

# 2	Simulation Results

2. 1 Requirements for 350km/h

In this subsection, the updated ideal and impairment results for both open and tunnel scenario under 350km/h are provided for alignment.

Table 1: Simulation results for NR HST PUSCH with CP-OFDM for open scenario with 350km/h

| SCS& BW | MCS | Tx/Rx | DMRS configuration | Channel | SNR(@70%) (ideal ) | SNR(@70%) (impair ) |
| --- | --- | --- | --- | --- | --- | --- |
| 15KHz, 5MHz | 2 | 1T2R | 1+1+1(2 or 3,7,11) | AWGN 1340Hz | -5.87 | -3.87 |
| 15KHz, 5MHz | 16 | 1T2R | 1+1+1(2 or 3,7,11) | AWGN 1340Hz | 6.96 | 8.96 |
| 15KHz, 5MHz | 2 | 1T8R | 1+1+1(2 or 3,7,11) | AWGN 1340Hz | -11.18 | -9.18 |
| 15KHz, 5MHz | 16 | 1T8R | 1+1+1(2 or 3,7,11) | AWGN 1340Hz | 1.14 | 3.14 |
| 30KHz,10MHz | 2 | 1T2R | 1+1+1(2 or 3,7,11) | AWGN 2334Hz | -5.92 | -3.92 |
| 30KHz,10MHz | 16 | 1T2R | 1+1+1(2 or 3,7,11) | AWGN 2334Hz | 6.62 | 8.62 |
| 30KHz,10MHz | 2 | 1T8R | 1+1+1(2 or 3,7,11) | AWGN 2334Hz | -11.18 | -9.18 |
| 30KHz,10MHz | 16 | 1T8R | 1+1+1(2 or 3,7,11) | AWGN 2334Hz | 0.87 | 2.87 |

Table 2: Simulation results for NR HST PUSCH with CP-OFDM for tunnel scenario with 350km/h

| SCS& BW | MCS | Tx/Rx | DMRS configuration | Channel | SNR(@70%) (ideal) | SNR(@70%) (impair) |
| --- | --- | --- | --- | --- | --- | --- |
| 15KHz, 5MHz | 2 | 1T1R | 1+1+1(2 or 3,7,11) | AWGN 1340Hz | -2.81 | -0.81 |
| 15KHz, 10MHz | 2 | 1T1R | 1+1+1(2 or 3,7,11) | AWGN 1340Hz | -2.83 | -0.83 |
| 15KHz, 5MHz | 2 | 1T2R | 1+1+1(2 or 3,7,11) | AWGN 1340Hz | -5.80 | -3.80 |
| 15KHz, 5MHz | 16 | 1T2R | 1+1+1(2 or 3,7,11) | AWGN 1340Hz | 7.53 | 9.53 |
| 30KHz,10MHz | 2 | 1T1R | 1+1+1(2 or 3,7,11) | AWGN 2334Hz | -2.88 | -0.88 |
| 30KHz, 40MHz | 2 | 1T1R | 1+1+1(2 or 3,7,11) | AWGN 2334Hz | -2.82 | -0.82 |
| 30KHz, 10MHz | 2 | 1T2R | 1+1+1(2 or 3,7,11) | AWGN 2334Hz | -5.87 | -3.87 |
| 30KHz, 10MHz | 16 | 1T2R | 1+1+1(2 or 3,7,11) | AWGN 2334Hz | 7.00 | 9.00 |

2. 2 Requirements for 500km/h

In this subsection, the updated ideal and impairment results for both open and tunnel scenario under 500km/h are provided for alignment.

Table 3: Simulation results for NR HST PUSCH with CP-OFDM for open scenario with 500km/h

| SCS& BW | MCS | Tx/Rx | DMRS configuration | Channel | SNR(@70%) (ideal) | SNR(@70%) (impairment) |
| --- | --- | --- | --- | --- | --- | --- |
| 15KHz, 5MHz | 2 | 1T2R | 1+1+1(2 or 3,7,11) | AWGN 1740Hz | -5.83 | -3.83 |
| 15KHz,5MHz | 16 | 1T2R | 1+1+1(2 or 3,7,11) | AWGN 1740Hz | 7.33 | 9.33 |
| 15KHz, 5MHz | 2 | 1T8R | 1+1+1(2 or 3,7,11) | AWGN 1740Hz | -11.11 | -9.11 |
| 15KHz,5MHz | 16 | 1T8R | 1+1+1(2 or 3,7,11) | AWGN 1740Hz | 1.44 | 3.44 |
| 30KHz, 10MHz | 2 | 1T2R | 1+1+1(2 or 3,7,11) | AWGN 3334Hz | -5.87 | -3.87 |
| 30KHz, 10MHz | 16 | 1T2R | 1+1+1(2 or 3,7,11) | AWGN 3334Hz | 7.01 | 9.01 |
| 30KHz, 10MHz | 2 | 1T8R | 1+1+1(2 or 3,7,11) | AWGN 3334Hz | -11.14 | -9.14 |
| 30KHz, 10MHz | 16 | 1T8R | 1+1+1(2 or 3,7,11) | AWGN 3334Hz | 1.18 | 3.18 |

Table 4: Simulation results for NR HST PUSCH with CP-OFDM for tunnel scenario with 500km/h

| SCS& BW | MCS | Tx/Rx | DMRS configuration | Channel | SNR(@70%) (ideal) | SNR(@70%) (impairment) |
| --- | --- | --- | --- | --- | --- | --- |
| 15KHz, 5MHz | 2 | 1T1R | 1+1+1(2 or 3,7,11) | AWGN 1740Hz | -2.68 | -0.68 |
| 15KHz,10MHz | 2 | 1T1R | 1+1+1(2 or 3,7,11) | AWGN 1740Hz | -2.69 | -0.69 |
| 15KHz, 5MHz | 2 | 1T2R | 1+1+1(2 or 3,7,11) | AWGN 1740Hz | -5.68 | -3.68 |
| 15KHz, 5MHz | 16 | 1T2R | 1+1+1(2 or 3,7,11) | AWGN 1740Hz | 8.53 | 10.53 |
| 30KHz,10MHz | 2 | 1T1R | 1+1+1(2 or 3,7,11) | AWGN 3334Hz | -2.77 | -0.77 |
| 30KHz,40MHz | 2 | 1T1R | 1+1+1(2 or 3,7,11) | AWGN 3334Hz | -2.67 | -0.67 |
| 30KHz, 10MHz | 2 | 1T2R | 1+1+1(2 or 3,7,11) | AWGN 3334Hz | -5.76 | -3.76 |
| 30KHz,10MHz | 16 | 1T2R | 1+1+1(2 or 3,7,11) | AWGN 3334Hz | 8.01 | 10.01 |

2. 3 Requirements for fading channel with high Doppler

In the last meeting, RAN4 agreed to introduce the PUSCH requirement with CP-OFDM waveform for fading channel under high Doppler scenario. In this subsection, the ideal and impairment results are provided for alignment.

Table 5: Ideal and impairment results for NR HST PUSCH with CP-OFDM for fading channel

| SCS& BW | MCS | Tx/Rx | DMRS configuration | Channel | Frequency Offset (Hz) | SNR(@70%) (ideal) | SNR(@70%) (impairment) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 15KHz, 5MHz | 2 | 1T2R | 1+1+1(2 or 3,7,11) | TDLC300-600 | 0 | -3.37 | -1.37 |
| 30KHz, 10MHz | 2 | 1T2R | 1+1+1(2 or 3,7,11) | TDL300-1200 | 0 | -3.40 | -1.40 |

# 3	Conclusion

In this contribution, the updated simulation results for NR HST PUSCH requirement are provided.

# References

R4-2012676 WF on Rel-16 NR HST BS demodulation requirements, Nokia