3GPP TSG RAN4 Meeting #97e	    R4-2015288

# Electronic Meeting, November 2 - 13, 2020

Title: 	Discussion on the new SUL band n97 UE requirements

Source: 	Huawei, HiSilicon

Agenda item:	10.27.1

Document for:	Discussion

# Introduction

In the June 2020 plenary meeting, a new WI to introduce a new SUL band for 2300 – 2400MHz is approved in [1]. The new proposed SUL band overlaps with the exact frequencies of TDD band n40. With the previous experiences in introducing the SUL band n95 which overlaps with the TDD band n34, it is straightforward that RAN4 introduces the corresponding RF requirements to facilitate complete introduction of the new SUL band n97.

For UE RF requirements, introducing SUL band n97 involves the items listed in the below box which are all specified in TS 38.101-1:

UE RF requirements - Operating bands - UE channel bandwidth - Channel raster - UE MOP and MPR/AMPR - Spurious emissions

For the BS requirements, introducing SUL band n97 involves the BS coexistence requirements: Additional spurious emission and co-location emission requirements. The related specs are: TS 38.104, 38.141-1/-2, 36.104, 36.141, 37.104, 37.141, 37.105, 37.145-1/-2. A parallel discussion paper in [2] discusses the BS requirements correspondingly.

Thorough discussion was seen in the last meeting while the only remaining issue is how to put a note in the operating band table to clarify that the new SUL band n97 is not interfering with the corresponding TDD bands 40/n40 in terms of cross UL/DL interferences. In this paper, we discuss the necessary UE RF requirements to be specified in order to accomplish introduction of the new SUL band n97.

# UE RF Requirements

Operating band number

It is consensus in the last WG meeting that the new SUL band is to be introduced for 2300 – 2400MHz as band n97.

Table 5.2-1: NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n97 | 2300 MHz – 2400 MHz | N/A | SUL |

Proposal 1:  Introduce the new SUL band for 2300 – 2400MHz as band n97.

UE channel bandwidth

It is natural that for the new SUL band n97, the UE supporting this band should support all the channel bandwidths specified for band n40 as listed below, since n40 and n97 have the exact same frequencies for UL. To note that for BS, supported channel bandwidths also include 100MHz in addition to those of the UE channel bandwidths.

Table 5.3.5-1 Channel bandwidths for each NR band

| NR band / SCS / UE Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n97 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |
|  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |

Channel raster

The channel raster for n97 should follow exactly the one for band n40 UL.

Table 5.4.2.3-1: Applicable NR-ARFCN per operating band

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n97 | 100 | 460000 – <20> – 480000 | N/A |

MOP and A-MPR

The MOP requirements of PC3 for SUL band n97 should follow the ones for band n40.

Table 6.2.1-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n97 |  |  |  |  | 23 | ±2 |

For MPR requirements, the generic ones apply to n97.

For A-MPR requirements, it is reasonable that band n97 should follow band n40 (a default NS_01).

Table 6.2.3.1-1A: Mapping of network signaling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n97 | NS_01 |  |  |  |  |  |  |  |

Spurious emissions

Regarding spurious emissions, band n96 should have the same requirements as those for band n39 as below.

Table 6.5.3.2-1: Requirements for spurious emissions for UE co-existence

| NR Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n40, n97 | E-UTRA Band 1, 3, 5, 7, 8, 20, 22, 26, 27, 28, 31, 32, 33, 34, 38, 39, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 74, 75, 76, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |

Proposal 2:  Specify UE RF requirements for the new SUL band n97 following band n40.

# Conclusions

In this paper, we discuss the necessary RF requirements to be specified in order to accomplish introduction of the new SUL band n97. We propose:

Proposal 1:  Introduce the new SUL band for 2300 – 2400MHz as band n97.

Proposal 2:  Specify UE RF requirements for the new SUL band n97 following band n40.

# References

[1] RP-201363, New WID on introduction of 1880-1920MHz SUL (supplemental uplink) band for NR, CMCC

[2] R4-2015289, Discussion on the new SUL band n97 BS requirements, Huawei, HiSilicon