3GPP TSG-RAN WG4 Meeting # 97-e                                  		 R4-2015392

Electronic Meeting, 2nd – 13th November, 2020

Source:	Huawei, HiSilicon

Title:	TP for TR 36.717-02-01 CA_2A-8A

Agenda item:	14.1.2

Document for:	Approval

Background

This contribution provides text proposal on the LTE CA configuration CA_2A-8A as defined in the revised WID [1].

Reference

[1]	RP-201754, “Revised WID: Rel17 LTE inter-band CA for 2 bands DL with 1 band UL”, Qualcomm Incorporated

Text Proposal

---Start of changes---

5.x	CA_2-8

5.x.1	Channel bandwidths per operating band for CA

Table 5.x.1-1: Inter-band CA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
| 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |

Table 5.x.1-2: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_2A-8A | - | 2 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |

5.x.2 	Co-existence studies

Table 5.x.2-1 and 5.x.2-2 summarize frequency ranges where harmonics and/or harmonics mixing occur for CA _ 2-8.

Table 5.x.2-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 2 | 1850 | 1910 | 1930 | 1990 | 3700 | 3820 | 5550 | 5730 | 7400 | 7640 |
| 8 | 880 | 915 | 925 | 960 | 1760 | 1830 | 2640 | 2745 | 3520 | 3660 |

Table 5.x.2-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 2 | 1850 | 1910 | 1930 | 1990 | 3860 | 3980 | 5790 | 5970 | 7720 | 7960 |
| 8 | 880 | 915 | 925 | 960 | 1850 | 1920 | 2775 | 2880 | 3700 | 3840 |

The 2nd harmonic of band 8 Rx falls into frequency range of band 2 Tx. There is a harmonic mixing issue for CA_2A-8A.

5.x.3	∆TIB and ∆RIB values

Table 5.x.3-1: IB,c

| CA_2-8 | 2 | 0.3 |
| --- | --- | --- |
|  | 8 | 0.3 |

Table 5.x.3-2: R IB,c

| CA_2-8 | 2 | 0 |
| --- | --- | --- |
|  | 8 | 0 |

5.x.4 	REFSENS

The MSD due to harmonic mixing is TBD for CA_2-8.

---End of changes---