3GPP TSG-RAN WG4 Meeting # 97-e                                  R4-2015408

Electronic Meeting, 2nd – 13th November, 2020

Source: 	Huawei, HiSilicon

Title: 	TP for TR 37.717-31-11: DC_3A-8A-40A_n1A / DC_3A-8A-40C_n1A

Agenda Item:	10.5.2

Document for:	Approval

Introduction

The WID for NR DC was updated in RAN #89e meeting. This contribution provides a TP for TR 37.717-31-11 to finish the UE RF requirements for the band combination.

- References

[1]	RP-201572, “Revised WID on Rel-17 Dual Connectivity (DC) of 3 bands LTE inter-band CA (3DL/1UL) and 1 NR band (1DL/1UL)”, Ericsson

- Text Proposal

<TP for TR 37.717-31-11>

- 5.1.x	DC_3-8-40_n1

5.1.x.1	 Configurations for EN-DC

Table 5.1.x.1-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-8A-40A_n1A DC_3A-8A-40C_n1A | DC_3A_n1A DC_8A_n1A DC_40A_n1A |

5.1.x.2	 ∆TIB and ∆RIB values

Table 5.1.x.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-8-40_n1 | 3 | 0.5 |
|  | 8 | 0.5 |
|  | 40 | 0.6 |
|  | n1 | 0.5 |

Table 5.1.x.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-8-40_n1 | 3 | 0 |
|  | 8 | 0 |
|  | 40 | 0.2 |
|  | n1 | 0.1 |

5.1.x.3		Reference sensitivity exceptions

REFSENS exceptions are not needed.

<End of TP >