3GPP TSG-RAN WG4 Meeting # 97-e                                  		R4-2015429

Electronic Meeting, 2nd – 13th November, 2020

Source:	Huawei, HiSilicon

Title:	TP to TR 38.717.02-01 for CA_n8A-n28A

Agenda item:	10.2.2

Document for:	Approval

Background

This contribution provides text proposal on the NR CA band combination CA_n8A-n28A as defined in New WID on NR Inter-band Carrier Aggregation/Dual Connectivity  for 2 bands DL with x bands UL (x=1,2) RP-201539 [1].

Text Proposal

---Start of changes---

6.X	CA_n8-n28

6.X.1	Common for 1 band UL and 2 bands UL CA

6.X.1.1 Operating bands for CA

Table 6.X.1.1-1: CA band combination CA_n8A-n28A

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n8-n28 | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

6.X.1.2	Channel bandwidths per operating band for CA

Table 6.X.1.2-1: Supported bandwidths per CA band combination CA_n8A-n28A

|  | CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration |  | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n8A-n28A |  | - | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.X.1.3	UE Co-existence studies

Table 6.X.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n8-n28.

Table 6.X.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n8 | 880 | 915 | 925 | 960 | 1760 | 1830 | 2640 | 2745 | 3520 | 3660 |
| n28 | 703 | 748 | 758 | 803 | 1406 | 1496 | 2109 | 2244 | 2812 | 2992 |

Based on above table, there is no harmonic interference.

Table 6.X.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n8 | 880 | 915 | 925 | 960 | 1850 | 1920 | 2775 | 2880 | 3700 | 3840 |
| n28 | 703 | 748 | 758 | 803 | 1516 | 1606 | 2274 | 2409 | 3032 | 3212 |

Based on above table, there is no harmonic mixing issue.

6.X.1.4	∆TIB and ∆RIB values

For CA_n8A-n28A, the TIB,c and RIB are given in the tables below reusing the values of DC_8_n28.

Table 6.X.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n8-n28 | n8 | 0.6 |
|  | n28 | 0.5 |

Table 6.X.1.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n8-n28 | n8 | 0.2 |
|  | n28 | 0.2 |

6.X.1.5	REFSENs requirements

There is no MSD exception requirement for CA_n8-n28.

6.X.1.6	OOB blocking exception requirements

There is no MSD exception requirement for CA_n8-n28.

---End of changes---

Reference

[1]	RP-201539, “Rel-17 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2)”, ZTE Corporation