3GPP TSG-RAN4 Meeting #97-e	R4-2015491

# Electronic Meeting, 2-13 Nov., 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 1278 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on CSSF definition for CSI-RS based measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CSIRS_L3meas-Core |  |  |  |  |  | Date: |  |  | 2020-10-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | [R4-2012181] on CSSF for CSI-RS based measurement was endorsed in RAN4#96e. Some correction and clarification are made on top of [R4-2012181]: - It is agreed that Core/performance requirements in TS38.133 are specified based on the assumption that UE does not support simultaneous reception of CSI-RS and SSB. It means that CSI-RS L3 measurement and SSB based measurement are time division. It is also agreed that Number of SSB layers should include SSB for mobility and that as associatedSSB for CSI-RS mobility. Based on the above background, for CSSFoutsidegap, take EN-DC with FR1 only CA as an example, if SCell#1 is configured with both ssb-ConfigMobility and csi-rs-ResourceConfigMobility, SCell#2 is configured with csi-rs-ResourceConfigMobility only, SCell#3 is configured with ssb-ConfigMobility only, and there is one inter-frequency layer without gap, then SCell#1 and SCell#2 are regarded as 2 MOs including SSB and CSI-RS. The CSSF for each candidate shall be [2(for SCell#1) +2(for SCell#2)+ 1(for SCell#3)+1 (for inter-frequency layer w/o gap)]. - Make some clarification on SSB MOs The number of SSB measurement object shall include the total number of MOs with - ssb-ConfigMobility configured, or - ssb-ConfigMobility not configured but csi-rs-ResourceConfigMobility configured with associatedSSB. If ssbfrequency, smtc1, smtc2 and ssbSubcarrierSpacing are same in multiple MOs, the multiple MOs are counted as one SSB measurement object. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the CSSFoutsidegap. take EN-DC with FR1 only CA as an example, the scaling factor for FR1 SCC shall be: “2*Number of configured FR1 SCell(s) with both SSB and CSI-RS L3 configured +2*Number of configured FR1 SCell(s) if only CSI-RS L3 measurement configured + Number of configured FR1 SCell(s) with only SSB configured+Y” - Make some clarification on SSB MOs The number of SSB measurement object shall include the total number of MOs with - ssb-ConfigMobility configured, or - ssb-ConfigMobility not configured but csi-rs-ResourceConfigMobility configured with associatedSSB. If ssbfrequency, smtc1, smtc2 and ssbSubcarrierSpacing are same in multiple MOs, the multiple MOs are counted as one SSB measurement object. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CSI-RS measurement requirements are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

9.1.5	Carrier-specific scaling factor

This clause specifies the derivation of carrier-specific scaling factor (CSSF) values, which scales the measurement delay requirements given in clause 9.2, 9.3, 9.4, and NR PRS-based positioning measurements in clause 9.9 when UE is configured to monitor multiple measurement objects. The CSSF values are categorized into CSSFoutside_gap,i and CSSFwithin_gap,i, for the measurements conducted outside measurement gaps and within measurement gaps, respectively.

9.1.5.1	Monitoring of multiple layers outside gaps

The carrier-specific scaling factor CSSFoutside_gap,i for measurement object i derived in this chapter is applied to following measurement types:

-	SSB-based intra-frequency measurement with no measurement gap in clause 9.2.5, when none of the SMTC occasions of this intra-frequency measurement object are overlapped by the measurement gap.

-	SSB-based intra-frequency measurement with no measurement gap in clause 9.2.5, when part of the SMTC occasions of this intra-frequency measurement object are overlapped by the measurement gap.

-	CSI-based intra-frequency measurement with no measurement gap in clause 9.10.2, when none of CSI-RS resources for L3 measurement of this intra-frequency measurement object are overlapped by the measurement gap.

-	CSI-based intra-frequency measurement with no measurement gap in clause 9.10.2, when all CSI-RS resources for L3 measurement of this intra-frequency measurement object are partially overlapped by the measurement gap.

-	SSB-based inter-frequency measurement with no measurement gap in clause 9.3.9, when none of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap, if UE supports interFrequencyMeas-NoGap-r16 and the flag interFrequencyConfig-NoGap-r16 is configured by the Network.

-	SSB-based inter-frequency measurement with no measurement gap in clause 9.3.9, when part of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap, if UE supports interFrequencyMeas-NoGap-r16 and the flag interFrequencyConfig-NoGap-r16 is configured by the Network.

UE is expected to conduct the measurement of this measurement object i only outside the measurement gaps.

The number of SSB measurement object shall include the total number of MOs with

-	ssb-ConfigMobility configured, or

-	ssb-ConfigMobility not configured but csi-rs-ResourceConfigMobility configured with associatedSSB.

If ssbfrequency, smtc1, smtc2 and ssbSubcarrierSpacing are same in multiple MOs, the multiple MOs are counted as one SSB measurement object.

If the higher layer signaling in TS 38.331 [2] of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, CSSFoutside_gap,i and requirements derived from CSSFoutside_gap,i are not specified.

The UE cell identification and measurement periods derived based on CSSFoutside_gap,i in clauses 9.2.5.1, 9.2.5.2 may be extended for measurement objects of which the cell identification and measurement periods are overlapped with Tmeasure_SFTD1 specified in clause 9.3.8 when no measurement gaps are provided.

The requirements in this clause apply provided that

- 	There are only SCCs in FR2, or

- 	The SMTC on all CCs in FR2 have the same offset, and one of following conditions is met

-	If smtc2 is configured on any FR2 CC,

-	All CCs have the same configuration for smtc1, and

-	All CCs configured with smtc2 have the same configuration for smtc2

-	If smtc2 is not configured on any FR2 CC,

-	The total number of different SMTC periodicities on all serving CCs does not exceed 4.

-   All CSI-RS resources in the same MO are confined within a periodic 5ms window.

Note:	Longer delays for cell identification and measurement periods derived based on CSSFoutside_gap,i in clauses 9.2.5.1, 9.2.5.2, can be expected, if the UE is configured with more than 4 different SMTC periodicities on FR2 serving carriers. The longer delay applies for the FR2 intra-frequency measurement objects with the longest SMTC periodicity/periodicities.

9.1.5.1.1	EN-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS L3 measurements performed outside gaps

For UE configured with the E-UTRA-NR dual connectivity operation, the carrier-specific scaling factor CSSFoutside_gap,i for intra-frequency SSB based measurements and inter-frequency SSB-based measurements performed outside measurements gaps and intra-frequency CSI-RS L3 measurements will be as specified in Table 9.1.5.1.1-1.

Table 9.1.5.1.1-1: CSSFoutside_gap,i scaling factor for EN-DC mode

| Scenario | CSSFoutside_gap,i for FR1 PSCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PSCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required Note 2 | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gp |
| --- | --- | --- | --- | --- | --- | --- |
| EN-DC with FR1 only CA | 1: if only SSB measurement configured for PSCC; 2: if both SSB and CSI-RS L3 measurements configured or if only CSI-RS measurement configured for PSCC | 2*Number of configured FR1 SCell(s) with both SSB and CSI-RS L3 configured +2*Number of configured FR1 SCell(s) if only CSI-RS L3 measurement configured + Number of configured FR1 SCell(s) with only SSB configured+Y | N/A | N/A | N/A | 2*Number of configured FR1 SCell(s) with both SSB and CSI-RS L3 configured +2*Number of configured FR1 SCell(s) if only CSI-RS L3 measurement configured + Number of configured FR1 SCell(s) with only SSB configured+Y |
| EN-DC with FR2 only intra band CA | N/A | N/A | 1: if only SSB measurement configured for PSCC; 2: if both SSB and CSI-RS L3 measurements configured or if only CSI-RS measurement configured for PSCC | N/A | 2*Number of configured FR2 SCell(s) with both SSB and CSI-RS L3 configured +2*Number of configured FR2 SCell(s) if only CSI-RS L3 measurement configured + Number of configured FR2 SCell(s) with only SSB configured+Y | 2*Number of configured FR2 SCell(s) with both SSB and CSI-RS L3 configured +2*Number of configured FR2 SCell(s) if only CSI-RS L3 measurement configured + Number of configured FR2 SCell(s) with only SSB configured+Y |
| EN-DC with FR2 only inter band CA | N/A | N/A | 1: if only SSB measurement configured for PSCC; 2: if both SSB and CSI-RS L3 measurements configured or if only CSI-RS measurement configured for PSCC | 2 Note 3,5：if only SSB measurement configured for the SCC; 4 Note 3,5: if both SSB and CSI-RS L3 measurements configured or if only CSI-RS measurement configured for the SCC | 2×(2*Number of configured SCell(s) with both SSB and CSI-RS L3 configured +2*Number of configured SCell(s) if only CSI-RS L3 measurement configured + Number of configured SCell(s) with only SSB configured+Y-1) | 2×(2*Number of configured SCell(s) with both SSB and CSI-RS L3 configured +2*Number of configured SCell(s) if only CSI-RS L3 measurement configured + Number of configured SCell(s) with only SSB configured+Y-1) |
| EN-DC with FR1 +FR2 CA (FR1 PSCell) Note 1 | 1: if only SSB measurement configured for PSCC; 2: if both SSB and CSI-RS L3 measurements configured or if only CSI-RS measurement configured for PSCC | 2×(2*Number of configured SCell(s) with both SSB and CSI-RS L3 configured +2*Number of configured SCell(s) if only CSI-RS L3 measurement configured + Number of configured SCell(s) with only SSB configured+Y-1) | N/A | 2 Note 3：if only SSB measurement configured for the SCC; 4 Note 3: if both SSB and CSI-RS L3 measurements configured or if only CSI-RS measurement configured for the SCC | 2×(2*Number of configured SCell(s) with both SSB and CSI-RS L3 configured +2*Number of configured SCell(s) if only CSI-RS L3 measurement configured + Number of configured SCell(s) with only SSB configured+Y-1) | 2×(2*Number of configured SCell(s) with both SSB and CSI-RS L3 configured +2*Number of configured SCell(s) if only CSI-RS L3 measurement configured + Number of configured SCell(s) with only SSB configured+Y-1) |
| EN-DC with FR1 +FR2 CA (FR2 PSCell) Note 1 | N/A | 2*Number of configured SCell(s) with both SSB and CSI-RS L3 configured +2*Number of configured FR1 SCell(s) if only CSI-RS L3 measurement configured + Number of configured SCell(s) with only SSB configured+Y | 1: if only SSB measurement configured for PSCC; 2: if both SSB and CSI-RS L3 measurements configured or if only CSI-RS measurement configured for PSCC | N/A | 2*Number of configured SCell(s) with both SSB and CSI-RS L3 configured +2*Number of configured FR1 SCell(s) if only CSI-RS L3 measurement configured + Number of configured SCell(s) with only SSB configured+Y | 2*Number of configured SCell(s) with both SSB and CSI-RS L3 configured +2*Number of configured FR1 SCell(s) if only CSI-RS L3 measurement configured + Number of configured SCell(s) with only SSB configured+Y |
| Note 1: Only one NR FR1 operating band and one NR FR2 operating band are included for FR1+FR2 inter-band EN-DC. Note 2: Selection of FR2 SCC where neighbour cell measurement is required follows clause 9.2.3.2. Note 3: CSSFoutside_gap,i =1 if only one SCell with SSB only measurement is configured and no inter-frequency MO without gap; CSSFoutside_gap,i =2 if only one SCell with both SSB and CSI-RS L3 measurements is configured or if only one SCell with CSI-RS only measurement is configured and no inter-frequency MO without gap. Note 4: Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG Note 5: Only two NR FR2 operating band are included for EN-DC with FR2 only inter-band CA |  |  |  |  |  |  |

9.1.5.1.2	SA mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps

For UE in SA operation mode, the carrier-specific scaling factor CSSFoutside_gap,i for intra-frequency and inter-frequency SSB-based measurements and intra-frequency CSI-RS L3 measurement performed outside measurements gaps will be as specified in Table 9.1.5.1.2-1, which shall also be applied for a UE configured with NE-DC operation.

Table 9.1.5.1.2-1: CSSFoutside_gap,i scaling factor for SA mode

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required | CSSF outside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gap |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 only CA | 1: if only SSB measurement configured for FR1 PCC; 2: if both SSB and CSI-RS L3 measurements configured or if only CSI-RS measurement configured for FR1 PCC | 2*Number of configured FR1 SCell(s) with both SSB and CSI-RS L3 configured +2*Number of configured FR1 SCell(s) if only CSI-RS L3 measurement configured + Number of configured FR1 SCell(s) with only SSB configured+Y | N/A | N/A | N/A | 2*Number of configured FR1 SCell(s) with both SSB and CSI-RS L3 configured +2*Number of configured FR1 SCell(s) if only CSI-RS L3 measurement configured + Number of configured FR1 SCell(s) with only SSB configured+Y |
| FR2 only intra band CA | N/A | N/A | 1: if only SSB measurement configured for FR2 PCC; 2: if both SSB and CSI-RS L3 measurements configured or if only CSI-RS measurement configured for FR2 PCC | N/A | 2*Number of configured FR2 SCell(s) with both SSB and CSI-RS L3 configured +2*Number of configured FR2 SCell(s) if only CSI-RS L3 measurement configured + Number of configured FR2 SCell(s) with only SSB configured+Y | 2*Number of configured FR2 SCell(s) with both SSB and CSI-RS L3 configured +2*Number of configured FR2 SCell(s) if only CSI-RS L3 measurement configured + Number of configured FR2 SCell(s) with only SSB configured+Y |
| FR2 only inter band CA | N/A | N/A | 1: if only SSB measurement configured for FR2 PCC; 2: if both SSB and CSI-RS L3 measurements configured or if only CSI-RS measurement configured for FR2 PCC | 2 Note 3,5：if only SSB measurement configured for the SCC; 4 Note 3,5: if both SSB and CSI-RS L3 measurements configured or if only CSI-RS measurement configured for the SCC | 2×(2*Number of configured SCell(s) with both SSB and CSI-RS L3 configured +2*Number of configured SCell(s) if only CSI-RS L3 measurement configured + Number of configured SCell(s) with only SSB configured+Y-1) | 2×(2*Number of configured SCell(s) with both SSB and CSI-RS L3 configured +2*Number of configured SCell(s) if only CSI-RS L3 measurement configured + Number of configured SCell(s) with only SSB configured+Y-1) |
| FR1 +FR2 CA (FR1 PCell) Note 1 | 1: if only SSB measurement configured for PCC; 2: if both SSB and CSI-RS L3 measurements configured or if only CSI-RS measurement configured for PCC | 2×(2*Number of configured SCell(s) with both SSB and CSI-RS L3 configured +2*Number of configured SCell(s) if only CSI-RS L3 measurement configured + Number of configured SCell(s) with only SSB configured+Y-1) | N/A | 2 Note 3：if only SSB measurement configured for the SCC; 4 Note 3: if both SSB and CSI-RS L3 measurements configured or if only CSI-RS measurement configured for the SCC | 2×(2*Number of configured SCell(s) with both SSB and CSI-RS L3 configured +2*Number of configured SCell(s) if only CSI-RS L3 measurement configured + Number of configured SCell(s) with only SSB configured+Y-1) | 2×(2*Number of configured SCell(s) with both SSB and CSI-RS L3 configured +2*Number of configured SCell(s) if only CSI-RS L3 measurement configured + Number of configured SCell(s) with only SSB configured+Y-1) |
| Note 1: Only one FR1 operating band and one FR2 operating band are included for FR1+FR2 inter-band CA. Note 2: Selection of FR2 SCC where neighbour cell measurement is required follows clause 9.2.3.2. Note 3: CSSFoutside_gap,i =1 if only one SCell with SSB only measurement is configured and no inter-frequency MO without gap; CSSFoutside_gap,i =2 if only one SCell with both SSB and CSI-RS L3 measurements is configured or if only one SCell with CSI-RS only measurement is configured and no inter-frequency MO without gap. Note 4: Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG. Note 5: Only two NR FR2 operating bands are included for FR2 inter-band CA. |  |  |  |  |  |  |

9.1.5.1.3	NR-DC mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps

For UE configured with NR-DC operation, the carrier-specific scaling factor CSSFoutside_gap,i for intra-frequency and inter-frequency SSB-based measurements and intra-frequency CSI-RS based L3 measurement performed outside measurements gaps will be as specified in Table 9.1.5.1.3-1.

Table 9.1.5.1.3-1: CSSFoutside_gap,i scaling factor for NR-DC mode

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PSCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gap |
| --- | --- | --- | --- | --- | --- |
| FR1 + FR2 NR-DC (FR1 PCell and FR2 PScell) Note 1 | 1: if only SSB measurement configured for PCC; 2: if both SSB and CSI-RS L3 measurements configured or if only CSI-RS measurement configured for PCC | 2×(2*Number of configured SCell(s) with both SSB and CSI-RS L3 configured +2*Number of configured SCell(s) if only CSI-RS L3 measurement configured + Number of configured SCell(s) with only SSB configured+Y) | 2 Note 2：if only SSB measurement configured for PSCC; 4 Note 3: if both SSB and CSI-RS L3 measurements configured or if only CSI-RS measurement configured for PSCC | 2×(2*Number of configured SCell(s) with both SSB and CSI-RS L3 configured +2*Number of configured SCell(s) if only CSI-RS L3 measurement configured + Number of configured SCell(s) with only SSB configured+Y) | 2×(2*Number of configured SCell(s) with both SSB and CSI-RS L3 configured +2*Number of configured SCell(s) if only CSI-RS L3 measurement configured + Number of configured SCell(s) with only SSB configured+Y) |
| Note 1: NR-DC in Rel-15 only includes the scenarios where all serving cells in MCG in FR1 and all serving cells in SCG in FR2. Note 2: CSSFoutside_gap,i =1 if no SCell with SSB only measurement is configured and no inter-frequency MO without gap; CSSFoutside_gap,i =2 if only one SCell with both SSB and CSI-RS L3 measurements is configured or if only one SCell with CSI-RS only measurement is configured and no inter-frequency MO without gap.. Note 3: Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG |  |  |  |  |  |

9.1.5.1.4	NE-DC mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps

For UE configured with NE-DC operation, the carrier-specific scaling factor CSSFoutside_gap,i for intra-frequency and inter-frequency SSB-based measurements and intra-frequency CSI-RS L3 measurements performed outside measurements gaps will be as specified in Table 9.1.5.1.4-1.

Table 9.1.5.1.4-1: CSSFoutside_gap,i scaling factor for NE-DC mode

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gap |
| --- | --- | --- | --- | --- | --- | --- |
| NE-DC with FR1 only CA | 1: if only SSB measurement configured for PCC; 2: if both SSB and CSI-RS L3 measurements configured or if only CSI-RS measurement configured for PCC | 2*Number of configured FR1 SCell(s) with both SSB and CSI-RS L3 configured +2*Number of configured FR1 SCell(s) if only CSI-RS L3 measurement configured + Number of configured FR1 SCell(s) with only SSB configured+Y | N/A | N/A | N/A | 2*Number of configured FR1 SCell(s) with both SSB and CSI-RS L3 configured +2*Number of configured FR1 SCell(s) if only CSI-RS L3 measurement configured + Number of configured FR1 SCell(s) with only SSB configured+Y |
| NE-DC with FR2 only intra band CA | N/A | N/A | 1: if only SSB measurement configured for PCC; 2: if both SSB and CSI-RS L3 measurements configured or if only CSI-RS measurement configured for PCC | N/A | 2*Number of configured FR2 SCell(s) with both SSB and CSI-RS L3 configured +2*Number of configured FR2 SCell(s) if only CSI-RS L3 measurement configured + Number of configured FR2 SCell(s) with only SSB configured+Y | 2*Number of configured FR2 SCell(s) with both SSB and CSI-RS L3 configured +2*Number of configured FR2 SCell(s) if only CSI-RS L3 measurement configured + Number of configured FR2 SCell(s) with only SSB configured+Y |
| NE-DC with FR2 only inter band CA | N/A | N/A | 1: if only SSB measurement configured for PCC; 2: if both SSB and CSI-RS L3 measurements configured or if only CSI-RS measurement configured for PCC | 2 Note 3,5：if only SSB measurement configured for the SCC; 4 Note 3,5: if both SSB and CSI-RS L3 measurements configured or if only CSI-RS measurement configured for the SCC | 2×(2*Number of configured SCell(s) with both SSB and CSI-RS L3 configured +2*Number of configured SCell(s) if only CSI-RS L3 measurement configured + Number of configured SCell(s) with only SSB configured+Y-1) | 2×(2*Number of configured SCell(s) with both SSB and CSI-RS L3 configured +2*Number of configured SCell(s) if only CSI-RS L3 measurement configured + Number of configured SCell(s) with only SSB configured+Y-1) |
| NE-DC with FR1 +FR2 CA (FR1 PCell) Note 1 | 1: if only SSB measurement configured for PCC; 2: if both SSB and CSI-RS L3 measurements configured or if only CSI-RS measurement configured for PCC | 2×(2*Number of configured SCell(s) with both SSB and CSI-RS L3 configured +2*Number of configured SCell(s) if only CSI-RS L3 measurement configured + Number of configured SCell(s) with only SSB configured+Y-1) | N/A | 2 Note 3：if only SSB measurement configured for the SCC; 4 Note 3: if both SSB and CSI-RS L3 measurements configured or if only CSI-RS measurement configured for the SCC | 2×(2*Number of configured SCell(s) with both SSB and CSI-RS L3 configured +2*Number of configured SCell(s) if only CSI-RS L3 measurement configured + Number of configured SCell(s) with only SSB configured+Y-1) | 2×(2*Number of configured SCell(s) with both SSB and CSI-RS L3 configured +2*Number of configured SCell(s) if only CSI-RS L3 measurement configured + Number of configured SCell(s) with only SSB configured+Y-1) |
| Note 1: Only one FR1 operating band and one FR2 operating band are included for FR1+FR2 inter-band CA. Note 2: Selection of FR2 SCC where neighbour cell measurement is required follows clause 9.2.3.2. Note 3: CSSFoutside_gap,i =1 if only one SCell with SSB only measurement is configured and no inter-frequency MO without gap; CSSFoutside_gap,i =2 if only one SCell with both SSB and CSI-RS L3 measurements is configured or if only one SCell with CSI-RS only measurement is configured and no inter-frequency MO without gap.. Note 4: Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG. Note 5: Only two NR FR2 operating band are included for NE-DC with FR2 only inter-band CA. |  |  |  |  |  |  |

9.1.5.2	Monitoring of multiple layers within gaps

The carrier-specific scaling factor CSSFwithin_gap,i for a measurement object i derived in this chapter is applied to following measurement types:

-	SSB-based intra-frequency measurement object with no measurement gap in clause 9.2.5, when all of the SMTC occasions of this intra-frequency measurement object are overlapped by the measurement gap.

-	SSB-based intra-frequency measurement object with measurement gap in clause 9.2.6.

-	SSB-based inter-frequency measurement with no measurement gap in clause 9.3.9, when all of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap, if UE supports interFrequencyMeas-NoGap-r16.

-	CSI-RS based inter-frequency measurement in clause 9.10.3, when all CSI-RS resources for L3 measurement of this inter-frequency measurement object are overlapped by the measurement gap.

-	CSI-RS based inter-frequency measurement in clause 9.10.3, when all of CSI-RS resources for L3 measurement of this inter-frequency measurement object are partially overlapped by the measurement gap.

-	SSB-based inter-frequency measurement object with measurement gap in clause 9.3.4.

-	E-UTRA Inter-RAT measurement object in clauses 9.4.2 and 9.4.3.

-	NR PRS-based measurements for positioning in clause 9.9.

-	E-UTRA Inter-RAT RSTD and E-CID measurements in clauses 9.4.4 and 9.4.5.

-	NR Inter-RAT measurement object configured by the E-UTRAN PCell (TS 36.133 [15] clause 8.17.4).

-	E-UTRAN Inter-frequency measurement object configured by the E-UTRAN PCell (TS 36.133 [15] clause 8.17.3) and by the E-UTRAN PSCell (TS 36.133 [15] clause 8.19.3).

-	E-UTRAN Inter-frequency RSTD measurement configured by the E-UTRAN PCell (TS 36.133 [15] clause 8.17.15).

-	UTRA Inter-RAT measurement object configured by the E-UTRAN PCell (TS 36.133 [15] clauses 8.17.5 to 8.17.12).

-	GSM Inter-RAT measurements configured by the E-UTRAN PCell (TS 36.133 [15] clauses 8.17.13 and 8.17.14).

UE is expected to conduct the measurement of this measurement object i only within the measurement gaps.

If the higher layer signaling in TS 38.331 [2] of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, CSSFwithin_gap,i and requirements derived from CSSFoutside_gap,i are not specified.

SSB and CSI-RS for mobility configured in the same MO should be considered as 2 measurement objects.

Number of SSB measurement object should include SSB for mobility and that as associatedSSB for CSI-RS mobility.

FFS

9.1.5.2.1	EN-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based measurements performed within gaps

The scaling value CSSFwithin_gap,i below has been derived without considering GSM inter-RAT carriers.

When one or more measurement objects are monitored within measurement gaps, the carrier specific scaling factor for a target measurement object with index i is designated as CSSFwithin_gap,i and is derived as described in this clause.

If measurement object i refers to an RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured, CSSFwithin_gap,i=1. Otherwise, the CSSFwithin_gap,i for other measurement objects (including RSTD measurement with periodicity Tprs=160ms) participate in the gap competition are derived as below.

For each measurement gap j not used for an RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured within an arbitrary 160ms period, count the total number of intra-frequency measurement objects and inter-frequency/inter-RAT measurement objects which are candidates to be measured within the gap j.

-	An NR SSB measurement object is a candidate to be measured in a gap if its SMTC duration is fully covered by the MGL excluding RF switching time. For intra-frequency NR carriers, if the higher layer in TS 38.331 [2] signaling of smtc2 is configured, the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc2; otherwise the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc1.

Note: The number of SSB measurement object shall include the total number of MOs with

-	ssb-ConfigMobility configured, or

-	ssb-ConfigMobility not configured but csi-rs-ResourceConfigMobility configured with associatedSSB.

If ssbfrequency, smtc1, smtc2 and ssbSubcarrierSpacing are same in multiple MOs, the multiple MOs are counted as one candidate for SSB measurement.

-   An NR CSI-RS measurement object is a candidate to be measured in a gap if the window confining all CSI-RS resources are fully covered by the MGL excluding RF switching time.

-	An inter-RAT UTRA measurement object configured by E-UTRA PCell [15] is a candidate to be measured in all measurement gaps.

-	An inter-frequency E-UTRA measurement object configured by E-UTRA PCell [15] is a candidate to be measured in all measurement gaps.

-	For UEs which support and are configured with per FR gaps, the counting is done on a per FR basis, and for UEs which are configured with per UE gaps the counting is done on a per UE basis.

-	Mintra,i,j: Number of intra-frequency measurement objects, including SSB based and/or CSI-RS based, which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Mintra,i,j  equals 0.

-	Minter,i,j : Number of NR inter-frequency measurement objects or NR inter-RAT measurement objects, including SSB based and/or CSI-RS based, configured by E-UTRA PCell, EUTRA inter-frequency measurement objects configured by E-UTRA PCell, UTRA inter-RAT measurement objects configured by E-UTRA PCell which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Minter,i,j  equals 0.

-	Mtot,i,j = Mintra,i,j + Minter,i,j : Total number of intra-frequency, inter-frequency and inter-RAT measurement objects which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Mtot,i,j equals 0.

For each measurement gap j used for an RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured within an arbitrary 160ms period, Mintra,i,j = Minter,i,j = Mtot,i,j =0.

The carrier specific scaling factor CSSFwithin_gap,i is given by:

If measGapSharingScheme is equal sharing, CSSFwithin_gap,i= max(ceil(Ri×Mtot,i,j)), where j=0…(160/MGRP)-1

If measGapSharingScheme is not equal sharing and

-	measurement object i is an intra-frequency measurement object, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kintra×Mintra,i,j) in gaps where Minter,i,j≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×Mintra,i,j) in gaps where Minter,i,j=0, where j=0…(160/MGRP)-1

-	measurement object i is an inter-frequency or inter-RAT measurement object, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kinter×Minter,i,j) in gaps where Mintra,i,j ≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×Minter,i,j) in gaps where Mintra,i,j=0, where j=0…(160/MGRP)-1

Where Ri is the maximal ratio of the number of measurement gap where measurement object i is a candidate to be measured over the number of measurement gap where measurement object i is a candidate and not used for RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured within an arbitrary 1280ms period.

9.1.5.2.2	SA mode: carrier-specific scaling factor for SSB-based and CSI-RS based measurements performed within gaps

When one or more measurement objects are monitored within measurement gaps, the carrier specific scaling factor for a target measurement object with index i is designated as CSSFwithin_gap,i and is derived as described in this clause.

If measurement object i refers to a long-periodicity measurement which is any of:

-	an E-UTRA RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured, or

-	an NR measurement for positioning based on PRS configurations in Table 9.1.5.2.2-1

then CSSFwithin_gap,i=1. Otherwise, the CSSFwithin_gap,i for other measurement objects (including E-UTRA RSTD measurement with periodicity Tprs=160ms) participate in the gap competition and the CSSFwithin_gap,i are derived as below.

Table 9.1.5.2.2-1: PRS configurations for long-periodicity NR measurements for positioning

| [PRS periodicity] (ms) | DL-PRS-MutingPattern configuration |
| --- | --- |
| 320, 640, … ,10240 | [With or without muting] |
| Other values (≤ 160) | FFS |

For each measurement gap j not used for a long-periodicity measurement defined above, count the total number of intra-frequency measurement objects and inter-frequency/inter-RAT measurement objects and [TBD for NR positioning measurements] which are candidates to be measured within the gap j.

-	An NR SSB measurement object is a candidate to be measured in a gap if its SMTC duration is fully covered by the MGL excluding RF switching time. For intra-frequency NR measurement objects, if the higher layer in TS 38.331 [2] signaling of smtc2 is configured, the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc2; otherwise the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc1.

Note: The number of SSB measurement object shall include the total number of MOs with

-	ssb-ConfigMobility configured, or

-	ssb-ConfigMobility not configured but csi-rs-ResourceConfigMobility configured with associatedSSB.

If ssbfrequency, smtc1, smtc2 and ssbSubcarrierSpacing are same in multiple MOs, the multiple MOs are counted as one candidate for SSB measurement.

-   An NR CSI-RS measurement object is a candidate to be measured in a gap if the window confining all CSI-RS resources are fully covered by the MGL excluding RF switching time.

-	An inter-RAT measurement object is a candidate to be measured in all meausrement gaps.

-	An inter-frequency SFTD measurement object, if to be measured with measurement gaps, is a candidate to be measured in all measurement gaps.

-	For UEs which support and are configured with per FR gaps, the counting is done on a per FR basis, and for UEs which are configured with per UE gaps the counting is done on a per UE basis. For UEs which support and are configured with per FR gaps, the CSSF requirements do not apply when NR PRS measurement in one FR gap collides with SSB/CSI-RS/PRS measurements in the other FR gap in time domain.

-	Mintra,i,j: Number of intra-frequency measurement objects, including SSB based and/or CSI-RS based, which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Mintra,i,j  equals 0.

-	Minter,i,j : Number of NR inter-frequency, EUTRA inter-RAT and UTRA inter-RAT objects, including SSB based and/or CSI-RS based, and [TBD for NR positioning measurements] which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Minter,i,j  equals 0.

-	Mtot,i,j = Mintra,i,j + Minter,i,j : Total number of intra-frequency, inter-frequency and inter-RAT measurement objects and [TBD for NR positioning measurements] which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Mtot,i,j equals 0.

For each measurement gap j used for a long-periodicity measurement defined above, Mintra,i,j = Minter,i,j = Mtot,i,j =0.

The carrier specific scaling factor CSSFwithin_gap,i is given by:

If measGapSharingScheme is equal sharing, CSSFwithin_gap,i= max(ceil(Ri×Mtot,i,j)), where j=0…(160/MGRP)-1

If measGapSharingScheme is not equal sharing and

-	measurement object i is an intra-frequency measurement object, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kintra×Mintra,i,j) in gaps where Minter,i,j≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×Mintra,i,j) in gaps where Minter,i,j=0, where j=0…(160/MGRP)-1

-	measurement object i is an inter-frequency or inter-RAT measurement object or [TBD for NR positioning measurements], CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kinter×Minter,i,j) in gaps where Mintra,i,j ≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×Minter,i,j) in gaps where Mintra,i,j=0, where j=0…(160/MGRP)-1

Where Ri is the maximal ratio of the number of measurement gap where measurement object i is a candidate to be measured over the number of measurement gap where measurement object i is a candidate and not used for a long-periodicity measurement defined above.

CSSFwithin_gap,k=1 during TDetect, E-UTRAN FDD specified in clause 9.4.4.1.2.2 and TDetect, E-UTRAN TDD specified in clause 9.4.4.2.2.2, where k is the carrier frequency where the UE is performing cell detection of the inter-RAT E-UTRA OTDOA assistance data reference cell when acquiring the subframe and slot timing of the cell according to clause 9.4.4. In this case, the UE cell identification and measurement periods derived based on CSSFwithin_gap,i in clauses 9.2.5.1, 9.2.5.2, 9.2.6.2, 9.2.6.3, 9.3.4, 9.3.5, 9.4.2.2, and 9.4.2.3 may be extended for measurement objects of which the cell identification and measurement periods are overlapped with TDetect, E-UTRAN FDD and TDetect, E-UTRAN TDD.

9.1.5.2.3	NE-DC: carrier-specific scaling factor for SSB-based and CSI-RS based measurements performed within gaps

When one or more measurement objects are monitored within measurement gaps, the carrier specific scaling factor for a target measurement object with index i is designated as CSSFwithin_gap,i and is derived as described in this clause.

If measurement object i refers to a long-periodicty measurement which is any of:

-	an E-UTRA RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured, or

-	an NR measurement for positioning based on PRS configurations in Table 9.1.5.2.2-1

then CSSFwithin_gap,i=1. Otherwise, the CSSFwithin_gap,i for other measurement objects (including E-UTRA RSTD measurement with periodicity Tprs=160ms) participate in the gap competition are derived as below.

For each measurement gap j not used for a long-periodicity measurement defined above, count the total number of intra-frequency measurement objects and inter-frequency/inter-RAT measurement objects and [TBD for NR positioning measurements] which are candidates to be measured within the gap j.

-	An NR SSB measurement object is a candidate to be measured in a gap if its SMTC duration is fully covered by the MGL excluding RF switching time. For intra-frequency NR measurement objects, if the higher layer in TS 38.331 [2] signaling of smtc2 is configured, the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc2; otherwise the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc1.

Note: The number of SSB measurement object shall include the total number of MOs with

-	ssb-ConfigMobility configured, or

-	ssb-ConfigMobility not configured but csi-rs-ResourceConfigMobility configured with associatedSSB.

If ssbfrequency, smtc1, smtc2 and ssbSubcarrierSpacing are same in multiple MOs, the multiple MOs are counted as one candidate for SSB measurement.

-   An NR CSI-RS measurement object is a candidate to be measured in a gap if the window confining all CSI-RS resources are fully covered by the MGL excluding RF switching time.

-	An inter-RAT measurement object is a candidate to be measured in all measurement gaps.

-	An inter-frequency E-UTRA measurement object is a candidate to be measured in all measurement gaps.

For UEs which support and are configured with per FR gaps, the counting is done on a per FR basis, and for UEs which are configured with per UE gaps the counting is done on a per UE basis. For UEs which support and are configured with per FR gaps, the CSSF requirements do not apply when NR PRS measurement in one FR gap collides with SSB/CSI-RS/PRS measurements in the other FR gap in time domain.

If the number of configured interfrequency and interRAT measuerement objects and [TBD for NR positioning measurements] is non-zero and the UE is configured with per UE gaps, or if the UE is configured with per FR gaps:

FR1 and FR2 intrafrequency measurement objects belong to group A

Interfrequency and interRAT measurement objects belong to group B

MgroupA,i,j: Sum of the number of FR1 intra-frequency measurement objects Mintra-FR1,i,j and the number of FR2 intra-frequency measurement objects Mintra-FR2,i,j, including SSB based and/or CSI-RS based, which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupA,i,j  equals 0.

MgroupBi,j: Number of NR inter-frequency, EUTRA inter-RAT and UTRA inter-RAT measurement objects, including SSB based and/or CSI-RS based, and [TBD for NR positioning measurements]which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupB,i,j  equals 0.

If the number of configured inter-frequency and inter-RAT measuerement objects and [TBD for NR positioning measurements] is zero and the UE is configured with per UE gaps:

FR1 intrafrequency measurement objects belong to group A

FR2 intrafrequency measurement objects belong to group B

MgroupA,i,j: The number of FR1 intrafrequency measurement objects Mintra-FR1,i,j which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupA,i,j  equals 0.

MgroupBi,j : The number of FR2 intrafrequency measurement objects Mintra-FR2,i,j which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupB,i,j  equals 0.

Mtot,i,j = MgroupA,i,j + MgroupB,i,j : Total number of group A and group B measurement objects which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Mtot,i,j equals 0.

For each measurement gap j used for a long-periodicity measurement defined above, Mintra,i,j = Minter,i,j = Mtot,i,j =0.

The carrier specific scaling factor CSSFwithin_gap,i is given by:

If measGapSharingScheme is equal sharing, CSSFwithin_gap,i= max(ceil(Ri×Mtot,i,j)), where j=0…(160/MGRP)-1

If measGapSharingScheme is not equal sharing and

-	measurement object i is a group A measurement object, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kintra×MgroupA,i,j) in gaps where MgroupB,i,j≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×MgroupA,i,j) in gaps where MgroupB,i,j=0, where j=0…(160/MGRP)-1

-	measurement object i is an group B measurement object, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kinter×MgroupBi,j) in gaps where MgroupA,i,j ≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×MgroupB,i,j) in gaps where MgroupA,i,j=0, where j=0…(160/MGRP)-1

Where Ri is the maximal ratio of the number of measurement gap where measurement object i is a candidate to be measured over the number of measurement gap where measurement object i is a candidate and not used for a long-periodicity measurement defined above.

9.1.5.2.4	NR-DC: carrier-specific scaling factor for SSB-based and CSI-RS based measurements performed within gaps

When one or more measurement objects are monitored within measurement gaps, the carrier specific scaling factor for a target measurement object with index i is designated as CSSFwithin_gap,i and is derived as described in this clause.

If measurement object i refers to a long-periodicity measurement which is any of:

-	an E-UTRA RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured, or

-	an NR positioning measurement corresponding to a positioning frequency layer with configurations of maximum DL-PRS-Periodicity and maximum bitmap size of DL-PRS-MutingPattern among its PRS resource sets as in Table 9.1.5.2.2-1

then CSSFwithin_gap,i=1. Otherwise, the CSSFwithin_gap,i for other measurement objects (including E-UTRA RSTD measurement with periodicity Tprs=160ms) participate in the gap competition and the CSSFwithin_gap,i are derived as below.

For each measurement gap j not used for an RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured within an arbitrary 160ms period, count the total number of intra-frequency measurement objects and inter-frequency/inter-RAT measurement objects and [TBD for NR positioning measurements] which are candidates to be measured within the gap j.

-	An NR measurement object is a candidate to be measured in a gap if its SMTC duration is fully covered by the MGL excluding RF switching time. For intra-frequency NR measurement objects, if the higher layer in TS 38.331 [2] signaling of smtc2 is configured, the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc2; otherwise the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc1.

Note: The number of SSB measurement object shall include the total number of MOs with

-	ssb-ConfigMobility configured, or

-	ssb-ConfigMobility not configured but csi-rs-ResourceConfigMobility configured with associatedSSB.

If ssbfrequency, smtc1, smtc2 and ssbSubcarrierSpacing are same in multiple MOs, the multiple MOs are counted as one candidate for SSB measurement.

-   An NR CSI-RS measurement object is a candidate to be measured in a gap if the window confining all CSI-RS resources are fully covered by the MGL excluding RF switching time.

-	An inter-RAT measurement object is a candidate to be measured in all measurement gaps.

For UEs which support and are configured with per FR gaps, the counting is done on a per FR basis, and for UEs which are configured with per UE gaps the counting is done on a per UE basis. For UEs which support and are configured with per FR gaps, the CSSF requirements do not apply when NR PRS measurement in one FR gap collides with SSB/CSI-RS/PRS measurements in the other FR gap in time domain.

If the number of configured interfrequency and interRAT measuerement objects and [TBD for NR positioning measurements] is non-zero and the UE is configured with per UE gaps, or if the UE is configured with per FR gaps:

FR1 and FR2 intrafrequency measurement objects belong to group A

Interfrequency and interRAT measurement objects and [TBD for NR positioning measurements] belong to group B

MgroupA,i,j: Sum of the number of FR1 intra-frequency measurement objects Mintra-FR1,i,j and the number of FR2 intra-frequency measurement objects Mintra-FR2,i,j, including SSB based and/or CSI-RS based, which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupA,i,j  equals 0.

MgroupBi,j : Number of NR inter-frequency, EUTRA inter-RAT and UTRA inter-RAT measurement objects, including SSB based and/or CSI-RS based, and [TBD for NR positioning measurements] which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupB,i,j  equals 0.

If the number of configured interfrequency and interRAT measuerement objects and [TBD for NR positioning measurements] is zero and the UE is configured with per UE gaps:

FR1 intrafrequency measurement objects belong to group A

FR2 intrafrequency measurement objects belong to group B

MgroupA,i,j: The number of FR1 intrafrequency measurement objects Mintra-FR1,i,j which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupA,i,j  equals 0.

MgroupBi,j : The number of FR2 intrafrequency measurement objects Mintra-FR2,i,j which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupB,i,j  equals 0.

Mtot,i,j = MgroupA,i,j + MgroupB,i,j : Total number of group A and group B measurement objects which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Mtot,i,j equals 0.

For each measurement gap j used for a long-periodicity measurement defined above, Mintra,i,j = Minter,i,j = Mtot,i,j =0.

The carrier specific scaling factor CSSFwithin_gap,i is given by:

If measGapSharingScheme is equal sharing, CSSFwithin_gap,i= max(ceil(Ri×Mtot,i,j)), where j=0…(160/MGRP)-1

If measGapSharingScheme is not equal sharing and

-	measurement object i is a group A measurement object, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kintra×MgroupA,i,j) in gaps where MgroupB,i,j≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×MgroupA,i,j) in gaps where MgroupB,i,j=0, where j=0…(160/MGRP)-1

-	measurement object i is an group B measurement object, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kinter×MgroupBi,j) in gaps where MgroupA,i,j ≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×MgroupB,i,j) in gaps where MgroupA,i,j=0, where j=0…(160/MGRP)-1

Ri is the maximal ratio of the number of measurement gap where measurement object i is a candidate to be measured over the number of measurement gap where measurement object i is a candidate and not used for a long-periodicity measurement defined above.

9.1.5.2.5	SA mode: carrier-specific scaling factor for PRS-based measurements performed within gaps

The requirements in this clause apply for NR PRS-based measurements for positioning in clause 9.9.

Editor’s note: FFS whether/how the contents of 9.1.5.2.2 can be reused here.

9.1.5.2.6	NE-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps

The requirements in this clause apply for NR PRS-based measurements for positioning in clause 9.9.

Editor’s note: FFS whether/how the contents of 9.1.5.2.3 can be reused here.

9.1.5.2.7	NR-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps

The requirements in this clause apply for NR PRS-based measurements for positioning in clause 9.9.

Editor’s note: FFS whether/how the contents of 9.1.5.2.4 can be reused here.

<End of Change 1>

<Start of Change 2>