3GPP TSG-RAN WG4 Meeting # 97-e 	                              R4-2015510

Electronic Meeting, 2-13 Nov., 2020

Agenda item:	7.4.5.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM performance requirements for IAB

Document for:	Discussion

- Introduction

The core requirements for IAB were completed in the RAN4#96e meeting [1], and the performance works are scheduled for the coming RAN4#97e meeting. Considering the deployment scenarios and the characteristics of IAB, the contribution provides views on the scope and general principles at the beginning of the performance stage to have a clear picture of the performance work.

- Discussion

IAB is designed to enable flexible and very dense deployment of NR cells with access links and backhaul links. For each IAB node, it contains IAB-DU part and IAB-MT part, and different types of IAB-DU and IAB-MT are defined in RF session considering the deployment scenarios and the characteristic of IAB.

The performance testing work in RAN4 is proceeding for both IAB-DU and IAB-MT for all requirements including RF, RRM, demodulation, etc. It is technically reasonable to create and maintain the complete performance test work of IAB within RAN4, as there is no IAB related discussions and pre-work in RAN5. It was also the common understanding and was agreed in the demod session of IAB which was triggered in the last RAN4#96-e meeting that all the conformance testing work shall be down within RAN4 [2].

We think for the RRM requirements testing, it is reasonable to defined and maintained within RAN4 only and there is no need to involve RAN5’s work, otherwise it does not make much sense that part of the IAB requirements are defined in RAN4 while others may be discussed and maintained in RAN5, which will greatly prolong and complex the specification work and increase the cost of commercial deployment, let alone we have types of requirements (RF, RRM and demodulations) for both IAB-DU and IAB-MT for a single IAB-node. As the IAB is functionally a network node, the conformance testing defined in RAN5 for original UE is not needed for IAB and RAN5 is less prepared for such testing work.

Proposal 1: The RRM performance testing requirements shall be defined and maintained in RAN4.

Base on the discussion of the core requirements defined for IAB in the previous meetings, limited RRM requirements are defined with relaxations considering the immobility of the IAB-node and some unique characteristics of IAB. From our understanding, no performance requirements work is needed for IAB-DU and we should focus on defining the performance requirements for IAB-MT only, and the IAB-MT shall be tested with the DU part disabled.

Proposal 2: Only RRM performance requirements for IAB-MT are needed and the IAB-MT shall be tested with DU part disabled.

Also different from original UE, different types of IAB are defined (type 1-H, type 1-O and type 2-O) following the BS manner. These should also be taken into account for the performance requirements about the test methodology (conducted/OTA). For the performance testing for the original UE, the conducted manner is adopted for FR1 and OTA is adopted for FR2. Considering the IAB types, it is suggested to use conducted testing for IAB type 1-H and OTA testing for IAB type 1-O and 2-O.

Proposal 3: Different types of IAB (type 1-H, type 1-O and type 2-O) shall be considered to decide the test methodology (conducted/OTA). Use conducted testing for IAB type 1-H and OTA testing for IAB type 1-O and 2-O.

Another issue is related to CA and EN-DC operation. It was agreed in the last RAN4 meeting that the CA is not considered for transmit timing requirements. Also there has been no technical discussion and clear framework for EN-DC, thus it is suggested to define test cases for RRM requirements under NR SA.

Proposal 4: Define test cases for RRM requirements under NR SA.

We also notice that there are parallel discussions in RF and demodulation sessions about the performance requirements of IAB. From our understanding, it is technically reasonable to include all the performance requirements in a single dedicated spec for IAB, as what we have in TS 38.174 for the core requirements. As mentioned above, the IAB-MT and IAB-DU are essentially serving as a single node in the network, thus it is preferred to have unified test methodology and setup for the IAB node. Otherwise, it will bring tremendous standardization efforts and commercial cost for IAB testing if some requirements are tested following a certain test procedure and setup while some others will be tested under another procedure and setup defined in another test spec considering that we have RF RRM and Demod requirements for IAB-MT and IAB-DU for a single IAB node.

Proposal 5: Align with the conclusion from RF and Demod sessions and include all performance requirements in a single dedicated spec for IAB.

For the RRM requirements for IAB-MT, only limited requirements are defined with necessary relaxation compared to the requirements for UE. Though IAB-MT adopts some functions of UE, the IAB-MT and IAB-DU are essentially the same device and it will serve as a single node in the network without mobility. The primary goal of IAB is to seek for high throughput without much concerns about the power consumptions, which is quite different from an ordinary UE. All these unique features were taken into account when defining the requirements for IAB. In the practical scenarios, the IAB nodes will be deployed by operators in a pre-planned manner, which means the channel condition is much more stable and predictable, and the prior information could be utilized for organizing the IAB nodes. In other words, it is less likely for an IAB node to experience the link broken or searching for other nodes for access after the IAB node is deployed.

Observation1: The IAB node will be deployed in a pre-planned manner with stable channel conditions, which means it is less likely for an IAB node to experience the link broken or re-establishment after the IAB node is deployed compared with original UE.

Also following the RF assumptions, the IAB-MT is quite different from the original UE with the declared operating bandwidth and maximum output power, especially for the wide-area IAB-MT. And the channel conditions are considered to be more stable when defining the dynamic range for IAB-MT. It could also be observed that the certification process for an IAB-MT is different from UE. Thus the test procedures and set up for original UE may not be applicable to IAB-MT.

Observation 2: The test procedures and setup for original UE defined in RAN5 may not be applicable to IAB-MT.

Based on the analysis above, considering that the IAB-MT will serve as an network node in stable and good channel conditions in practical scenarios and with limited RRM requirements defined, it is reasonable to consider a simplification of the RRM requirements testing compared with UE. For the original UE test cases, we have multiple test cases for each RRM requirements for different combinations: with/without DRX, with/without gap, known/unknown target Cell, etc. But for IAB-MT, as mentioned above, the channel conditions are considered to be stable, thus it is reasonable to only consider the typical scenarios of IAB deployment of the requirements for testing and define the major procedures and test configurations as are defined for UE in TS 36.133/38.133; otherwise, there will be tons of details of procedures and configurations to be considered.

Proposal 6: RAN4 to consider simplifying the RRM requirements testing for IAB-MT to define typical scenarios and key configurations for RRM requirements and taking TS 38.133 as the baseline.

As for the details of related test cases list for the corresponding requirements, we have a separate paper [3] to analyze the scenarios and factors in details case by case for each RRM requirements.

Another issues is related to the type of IAB-MT. Two different types of IAB-MT are defined in RF session: Wide Area IAB-MT and Local Area IAB-MT. These two types of IAB-MT target different scenarios and have different structures in RF session. For wide area IAB-MT, it is characterized by Macro Cell and/or Micro Cell scenarios while for the local area IAB-MT, it is characterized by Pico Cell and/or Micro Cell scenarios. The wide area IAB is considered as the BS-like type of network nodes with well-planed deployment. And for local area IAB-MT, it is more likely to experience the changing of the channel as the planning maybe not needed when deploying the local area IAB-MT. From RRM’s perspective, some requirements are only defined for local area IAB-MT, i.e. RLM and link recovery. Thus, the performance requirements shall be differentiated between wide area IAB-MT and local area IAB-MT if needed.

Proposal 7: The performance requirements shall be differentiated between wide area IAB-MT and local area IAB-MT if needed.

- Conclusions

Proposal 1: The RRM performance testing requirements shall be defined and maintained in RAN4.

Proposal 2: Only RRM performance requirements for IAB-MT are needed and the IAB-MT shall be tested with DU part disabled.

Proposal 3: Different types of IAB (type 1-H, type 1-O and type 2-O) shall be considered to decide the test methodology (conducted/OTA). Use conducted testing for IAB type 1-H and OTA testing for IAB type 1-O and 2-O.

Proposal 4: Define test cases for RRM requirements under NR SA.

Proposal 5: Align with the conclusion from RF and Demod sessions and include all performance requirements in a single dedicated spec for IAB.

Observation1: The IAB node will be deployed in a pre-planned manner with stable channel conditions, which means it is less likely for an IAB node to experience the link broken or re-establishment after the IAB node is deployed compared with original UE.

Observation 2: The test procedures and setup for original UE defined in RAN5 may not be applicable to IAB-MT.

Proposal 6: RAN4 to consider simplifying the RRM requirements testing for IAB-MT to define typical scenarios and key configurations for RRM requirements and taking TS 38.133 as the baseline.

Proposal 7: The performance requirements shall be differentiated between wide area IAB-MT and local area IAB-MT if needed.

References

[1] R4-2012234 on RLM requirements for IAB-MT, ZTE Corporation

[2] R4-2012644 WF on Rel-16 NR IAB demodulation requirements, Nokia, Nokia Shanghai Bell

[3] R4-2015511