3GPP TSG-RAN WG4 Meeting #97-e	R4-2015612

# Electronic Meeting, 2nd - 13th Nov, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.104 | CR | - | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on BS demodulation requirements for 2-step RACH for FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_2step_RACH-Perf |  |  |  |  |  | Date: |  |  | 2020-10-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduce BS demodulation requirements for 2-step RACH for FR2 as per RAN4 agreements |  |  |  |  |  |  |  |  |
| Summary of change: |  | For requirements for BS type 1-O, add section 11.2.1.6. For requirements for BS type 2-O, add section 11.2.2.4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be inconsistence between the specification 38.104 and RAN 4 agreements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.2.1.6, 11.2.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.141-1, TS 38.141-2 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of the change 1>

11.2.1.6	Requirements for PUSCH for 2-step RACH

Apply the requirements defined in clause 8.2.6 for 2Rx.

<End of the change 1>

<Start of the change 2>

11.2.2.4	Requirements for PUSCH for 2-step RACH

11.2.2.4.1	General

The performance requirement of PUSCH for 2-step RACH is determined by a minimum required block error rate of MsgA for a given SNR for the FRCs listed in annex A. The performance requirements assume that the precedent preamble of MsgA is correctly detected in a 2-step RACH procedure and no HARQ retransmissions.

Table 11.2.2.4.1-1: Test parameters for testing PUSCH for 2-step RACH

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Channel bandwidth |  | 60 kHz SCS: 50 MHz 120 kHz SCS: 100 MHz |
| MCS |  | 2 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain resource | PUSCH mapping type | B |
|  | Start symbol | 0 |
|  | Allocation length | 10 |
| Frequency domain resource | RB assignment | 2 PRB |
|  | Starting PRB index | 0 |
|  | Frequency hopping | Disabled |

Table 11.2.2.4.1-2: Time Offset (TO) cycling level

| SCS[kHz] | Time Offset[µs] |  |
| --- | --- | --- |
|  | [Medium] | [High] |
| 60 | from 0 to 0.5, step 0.1 | from 0 to 0.6, step 0.1 |
| 120 | from 0 to 0.25, step 0.05 | from 0 to 0.6, step 0.1 |

11.2.2.4.2	Minimum requirements

The block error rate of MsgA shall be equal to or smaller than [1%] at the given SNR.

Table 11.2.2.4.2-1 Minimum requirements of PUSCH for 2-step RACH with mappying type B

| Number of TX antennas | Number of RX antennas | Cyclic prefix | TO cycling Level | Channel Bandwidth [MHz] | SCS [kHz] | Propagation conditions and correlation matrix (Annex G) | FRC (Annex A) | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | [Medium] | 50 | 60 | TDLA30-300 low | G-FR2-A9-1 | [TBD] |
|  |  |  |  | 100 | 120 | TDLA30-300 low | G-FR2-A9-1 | [TBD] |
|  |  |  | [High] | 50 | 60 | TDLA30-300 low | G-FR2-A9-1 | [TBD] |
|  |  |  |  | 100 | 120 | TDLA30-300 low | G-FR2-A9-1 | [TBD] |

<End of the change 2>