3GPP TSG-RAN WG4 Meeting #97-e	R4-2015638

# Electronic Meeting, 2-13 Nov., 2020

Title: 	Discussion on NR-U PUCCH performance requirements

Source: 	Huawei, HiSilicon

Agenda item:	7.1.8.4.2

Document for:	Discussion

Background

At the last meeting, RAN 4 agreed to introduce requirements for PRB-Interlaced PUCCH resource allocation [1] and options are listed as follows:

- PUCCH enhance format - Option 1: Format 0/1/2/3 - Option 2: Format 0/2 - FFS. - Number of interlaces for PRB-interlaced PUCCH Resource Allocation - Option 1: Single Interlace - Option 2: 2 interlaces. - Not applicable to all PUCCH formats; - FFS

In this paper, we provide our discussions on simulation assumptions for PUCCH performance requirements.

Discussion

Simulation setup

The features of NR-U PUCCH has been shown in Table 1:

Table 1: PUCCH features for NR-U

| features | Descriptions |
| --- | --- |
| PUCCH | Only support 20MHz - 15 kHz SCS: M = 10 interlaces with N = 10 or 11 PRBs / interlace - 30 kHz SCS: M = 5 interlaces with N = 10 or 11 PRBs / interlace - Frequence hopping is not used for interlaced transmission. |
| PF0 and PF1 | Enhanced PUCCH format 0 and PUCCH format 1 - One interlace can be configured, - The Rel-15 PUCCH sequence is repeated in each PRB of the interlace - The number of PRBs is 10 or 11 - A PRB-specific cyclic shift term is added in order to reduce the Cubic Metric of the repetitive sequence mapping |
| PF2 | Enhanced PUCCH format 2 - One or two interlaces can be configured, - UE multiplexing by means of OCCs in freqeuncy domain with spreading factors 2 and 4. UE multiplexing is enabled only when one interlaced is configured. - PRB-specific OCCs are used to reduce the Cubic Metric of the repeated modulation symbols from the OCC spreading |
| PF3 | Enhanced PUCCH format 3 - One or two interlaces can be configured, - The number of PRBs is 10 or 20 - Pre-DFT precoder OCCs with spreading factor 2 and 4 are used for UE multiplexing with one interlace - The OCCs are DFT sequences and result in FDM of UEs (e.g., comb-2 or comb-4 within the interlace) |

PUCCH format

From Table 1 we can see that PRB-interlaced design is allocated for PF0/1/2/3. In order to test BS for this feature supporting   different PUCCH formats, we prefer to define performance requirements for all formats of PUCCH supporting PRB-interlaced structure specified by RAN 1.i.e. PF0/1/2/3.

SCS

In order to test BS for this feature supporting different SCS, we propose to define the requirements for both 15 kHz and 30 kHz.

Test applicability rules

BS needn’t be tested for all the cases defined in specification. Similar to NR PUCCH, we propose to use following applicability rules:

Unless otherwise stated, PUCCH requirement tests shall apply only for each PUCCH format declared to be supported

Unless otherwise stated, PUCCH requirement tests shall apply only for each subcarrier spacing declared to be supported

Proposal 1: Define the requirements for PRB-interlaced PUCCH resource allocation with following simulation setups:

- PF0/1/2/3

- Both 15 kHz and 30 kHz

- Test applicability rules:

- Unless otherwise stated, PUCCH requirement tests shall apply only for each PUCCH format declared to be supported

- Unless otherwise stated, PUCCH requirement tests shall apply only for each subcarrier spacing declared to be supported

Number of interlaces

For PF0/1, only one interlace is supported. For PF2/3, one or two interlaces can be configured. From our understanding, number of interlace doesn’t change the receiving algorithm. We prefer to define requirements for all formats with same resource allocation. i.e. only one interlace is tested and interlaced index 0 can be used.

Proposal 2: Only test one interlace and use interlace index 0 for PF0/1/2/3.

Frequency hopping

No frequency hopping is supported for NR-U PUCCH with interlaced design. Our purpose is to verify the performance of PUCCH with interlaces structure, so frequency hopping shouldn’t be considered.

Proposal 3: Not configure frequency hopping for all cases.

Antenna configuration

For Rel-15 NR PUCCH performance requirements, 1T2R, 1T4R and 1T8R are defined. However, in this WI, our purpose is to verify the PUCCH’s performance with PRB-interlaced allocation. Therefore, we prefer to only define the requirements for 1T4R to reduce the number of test cases

Proposal 4: Use 1T4R for all cases.

Others:

For others parameters, we prefer to reuse them from NR PUCCH in 38.104 as much as possible.

The simulation assumptions for PF0 proposed by us are listed in Table 2:

Table 2: Test Parameters for PF0

| Parameter | Test |
| --- | --- |
| Number of UCI information bits | 1 |
| Intra-slot frequency hopping | N/A |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 13 |
| Antenna configuration | 1T4R |
| Channel bandwidth | 20MHz |
| SCS | 15kHz; 30kHz |
| Number of interlaces | 1 |
| Interlace index | 0 Note1 |
| Propagation conditions | TDLC300-100 low |
| Test metric | ProbPUCCH DTX→Ack bits ≤ 10-2 SNR@ProbACK miss=10-2 SNR@ProbNACK→ACK=10-3 |
| Note 1: RBs 0, 10, 20,…,100 are allocated for 15kHz and RBs 0,5,10,…,50 are allocated for 30kHz. |  |

The simulation assumptions for PF1 proposed by us are listed in Table 3:

Table 3: Test Parameters for PF1:

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of symbols | 14 |
| Intra-slot frequency hopping | N/A |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Antenna configuration | 1T4R |
| Channel bandwidth | 20MHz |
| SCS | 15kHz; 30kHz |
| Number of interlaces | 1 |
| Interlace index | 0 Note1 |
| Propagation conditions | TDLC300-100 low |
| Test metric | ProbPUCCH DTX→Ack bits ≤ 10-2 SNR@ProbACK miss=10-2 SNR@ProbNACK→ACK=10-3 |
| Note 1: RBs 0, 10, 20,…,100 are allocated for 15kHz and RBs 0,5,10,…,50 are allocated for 30kHz. |  |

For PF2, two cases are considered: one is in clause 8.3.4.1 in TS 38.104, the UCI bit number is 4 and the test metric is ACK miss; another is in clause 8.3.4.2, the UCI bit number is 22 and the test metric is UCI BLER. For NR-U, we prefer to only choose one from above two cases to reduce test numbers. The simulation assumptions provided by us are listed in Table 4.

Table 4: Test Parameters for PF2:

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| Intra-slot frequency hopping | N/A |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |
| Antenna configuration | 1T4R |
| Channel bandwidth | 20MHz |
| SCS | 15kHz; 30kHz |
| Number of interlaces | 1 |
| Interlace index | 0 Note 1 |
| Propagation conditions | TDLC300-100 low |
| OCC-Length-r16 | Not configured |
| Test metric | SNR @ ProbUCI block BLER=10-2 |
| Note 1: RBs 0, 10, 20,…,100 are allocated for 15kHz and RBs 0,5,10,…,50 are allocated for 30kHz. |  |

For PF3, too many test cases are defined such as different UCI symbols, different number of DMRS symbols. For NR-U PUCCH, only PRB-interlaced design and block-wise spreading is introduced, so not so many test cases are needed. We prefer to only choose one case as baseline to verify this new feature.

Table 5: Test Parameters for PF3:

| Parameter | Test 1 |
| --- | --- |
| Modulation order | QPSK |
| Intra-slot frequency hopping | N/A |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Additional DM-RS configuration | No additional DM-RS |
| Number of symbols | 4 |
| The number of UCI information bits | 16 |
| Channel bandwidth | 20MHz |
| SCS | 15kHz; 30kHz |
| Antenna configuration | 1T4R |
| Number of interlaces | 1 |
| Interlace index | 0Note 1 |
| Propagation conditions | TDLC300-100 low |
| NSFPUCCH,3 | 1 |
| Cyclic shift index for DMRS | 0 |
| Test metric | SNR @ UCI block BLER=10-2 |
| Note 1: RBs 0, 10, 20,…,90 are allocated for 15kHz and RBs 0,5,10,…,45 are allocated for 30kHz. Note 2: The UCI information does not contain CSI part 2. |  |

Proposal 5: Use Table 2~Table 5 as simulation assumptions for performance requirements for NR-U PF0/1/2/3 respectively

Conclusion

Proposal 1: Define the requirements for PRB-interlaced PUCCH resource allocation with following simulation setups:

- PF0/1/2/3

- Both 15 kHz and 30 kHz

- Test applicability rules:

- Unless otherwise stated, PUCCH requirement tests shall apply only for each PUCCH format declared to be supported

- Unless otherwise stated, PUCCH requirement tests shall apply only for each subcarrier spacing declared to be supported

Proposal 2: Only test one interlace and use interlace index 0 for PF0/1/2/3.

Proposal 3: Not configure frequency hopping for all cases.

Proposal 4: Use 1T4R for all cases.

Proposal 5: Use Table 2~Table 5 as simulation assumptions for performance requirements for NR-U PF0/1/2/3 respectively

Reference

[1]   R4-2012611 WF on NR-U BS demodulation requirements. Ericsson