3GPP TSG-RAN WG4 Meeting #97-e	R4-2015662

# Electronic Meeting, 2nd – 13th Nov., 2020

Title: 	Discussion on NR CA CQI reporting requirements

Source: 	Huawei, HiSilicon

Agenda item:	7.16.1.4

Document for:	Discussion

Background

In RAN4#96-e meeting, the WF[1] was approved with some open issues left. In this paper, we further share our views on those remaining open issues about NR CA CQI reporting requirements

Discussion

Duplex mode and SCS combinations

The options are shown as follows:

- Duplex mode and SCS combinations - For the performance requirements: - Option 1: Reuse the duplex mode and SCS combinations of PDSCH normal CA requirements - Option 2: - FR1: FDD + FDD with 15 kHz SCS and TDD + TDD with 30 kHz SCS - FR2: TDD + TDD with 120 kHz SCS - Option 3: - FR1: FDD + FDD with 15 kHz SCS, TDD + TDD with 30 kHz SCS, FDD 15 kHz +TDD 30kHz - FR2: TDD + TDD with 120 kHz SCS - Test applicability rule if the above proposed WF is agreed for FR1: - Option A: Test 3 cases - Option B : Test 2 cases - Candidate option for detailed applicability rule: - Test #1: FDD 15 kHz + TDD 30 kHz > FDD 15 kHz + FDD 15 kHz > FDD 15 kHz + TDD 15 kHz - Test #2: TDD 30 kHz + TDD 30 kHz > TDD 15 kHz + TDD 30 kHz - Option C: Test 1 cases

We prefer option 2. Supporting of different CA combinations are covered in PDSCH CA normal performance requirements and no need to repeat the verification of different CA duplex mode and SCS combinations in CA CQI test. FDD CA and TDD CA with same duplex mode and numerology is mandatory for UE to support, there is no scenario that one UE only supports FDD-TDD CA, from checking UE CA CQI performance point of view, it is enough to only cover the CA configurations with same duplex mode and same numerology.

Proposal 1: Use Duplex mode and SCS combinations as following:

- FR1: FDD + FDD with 15 kHz SCS and TDD + TDD with 30 kHz SCS

- FR2: TDD + TDD with 120 kHz SCS

Conclusion

In this paper, we share our view on remaining issues about NR CA CQI reporting requirements, our proposals are:

Proposal 1: Use Duplex mode and SCS combinations as following:

- FR1: FDD + FDD with 15 kHz SCS and TDD + TDD with 30 kHz SCS

- FR2: TDD + TDD with 120 kHz SCS

Reference

- R4-2012692, Way forward on CA CQI reporting requirements, RAN4#96e, China Telecom