| 3GPP TR 38.921 V0.2.0 (2020-10) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Parameters for IMT studies on 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5 GHz (Release 17 ) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	8

4	Co-existence study	9

4.1	Co-existence simulation scenarios	9

4.2	Co-existence simulation assumption	9

4.3	Co-existence simulation results	20

5	General parameters	20

6	BS parameters	20

6.1	Transmitter characteristics	20

6.2	Receiver characteristics	20

7	UE parameters	20

7.1	Transmitter characteristics	20

7.2	Receiver characteristics	20

8	Antenna characteristics	21

8.1	BS antenna characteristics	21

8.2	UE antenna characteristics	21

9	Other Information relevant for the sharing and compatibility studies	21

Annex <X> (informative): Change history	22

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document is a technical report for the study item on IMT parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz [2], covering the study on transmitter and receiver characteristics for both NR BS and NR UE, and related parameters for answering requests from ITU-R WP5D.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-200513: "Study on IMT parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz".

[3]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception"

[4]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone"

[5]	3GPP TR 38.820, “NR; 7-24 GHz frequency range”

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

3.2	Symbols

For the purposes of the present document, the following symbols apply:

NRB	Transmission bandwidth configuration, expressed in resource blocks

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AAS	Active Antenna System

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

AWGN	Additive White Gaussian Noise

BS	Base Station

BW	Bandwidth

FDD	Frequency Division Duplex

FR	Frequency Range

FRC	Fixed Reference Channel

ITU-R	Radiocommunication Sector of the International Telecommunication Union

LA	Local Area

MR	Medium Range

NR	New Radio

OTA	Over The Air

RB	Resource Block

RF	Radio Frequency

SCS	Sub-Carrier Spacing

TDD	Time division Duplex

WA	Wide Area

4	Co-existence study

4.1	Co-existence simulation scenarios

Table 4.1 summarizes the proposed scenarios to be considered for 6.425-7.125GHz and 10.0-10.5 GHz.

Table 4-1: Summary of considered scenario

| No. | Usage scenario | Aggressor | Victim | Direction | Simulation frequency | Deployment Scenario | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | eMBB | NR, 100MHz | NR, 100MHz | DL to DL | 7 GHz | Urban macro |  |
| 2 | eMBB | NR, 100MHz | NR, 100MHz | DL to DL | 7 GHz | Indoor hotspot |  |
| 3 | eMBB | NR, 100MHz | NR, 100MHz | DL to DL | 7 GHz | Dense urban | Down-prioritized |
| 4 | eMBB | NR, 100MHz | NR, 100MHz | UL to UL | 7 GHz | Urban macro |  |
| 5 | eMBB | NR, 100MHz | NR, 100MHz | UL to UL | 7 GHz | Indoor hotspot |  |
| 6 | eMBB | NR, 100MHz | NR, 100MHz | UL to UL | 7 GHz | Dense urban | Down-prioritized |
| 7 | eMBB | NR, 100MHz | NR, 100MHz | DL to DL | 10 GHz | Urban macro |  |
| 8 | eMBB | NR, 100MHz | NR, 100MHz | DL to DL | 10 GHz | Indoor hotspot |  |
| 9 | eMBB | NR, 100MHz | NR, 100MHz | DL to DL | 10 GHz | Dense urban | Down-prioritized |
| 10 | eMBB | NR, 100MHz | NR, 100MHz | UL to UL | 10 GHz | Urban macro |  |
| 11 | eMBB | NR, 100MHz | NR, 100MHz | UL to UL | 10 GHz | Indoor hotspot |  |
| 12 | eMBB | NR, 100MHz | NR, 100MHz | UL to UL | 10 GHz | Dense urban | Down-prioritized |

4.2	Co-existence simulation assumption

### 4.2.1		Network layout model

#### 4.2.1.1	Urban macro

Details on urban macro network layout model are listed in Tables 4.2.1.1-1 and 4.2.1.1-2.

Table 4.2.1.1-1: Single operator layout for urban macro

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |  |
| Inter-site distance |  | 0.45 km (urban) 0.9 km (suburban) | Based on cell radius: 0.3 km (urban) 0.6 km (suburban) |
| BS antenna height |  | 20 m (urban) 25 m (suburban) |  |
| UE location [6] | Outdoor/indoor | Outdoor and indoor |  |
|  | Indoor UE ratio | 20% |  |
|  | Low/high Penetration loss ratio | 50% low loss, 50% high loss |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | UE antenna height | Same as 3D-UMa in TR 36.873 |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 35 m |  |
| Channel model |  | UMa |  |
| Shadowing correlation |  | Between cells: 1.0 Between sites: 0.5 |  |

Table 4.2.1.1-2: Multi operators layout for urban macro

| Parameters | Values | Remark |
| --- | --- | --- |
| Multi operators layout | coordinated operation (0% Grid Shift) and un-coordinated operation (100% Grid Shift) | RAN4 has long been using un-coordinated operation in below 6GHz coexistence simulation |

| Coordinated Operation: each network with co-location of sites |  |
| --- | --- |

Figure 4.2.1.1-1: Coordinated operation

| Uncoordinated Operation: second network’s sites are located at the first network’s cell edge |  |
| --- | --- |

Figure 4.2.1.1-2: Uncoordinated operation

#### 4.2.1.2	Dense urban

It is agreed to down-prioritized the dense urban scenario in this coexistence study, because it has the least demanding ACIR requirements among the three simulated scenarios in TR 38.803.

#### 4.2.1.3	Indoor

Details on indoor network layout model are listed in Tables 4.2.1.3-1 and 4.2.1.3-2.

Table 4.2.1.3-1: Single operator layout for indoor

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | 50m x 120m, 12BSs |  |
| Inter-site distance |  | 20m |  |
| BS antenna height |  | 3 m | ceiling |
| UE location | Outdoor/indoor | Indoor |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | UE antenna height | 1 m |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 0 m |  |
| Channel model |  | Indoor Office |  |
| Shadowing correlation |  | NA |  |

Figure 4.2.1.3-1: Network layout for indoor

Table 4.2.1.3-2: Multi operators layout for indoor

| Parameters | Values | Remark |
| --- | --- | --- |
| Multi operator layout | Coordinated operation (0% Grid Shift) |  |

### 4.2.2		Propagation model

#### 4.2.2.1	Path loss

The pathloss models are summarized in Table 4.2.2.1-1 and the distance definitions are indicated in Figures 4.2.2.1-1 and 4.2.2.1-2. Note that the distribution of the shadow fading is log-normal, and its standard deviation for each scenario is given in Table 4.2.2.1-1.

| Figure 4.2.2.1-1: Definition of d2D and d3D for outdoor UTs | Figure 4.2.2.1-2: Definition of d2D-out, d2D-in and d3D-out, d3D-in for indoor UTs. |
| --- | --- |

Note that

(4.2.2-1)

Table 4.2.2.1-1: Pathloss models

| Scenario | LOS/NLOS | Pathloss [dB], fc is in GHz and d is in meters, see note 4 | Shadow fading std [dB] | Applicability range, antenna height default values |
| --- | --- | --- | --- | --- |
| UMa | LOS | , see note 1 |  |  |
|  | NLOS | for |  | Explanations: see note 3 |
|  |  | Optional |  |  |
| InH - Office | LOS |  |  |  |
|  | NLOS |  |  |  |
|  |  | Optional |  |  |
| Note 1: Breakpoint distance d'BP = 4 h'BS h'UT fc/c, where fc is the centre frequency in Hz, c = 3.0108 m/s is the propagation velocity in free space, and h'BS and h'UT are the effective antenna heights at the BS and the UT, respectively. The effective antenna heights h'BS and h'UT are computed as follows: h'BS = hBS – hE, h'UT = hUT – hE, where hBS and hUT are the actual antenna heights, and hE is the effective environment height. For UMi hE = 1.0m. For UMa hE=1m with a probability equal to 1/(1+C(d2D, hUT)) and chosen from a discrete uniform distribution uniform(12,15,…,(hUT-1.5)) otherwise. With C(d2D, hUT) given by , where . Note that hE depends on d2D and hUT and thus needs to be independently determined for every link between BS sites and UTs. A BS site may be a single BS or multiple co-located BSs. Note 2: The applicable frequency range of the PL formula in this table is 0.5 < fc < fH GHz, where fH = 30 GHz for RMa and fH = 100 GHz for all the other scenarios. It is noted that RMa pathloss model for >7 GHz is validated based on a single measurement campaign conducted at 24 GHz. Note 3: UMa NLOS pathloss is from TR36.873 with simplified format and PLUMa-LOS = Pathloss of UMa LOS outdoor scenario. Note 4: fc denotes the center frequency normalized by 1GHz, all distance related values are normalized by 1m, unless it is stated otherwise. |  |  |  |  |

#### 4.2.2.2	LOS probability

The Line-Of-Sight (LOS) probabilities are given in Table 4.2.2.2-1.

Table 4.2.2.2-1 LOS probability

| Scenario | LOS probability (distance is in meters) |
| --- | --- |
| UMa | where |
| Indoor - Mixed office |  |
| Indoor - Open office |  |
| Note: The LOS probability is derived with assuming antenna heights of 3m for indoor, 10m for UMi, and 25m for UMa |  |

#### 4.2.2.3	O-to-I penetration loss

#### 4.2.2.3.1	O-to-I building penetration loss

The pathloss incorporating O2I building penetration loss is modelled as in the following:

(4.2.2-2)

where  is the basic outdoor path loss given in Section 4.2.2.1, where  is replaced by  .  is the building penetration loss through the external wall,  is the inside loss dependent on the depth into the building, and σP is the standard deviation for the penetration loss.

is characterized as:

(4.2.2-3)

is an additional loss is added to the external wall loss to account for non-perpendicular incidence; , is the penetration loss of material i, example values of which can be found in Table 4.2.2.3-1;  is proportion of i-th materials, where ; and N is the number of materials.

Table 4.2.2.3-1: Material penetration losses

| Material | Penetration loss [dB] |
| --- | --- |
| Standard multi-pane glass |  |
| IRR glass |  |
| Concrete |  |
| Wood |  |
| Note: f is in GHz |  |

Table 4.2.2.3-2 gives ,  and σP for two O2I penetration loss models. The O2I penetration is UT-specifically generated and is added to the SF realization in the log domain.

Table 4.2.2.3-2: O2I building penetration loss model

|  | Path loss through external wall: in [dB] | Indoor loss: in [dB] | Standard deviation: σP in [dB] |
| --- | --- | --- | --- |
| Low-loss model |  | 0.5 | 4.4 |
| High-loss model |  | 0.5 | 6.5 |

is minimum of two independently generated uniformly distributed variables between 0 and 25 m for UMa and UMi-Street Canyon, and between 0 and 10 m for RMa.  shall be UT-specifically generated.

Both low-loss and high-loss models are applicable to UMa and UMi-Street Canyon.

Only the low-loss model is applicable to RMa.

Only the high-loss model is applicable to InF.

#### 4.2.2.3.2	O-to-I car penetration loss

The pathloss incorporating O2I car penetration loss is modelled as in the following:

(4.2.2-4)

where  is the basic outdoor path loss given in Section 4.2.2.1. μ = 9, and σP = 5. The car penetration loss shall be UT-specifically generated. Optionally, for metallized car windows, μ = 20 can be used. The O2I car penetration loss models are applicable for at least 0.6-60 GHz.

### 4.2.3		Antenna and beam forming pattern modelling

The BS antenna is modelled as described in subclause 8.1.1 using parameters for different BS deployments listed in subclause 8.1.2.

### 4.2.4		Transmission power control model

For downlink scenario, no power control scheme is applied.

For uplink scenario, TPC model specified in Section 9.1 TR 36.942 is applied with following parameters.

-	CLx-ile = 88 + 10*log10(200/X) + 11 – Y, where X is UL transmission BW (MHz) and Y is the BS noise figure

-	γ = 1

### 4.2.5		Received power model

The received power in downlink and uplink scenarios is defined as below:

RX_PWR = TX_PWR – Path loss + G_TX + G_RX

where:

RX_PWR is the received power

TX_PWR is the transmitted power

G_TX is the transmitter antenna gain (directional array gain)

G_RX is the receiver antenna gain (directional array gain).

### 4.2.6		ACLR and ACS modelling

For DL it seems reasonable from the perspective of simulating worst case scenarios that we assume BS ACLR is modelled as flat in space, and the UE ACS can be modelled flat in space.

If this assumption is for DL, then the similar assumption could be made for the UL because:

-	UE has a much small number of antennas, thus the effect of directivity should be smaller for ACLR (or the adjacent channel interference). It can also be reasonably assumed that the UE ACLR will play a dominant role than the BS ACS in the adjacent channel interference.

-	Again, BS ACS flat in space might mean worse coexistence performance than actual performance because BS has better capability of steering its receive antennas to suppress interference.

If a UE occupies a smaller bandwidth than the channel bandwidth for transmission, a two stop ACLR model could be considered in frequency to avoid overly estimating interference, as done in E-UTRA coexistence study (as recorded in TR 36.942).

Therefore, it is assumed that both ACLR (or the adjacent channel interference) and ACS are flat in both space and frequency. The ACIR model can be express as

(assuming ACLR, ACS and ACIR to be linear).

### 4.2.7		Link level performance for 5G NR coexistence

The throughput of a modem with link adaptation can be approximated by an attenuated and truncated form of the Shannon bound. (The Shannon bound represents the maximum theoretical throughput than can be achieved over an AWGN channel for a given SNIR). The following equations approximate the throughput over a channel with a given SNIR, when using link adaptation:

Where:

S(SNIR)		Shannon bound, S(SNIR) =log2(1+SNIR) bps/Hz
			Attenuation factor, representing implementation losses
SNIRMIN		Minimum SNIR of the code set, dB
SNIRMAX		Maximum SNIR of the code set, dB

The parameters α, SNIRMIN and SNIRMAX can be chosen to represent different modem implementations and link conditions. The parameters proposed in Table 4.2.7-1 represent a baseline case, which assumes:

-	1:1 antenna configuration

-	AWGN channel model

-	Link Adaptation (see Table 4.2.7-1 for details of the highest and lowest rate codes)

-	No HARQ

Table 4.2.7-1: Parameters describing baseline Link Level performance for 5G NR

| Parameter | DL | UL | Notes |
| --- | --- | --- | --- |
| α, attenuation | 0.6 | 0.4 | Represents implementation losses |
| SNIRMIN, dB | -10 | -10 | Based on QPSK, 1/8 rate (DL) & 1/5 rate (UL) |
| SNIRMAX, dB | 30 | 22 | Based on 256QAM 0.93(DL) & 64QAM 0.93 (UL) |

Note that the parameters proposed in Table 4.2.7-1 are targeted for eMBB coexistence scenario.

### 4.2.8		Other simulation parameters

Table 4.2.8-1: Other simulation parameters

| Parameters | Indoor | Urban macro | Dense urban |
| --- | --- | --- | --- |
| Carrier frequency | 7GHz, 10GHz | 7GHz, 10GHz | Down-prioritized |
| Channel bandwidth | 100MHz | 100MHz | Down-prioritized |
| Scheduled channel bandwidth per UE (DL) | 98.28MHz | 98.28MHz | Down-prioritized |
| Scheduled channel bandwidth per UE (UL) | 32.76MHz | 32.76MHz | Down-prioritized |
| The number of active UE (DL) (Note 1) | 1 | 1 | Down-prioritized |
| The number of active UE (UL) (Note 1) | 3 | 3 | Down-prioritized |
| Traffic model | Full buffer | Full buffer | Down-prioritized |
| DL power control | NO | NO | Down-prioritized |
| UL power control | YES | YES | Down-prioritized |
| BS max TX power in dBm | 24 | 43 | Down-prioritized |
| UE max TX power in dBm | 23 or 20 (Note 2) | 23 or 20 (Note 2) | Down-prioritized |
| UE min TX power in dBm | -33 | -33 | Down-prioritized |
| BS Noise figure in dB | 14 (@7GHz) 15 (@10GHz) | 6 (@7GHz) 7 (@10GHz) | Down-prioritized |
| UE Noise figure in dB | 9 | 9 | Down-prioritized |
| Handover margin | 3dB | 3dB | Down-prioritized |
| Note 1 Same as the number of BS beam(s) Note 2: 20dBm as optional case where CLx-ile should be reduced by 3dB |  |  |  |

### 4.2.9		Co-existence simulation methodology

Adopt following simulation steps.

1.	Aggressor and victim network are generated.

-	UEs are distributed randomly across the network.

2.	UE associations: UEs are associated to base station based on coupling loss.

-	Associations are made assuming a single element at both UE and BS.

3.	Once association is done, round robin scheduling is used. BF weights are adjusted to point to the LOS direction between BS-UE. This is done for both victim and aggressor networks.

4.	Throughput is computed in the victim systems without considering ACI as below:

-	, where  is the inter-cell interference.

5.	Throughput is computed considering ACI as below:

-	, where  is the adjacent channel interference.

6.	RF parameters are determined based on the degradation cause by ACI as below:

-	.

4.3	Co-existence simulation results

5	General parameters

5.1	Duplex mode

For both frequency ranges, even if FDD is not precluded, it’s most likely that TDD should be used in these frequency ranges.

5.2	Channel Bandwidth

A pragmatic, simple and non-ambiguous answers should be provided to ITU-R. While a number of channel bandwidth would be specified for these frequency ranges, 100MHz has been considered as a representative channel bandwidth that will be used.

5.3	Signal Bandwidth

The signal bandwidth for a 100MHz channel bandwidth signal is calculated based on the NR spectrum utilization for 30kHz SCS:

Signal bandwidth = NRB x SCS x 12

with NRB: Number of Resource block for 100 MHz channel bandwidth and 30kHz SCS, as specified in TS 38.104 [3].

5.4	SINR operating range

The throughput of a modem with link adaptation can be approximated by an attenuated and truncated form of the Shannon bound. (The Shannon bound represents the maximum theoretical throughput than can be achieved over an AWGN channel for a given SNIR). The following equations approximate the throughput over a channel with a given SNIR, when using link adaptation:

Where:

S(SNIR)		Shannon bound, S(SNIR) =log2(1+SNIR) [bps/Hz]
			Attenuation factor, representing implementation losses
SNIRMIN  	Minimum SNIR of the code set, dB
SNIRMAX 	Maximum SNIR of the code set, dB

The parameters α, SNIRMIN and SNIRMAX can be chosen to represent different modem implementations and link conditions. The parameters proposed in table 5.4-1 represent a baseline case, which assumes:

-  1:1 antenna configurations

-  AWGN channel model

-  Link Adaptation (see table 5.4-1 for details of the highest and lowest rate codes)

-  No HARQ

Table 5.4-1:	Parameters describing baseline Link Level performance for 5G NR

| Parameter | DL | UL | Notes |
| --- | --- | --- | --- |
| α | 0.6 | 0.4 | Represents implementation losses |
| SNIRMIN, dB | -10 | -10 | Based on QPSK, 1/8 rate (DL) & 1/5 rate (UL) |
| SNIRMAX, dB | 30 | 22 | Based on 256QAM 0.93(DL) & 64QAM 0.93 (UL) |

6	BS parameters

6.1	Transmitter characteristics

### 6.1.1	Power dynamic range

There is no power control in downlink and fixed power per resource block is assumed in the co-existence simulation. Hence 0 dB power dynamic range was agreed for the LS reply.

### 6.1.2	Spectral mask

### 6.1.3	ACLR

### 6.1.4	Spurious emissions

### 6.1.5	Maximum output power

The maximum output power will be provided in the antenna parameter table. It was agreed to be aligned with antenna characteristics.

The Total Radiated Power for two polarizations was agreed as shown in table 6.1.5-1 below.

| Parameter | Macro Sub-urban | Macro Urban | Micro Urban |
| --- | --- | --- | --- |
| Total Radiated Power for two polarizations (dBm) | 46 | 46 | 37 |

### 6.1.6	Average output power

It was agreed the average output power won’t be mentioned in the reply LS.

6.2	Receiver characteristics

### 6.2.1	Noise figure

From the TR 38.820 [5] for 7-24 GHz, the typical Noise Figure for a Wide Area BS operating at 10 GHz is 7dB (12dB for Medium Range BS and 15dB for Local Area BS).

Table 5.5.1.1-1 of TR 38.820: Typical noise figure for 7 – 24 GHz example frequencies

| Example frequency (GHz) | Typical NF values for NR BS (dB) | Typical NF values for NR UE (dB) |
| --- | --- | --- |
| 10 | 7 | 9 |
| 15 | 8 | 10 |
| 20 | 9 | 10 |

For 6.425-7.125 GHz, the typical Noise Figure for a Wide Area BS operating at 6 GHz was agreed to be 6dB (11dB for Medium Range BS and 14dB for Local Area BS).

### 6.2.2.	Sensitivity

As it is not clear if it will have a conducted sensitivity requirement for both frequency ranges, however the OTA sensitivity requirement will be needed either way and will be based on the NF and the antenna gain:

Where: BW is the configured bandwidth of the FRC, NF is the noise figure, IM is implementation margin not related to antenna array, SNR is the required SNR to reach 95% throughput and G is the antenna gain including RF losses and 3dB off peak margin.

However, the sensitivity is not a critical parameter for sharing and compatibility studies. It was agreed to not mention any value for this parameter.

### 6.2.3	Blocking response

### 6.2.4	ACS

7	UE parameters

7.1	Transmitter characteristics

7.2	Receiver characteristics

8	Antenna characteristics

8.1	BS antenna characteristics

### 8.1.1		Array antenna model

In Table 8.1.1-1, the parameters used by the parameterized array antenna model are described.

Table 8.1.1-1: Parameters of the parameterized array antenna model

| Parameter | Symbol | Unit |
| --- | --- | --- |
| Front to back ratio | Am | dB |
| Side lobe suppression | SLAv | dB |
| Horizontal HPBW | 3dB | Degrees |
| Vertical HPBW | 3dB | Degrees |
| Array element peak gain | GE,max | dBi |
| Number of radiating elements rows and columns | (M, N) | Integer |
| Horizontal element separation | dh | m |
| Vertical element separation | dv | m |
| Electrical down-tilt angle | etilt | Degrees |
| Electrical scan angle | escan | Degrees |

The parameterized antenna model is built around array antenna model where the element factor, array factor and linear phase progressing is characterized as described by equations in Table 8.1.1-2.

Table 8.1.1-2: Array antenna model details

| Description | Equation | Unit |
| --- | --- | --- |
| Peak normalized element radiation pattern |  | dB |
| Peak gain normalized element radiation pattern |  | dBi |
| Composite array radiation pattern | , where | dBi |

### 8.1.2		Array antenna parameters

In Table 8.1.2-1, base station array antenna parameters for different deployment scenarios is listed. Element parameters have been selected to produce correct element peak gain.

Table 8.1.2-1: BS array antenna parameters

| Parameter | Rural | Macro Sub-urban | Macro Urban | Micro Urban |
| --- | --- | --- | --- | --- |
| Am (dB) | N/A | 30 | 30 | 30 |
| SLAv (dB) | N/A | 30 | 30 | 30 |
| 3dB (deg.) | N/A | 90 | 90 | 90 |
| 3dB (deg.) | N/A | 65 | 90 | 90 |
| GE,max (dBi) | N/A | 6.4 | 5.5 | 5.5 |
| LE (dB) | N/A | 2.0 | 2.0 | 2.0 |
| (M, N) | N/A | (16, 8) | (16, 8) | (8,8) |
| dh (m) | N/A | 0.5 | 0.5 | 0.5 |
| dv (m) | N/A | 0.7 | 0.5 | 0.5 |
| Horizontal coverage range (deg.) | N/A | +/- 60 | +/- 60 | +/- 60 |
| Vertical coverage range (deg.) | N/A | 90 to 100 | 90 to 120 | 90 to 120 |
| Total Radiated Power (dBm) | N/A | 46 | 46 | 37 |
| Mechanical downtilt (deg.) | N/A | 6 | 10 | N/A |
| Note 1: MxN means there are M vertical and N horizontal elements Note 2: LE is included in GE,max Note 3: TRP includes the powers from two polarizations Note 4: The vertical coverage range includes the mechanical downtilt. |  |  |  |  |

8.2	UE antenna characteristics

The outcome of the RAN4 study for collecting technical background information relevant for the frequency range 7 to 24 GHz indicated that the frequency range 7.25-[10-13] GHz would have “FR1 like” requirements, and as such we can assume that in the 10-10.5GHz range this applies. The UE will most likely therefore have a conducted interface with an assumed isotropic radiation pattern antenna and no beam forming.

9	Other Information relevant for the sharing and compatibility studies

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2020-03 | RAN4#94bis-e | R4-2004477 |  |  |  | TR skeleton | 0.0.1 |
| 2020-08 | RAN4#96-e | R4-2010370 |  |  |  | - Agreed Text Proposal in RAN4#95-e: R4-2008928, “TP to TR 38.9xx: System level simulation methodology and assumptions for study on IMT parameters for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz” 2. A new clause is added on general parameters | 0.1.0 |
| 2020-10 | RAN4#97-e | R4-2015675 |  |  |  | Agreed Text Proposal in RAN4#96-e: R4-2011829, “TP to TR 38.921: BS IMT technology related parameters” R4-2011830, “TP to TR 38.921: Addition of BS antenna model and parameters in subclause 4.2.3 and subclause 8.1” | 0.2.0 |