3GPP TSG-RAN WG4 Meeting # 97-e	R4-2015681

# Electronic Meeting, 2-13 Nov., 2020

Title:	LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-23 (6.425 to 10.5 GHz)

Response to:	LS RP-200042 on Parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-23 from ITU-R Working Party 5D

Release:	-

Work Item:	-

Source:	TSG RAN WG4

To:	ITU-R WP5D

Cc:	RAN

Contact person:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1	Overall description

RAN WG4 received the incoming LS from ITU-R Working Party 5D on Parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-23 (Att. 7.4 to 5D/134. A first LS response was given from TSG RAN (RP-200514), followed by a response from TSG RAN WG4 covering parameters for operation between 470 MHz and 4990 MHz (RP-200559) and a response covering antenna parameters for operation in the range 6.425 to 10.5 GHz (R4-2011932). This document is a further response covering IMT-2020 technology-related and deployment-related parameters for operation in the range 6.425 to 10.5 GHz. RAN WG4 may forward additional information at a later stage.

The 5G RF parameters for the FR1 bands are specified in 3GPP specifications TS 38.104 for the BS and TS 38.101-1 for the UE. Some of the recommended IMT-2020 technology related parameters are given in Annex 1 of this LS with references to those two specifications.

In addition, RAN WG4 would like to inform WP5D that RAN WG4 has developed a Technical Report TR 38.921, to cover the study on transmitter and receiver characteristics for both NR BS and NR UE, and information relevant to the sharing and compatibility studies for answering requests from ITU-R WP5D. And 3GPP is willing to support WP5D to provide further feedback w.r.t features of IMT systems, if requested.

2	Actions

To ITU-R WP5D

ACTION: 	3GPP TSG RAN asks ITU-R WP 5D to take the above information on IMT system parameters for its consideration.

3	Dates of next TSG RAN WG 4 meetings

TSG-RAN4 Meeting #98-e	25 Jan. - 05 Feb. 2021	Online

ANNEX 1

IMT-2020 technology-related and deployment-related parameters for bands between 6425 and 10500 MHz

TABLE 1

IMT-2020 technology related parameters in 6425 – 10500 MHz

|  |  | IMT |  |
| --- | --- | --- | --- |
| No. | Parameter | Base station | Mobile station |
| 1 | Duplex Method | TDD | TDD |
| 2 | Channel bandwidth (MHz) | 100 MHz | 100 MHz |
| 3 | Signal bandwidth (MHz) | Signal bandwidth = NRB x SCS x 12. | Signal bandwidth = NRB x SCS x 12. |
| 4 | Transmitter characteristics |  |  |
| 4.1 | Power dynamic range (dB) | 0 dB | 56 dB for 100 MHz channel bandwidth |
| 4.2 | Spectral mask (dB) | See below “operating band unwanted emission limits” | See [2], § 6.5.2.2. |
| 4.3 | ACLR | 36 dB | 28 dB |
| 4.4 | Spurious emissions | See [1], § 6.6.5. | See [2], § 6.5.3. |
| 4.5 | Maximum output power | Provided in antenna parameter table | 23 dBm |
| 5 | Receiver characteristics |  |  |
| 5.1 | Noise figure (dB) | For 6.425-7.125 GHz: 6 dB (Wide Area BS) 11 dB (Medium Range BS) 14 dB (Local Area BS) For 10.0-10.55 GHz: 7 dB (Wide Area BS) 12 dB (Medium Range BS) 15 dB (Local Area BS) For BS class definitions, For BS class definitions, see [1], § 4.4 | 9 dB |
| 5.2 | Sensitivity (dBm) | n/a | n/a |
| 5.3 | Blocking response | See [1], § 7.5 and § 7.4.2. See [1], § 10.6 and § 10.5.2. There is one exception that the -15 dBm CW interfering signal applies from 1 MHz to FUL,low – 200MHz and from FUL,high + 200MHz up to 12750 MHz | See [2], § 7.6 and § 7.7 for frequency larger than 3300 MHz. |
| 5.4 | ACS | 46 dB | 33 dB |
| 5.5 | SINR operating range (dB) | See below “SINR operating range and mapping function” |  |

References used in the Table:

[1]	3GPP TS 38.104 v.16.5.0, “NR; Base Station (BS) radio transmission and reception”.

[2]	3GPP TS 38.101-1 v.16.5.0. “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

SINR operating range and mapping function

The following equations approximate the throughput over a channel with a given SNIR, when using link adaptation:

Where:

S(SNIR)	Shannon bound, S(SNIR) =log2(1+SNIR) [bps/Hz]
		Attenuation factor, representing implementation losses
SNIRMIN  	Minimum SNIR of the code set, dB
SNIRMAX 	Maximum SNIR of the code set, dB

The parameters α, SNIRMIN and SNIRMAX can be chosen to represent different modem implementations and link conditions. The parameters proposed in table 2 represent a baseline case, which assumes:

- 1:1 antenna configurations

- AWGN channel model

- Link Adaptation (see table 2 for details of the highest and lowest rate codes)

- No HARQ

Table 2: Parameters describing baseline Link Level performance for 5G NR

| Parameter | DL | UL | Notes |
| --- | --- | --- | --- |
| α | 0.6 | 0.4 | Represents implementation losses |
| SNIRMIN, dB | -10 | -10 | Based on QPSK, 1/8 rate (DL) & 1/5 rate (UL) |
| SNIRMAX, dB | 30 | 22 | Based on 256QAM 0.93(DL) & 64QAM 0.93 (UL) |

Operating band unwanted emission limits

The basic limits for operating band unwanted emission is defined as below.

Table 3: Wide Area BS operating band unwanted emission limits

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 50 MHz | 0.05 MHz f_offset < 50.05 MHz |  | 100 kHz |
| 50 MHz f < min(100 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -14 dBm | 100 kHz |
| 100 MHz f fmax | 100.5 MHz f_offset < f_offsetmax | -15 dBm | 1MHz |