3GPP TSG-RAN WG4 Meeting #97-e	R4-2015744

Electronic Meeting, 2 – 13 November, 2020

Title: 	Discussion on remaining issues in SCell dormancy and cross-carrier scheduled BWP switching

Source: 	Huawei, HiSilicon

Agenda item:	7.5.2.2

Document for:	Discussion

Introduction

RRM requirements for SCell dormancy have been discussed in RAN4#96-e, and the outcomes are captured in the WF [1]. Basically all the requirements related to SCell dormancy are concluded except two issues are pending on RAN1 agreement:

- Whether dormancy indication received after first 3 OFDM symbol is possible for DCI 0_1 and 1_1

- Whether dormancy indication received outside DRX active time causes visible delay and interruption

In addition, the delay for active BWP switching in cross-carrier scheduling case is also to be discussed.

In this paper we will provide our views on the remaining issues in SCell dormancy and cross-carrier scheduled BWP switching.

Discussion

  - SCell dormancy

In [1] it is agreed that

Agreement: - RAN4 captures the delay requirement for triggering after first 3 OFDM symbols in a DCI format-agnostic manner - In case RAN1 introduces restrictions for DCI formats 0_1 and/or 1_1, RAN4 will update the RRM specification accordingly. - Additional [1] slot relaxation is applied to compensate for cross carrier scheduling - In case SCS differs between spCell and SCell, the smaller SCS applies. - Additional Z=1 slot (spCell numerology) relaxation is applied for DCI received after first X=3 symbols in a slot

Correspondingly, an editor note is added in the specification 38.133:

Editor’s Note: The requirements are defined in DCI-agnostic manner, if RAN1 decides that DCI 0_1 and/or 1_1 for triggering dormancy switch cannot be transmitted after the first 3 OFDM symbols in a slot, RAN4 can revise the specification text accordingly.

Based on RAN1#102-e discussion, there is no restriction on the time location of dormancy indication in DCI 0_1/1_1:

Agreement: For DCI format 1_1/0_1 on primary cell with dormancy indication that indicates a BWP change between dormant and non-dormant BWPs of SCell(s), there is no additional restriction that it should be only in first 3 symbols of a slot - If the DCI format 1_1/0_1 is detected after the first 3 symbols of a slot, a delay of one additional slot is given for BWP change - Note: UE does not expect to detect DCI format 1_1/0_1 on primary cell with dormancy indication that indicates a BWP change between dormant and non-dormant BWPs of SCell(s) after the first 3 symbols of a slot if UE does not report any capability to support PDCCH monitoring outside the first three symbols of a slot on primary cell.

As a result, the editor note in 38.133 can be directly removed.

Proposal 1: Remove the following editor note in 38.133 based on RAN1 agreement:

Editor’s Note: The requirements are defined in DCI-agnostic manner, if RAN1 decides that DCI 0_1 and/or 1_1 for triggering dormancy switch cannot be transmitted after the first 3 OFDM symbols in a slot, RAN4 can revise the specification text accordingly.

In [1] it is agreed that

- For BWP transition in a single SCell into/out of dormancy triggered by DCI 2-6 outside active time, the same set of switch delay requirements shall be applied as for triggering inside active time. - For BWP transition in a single SCell into/out of dormancy triggered by DCI 2-6 outside active time, the same set of switch interruption requirements shall be applied as for triggering inside active time. - If RAN1 defines something that makes Dormant BWP switching time/interruption outside DRX active time to always be absorbed into WUS gap, RAN4 revises the specification text accordingly.

Correspondingly, an editor note is added in the specification 38.133:

Editor’s Note: The requirements are defined in DCI-agnostic manner, if RAN1 defines something that makes Dormant switching time/interruption to always be absorbed into WUS gap, RAN4 can revise the specification text accordingly.

Based on RAN1#102-e discussion, there is no conclusion on the time location of dormancy indication in DCI 2_6 (the related issues are discussed in RAN1 Email discussion [102-e-NR-MRDC-CA-Dormancy-02] without conclusion). Therefore, RAN4 should further wait for the RAN1 agreement.

Cross carrier scheduled DCI-based BWP switch

In [1] it is agreed

- Existing Rel-15/Rel-16 BWP switching requirements cannot apply for cross carrier scheduled DCI-based BWP switch on single/multiple CCs - RAN4 to define requirements for cross carrier scheduled DCI-based BWP switch on single/multiple CCs in Rel-16

In our previous paper [2], we discussed the additional factors in BWP switching that is triggered by cross-carrier scheduling as compared to that triggered by self-scheduling. They are re-produced below.

- UE internal signalling delay for cross-CC communication: Different serving cells may work on different hardware/software resources internally in the UE. There could be some signalling delay for cross-CC communication, which means the start of processing for BWP switching could be later when the DCI is received on a different cell (cross-carrier scheduling case) compared to the case when DCI is received on the same cell where BWP switch occurs (self-scheduling case).

- Receive time difference between CCs: If the timing of scheduled cell is earlier than the scheduling cell, the time left for UE processing is reduced by the receive time difference. For inter-band CA the MRTD can be 33us for FR1-FR1 CA and 25us for FR1-FR2 CA. The time is non-negligible especially for large SCS where the requirement is already tight.

- Different SCS between scheduling cell and scheduled cell

  - Case 1: SCS of scheduling cell is smaller than SCS of scheduled cell

As shown in Figure 1, where DCI on scheduling cell with 15kHz SCS triggers BWP switch on scheduled cell with 120kHz SCS, the end of the scheduling DCI is on symbol #9 of slot #(n+1) on scheduled cell. This means quite some UE processing time would be lost due to the long DCI duration, since for self-scheduling case, the DCI would end at symbol#3 of slot #n.

Figure 1: Cross carrier scheduled BWP switch (scheduling CC 15kHz and scheduled CC 120kHz)

  - Case 2: SCS of scheduling cell is larger than SCS of scheduled cell

As shown in Figure 2, where DCI on scheduling cell with 120kHz SCS triggers BWP switch on scheduled cell with 15kHz SCS, the end of the scheduling DCI is on symbol #12 of slot #m on scheduled cell. The start point of BWP switch should be determined by the scheduling cell, i.e. slot #(n+7) in case of Figure 2, instead of slot #m. On the other hand, in this case, the DCI duration is shorter compared to self-scheduling case, which can compensate the loss of processing time due to UE internal signalling delay for cross-CC communication and receive time difference between CCs.

Figure 2: Cross carrier scheduled BWP switch (scheduling CC 15kHz and scheduled CC 120kHz)

In RAN4#96-e, it is discussed how to address the additional challenges in dormancy switching, which is always triggered by cross-carrier scheduling, and it is agreed that the dormancy switching delay is relaxed by 1 slot (corresponding to the smaller SCS between the scheduling cell and the scheduled cell) compared to the Rel-15 BWP switching delay which is based on self-scheduling. In our view, it is reasonable to follow the same principle for BWP switching that is triggered by cross-carrier scheduling.

Proposal 2: Reuse the dormancy switching delay requirements for BWP switching triggered by cross-carrier scheduling

- BWP switching delay is relaxed by 1 slot w.r.t. the smaller SCS between the scheduling cell and the scheduled cell compared to the existing BWP switching delay

Conclusions

In this paper we provided our views on the remaining issues in SCell dormancy and cross-carrier scheduled BWP switching.

Proposal 1: Remove the following editor note in 38.133 based on RAN1 agreement:

Editor’s Note: The requirements are defined in DCI-agnostic manner, if RAN1 decides that DCI 0_1 and/or 1_1 for triggering dormancy switch cannot be transmitted after the first 3 OFDM symbols in a slot, RAN4 can revise the specification text accordingly.

Proposal 2: Reuse the dormancy switching delay requirements for BWP switching triggered by cross-carrier scheduling

- BWP switching delay is relaxed by 1 slot w.r.t. the smaller SCS between the scheduling cell and the scheduled cell compared to the existing BWP switching delay

Reference

- R4-2012278, Way Forward on SCell Dormancy, Ericsson

- R4-2011152, Discussion on SCell dormancy, Huawei, Hisilicon