3GPP TSG-RAN WG4 Meeting #97-e	R4-2015750

Electronic Meeting, 2 – 13 November, 2020

Title: 	Discussion on remaining issues for RSTD measurement requirements

Source: 	Huawei, HiSilicon

Agenda item:	7.7.2.1

Document for:	Discussion

Introduction

The RRM core requirements for RSTD measurements has been completed in RAN4#96-e, and the measurement requirements are introduced based on CR [1]. On the other hand, three are some remaining open issues as captured in [1] and WF [2].

- Requirements for multiple PRS layers in non-overlapping case

- Calculation of PRS sample duration L

- Measurement period extension due to SSB collision

- Measurement period when configured with PRS-RSRP

- Requirement applicability related to UE processing capability N

In this paper we will provide our views on the remaining issues for RSTD measurement requirements.

Discussion

  - Requirements for multiple PRS layers in non-overlapping case

When measurement gaps and processing time T have overlap between different positioning frequency layers, TRSTD,Total is defined as: TRSTD,Total=i=1LTRSTD,i+ L-1*maxTeffect,i Where , i is the index of positioning frequency layer, L is total number of positioning frequency layers, and Teffect,i is the periodicity of PRS-RSTD measurement in positioning frequency layer i as defined further in this clause. Editor’s note: FFS the RSTD measurement period when measurement gaps and processing time T do not have overlap between different positioning frequency layers.

Based on RAN1 agreement, UE is not required to process more than one PRS layers at a time. Therefore, when measurement gaps and processing time T have overlap between different positioning frequency layers, the PRS layers are assumed to be measured sequentially, and the measurement period is defined as the sum of the measurement periods for each individual PRS layer.

During RAN4#96-e, some companies proposed that for the case when measurement gaps and processing time T do not have overlap between different positioning frequency layers, the measurement period for multiple PRS layers should be defined as the max of the measurement periods for each individual PRS layer.

In our view, the proposal makes sense technically. As shown in Figure 1, since the gap and processing time between PRS layers are non-overlapping, the measurement of one PRS layer should not impact another, which means during the measurement period of one PRS layer, another PRS layer can be measured.

Figure 1: Illustration of non-overlapping case

On the other hand, defining split requirements for overlapping case and non-overlapping case would complicate the requirements. In particular, it is not easy to define the exact condition of non-overlapping. For two PRS layers, it may be defined based on the offsets between PRS samples compared to the processing time, the case will be complex when it comes to more than 2 PRS layers.

In addition, the necessity to have separate requirements for non-overlapping case is also a bit unclear. For example, the application of the requirements depends on network configuration and UE capability, so it may not be very useful for real deployment. Also, RAN4 requirements are minimum requirements and UE is always allowed to perform better than RAN4 requirements, and as such the requirements should be defined based on worst case.

Based on above, our preference is to not define separate requirements for the non-overlapping case in Rel-16. If the need for the enhancement is justified in the real deployments, it can be discussed in future releases.

Proposal 1: RAN4 not to define separate requirements for the case when measurement gaps and processing time T do not have overlap between different positioning frequency layers in Rel-16.

Calculation of PRS sample duration L

- Calculation of PRS sample duration - Option 1: Based on the type (type 1 or type 2) as UE used to report {N,T} - Option 2: Based on Type 2. RSTD measurement period for Type 1 PRS duration calculation shall be no longer than Type 2. - Other options are not precluded

The length of the aggregate PRS occasion LPRS can be calculated in two ways as defined in clause 6.1.5.6 of 38.214, namely type 1 and type 2. Selection between the two types is up to UE and is indicated to network when UE reports the buffering and processing capability (component 2). In our view, it is more reasonable to use the UE selected type for calculating LPRS for the measurement period requirements.

In RAN4#96-e, some companies proposed to base the calculation on type 2, and the same requirements apply for both type 1 and type 2 UEs. We agree that the resulted length with type 1 will be smaller than that with type 2, so the measurement period for type 1 UE will be no longer than type 2 UE, but in this way the requirements for type 1 UE is sub-optimal and unnecessarily relaxed, and we do not see the clear benefit to use type 2 as default instead of the type UE reported.

We thus suggest to adopt option 1 for the calculation of LPRS, as it is straightforward and the specification effort is minor (just reference to 38.214).

Proposal 2: Calculation of PRS sample duration L is based the type (type 1 or type 2) UE reported.

Measurement period extension due to SSB collision

- Measurement period extension due to SSB collision - Option 1: RSTD measurement period to be defined for cases when PRS occasions are not dropped - Option 2: The RSTD measurement period can be extended to compensate for the number of PRS symbols not available at the UE due to their overlap with SSB symbols - Option 3: The same measurement period requirement shall be met, regardless of whether some of the PRS symbols are dropped or not during this measurement period

The case where PRS occasion is punctured by SSB symbols may occur if

- PRS periodicity is smaller than SSB periodicity, e.g. PRS period = 80ms, SSB period = 160ms, or

- PRS periodicity is not dividable by SSB periodicity, e.g. PRS period = 64ms, SSB period = 20ms

In our view, either case is likely to happen in real deployment. It is also difficult to define the exact puncture rate for the second case because it would depend on the relative offset between SSB and PRS. Therefore, we do not think SSB collision needs to be accounted in the measurement period. If this is not acceptable, we can add a general statement that PRS measurement period will be extended if some PRS occasions collides with SSB symbols.

Proposal 3: SSB collision is not accounted in PRS measurement period. The PRS measurement requirements apply PRS occasions are not dropped due to collision with SSB.

Measurement period when configured with PRS-RSRP

- Measurement period when configured with PRS-RSRP - Option 1: If RSTD is configured together with PRS-RSRP, then the measurement periods for both measurements are defined as: max(TRSTD,TPRS-RSRP), where TPRS-RSRP and TRSTD are the measurement periods for PRS-RSRP and RSTD, when configured without other measurements. - Option 2: RSTD measurement period is not impacted by PRS-RSRP measurement.

In our view, for DL-TDOA positioning RSTD is the main measurement that is used for positioning fix, and PRS-RSRP is assistance data that served more like quality indicator. In this sense, the RSTD measurement period should not be impacted by PRS-RSRP measurement.

Proposal 4: RSTD measurement period is not impacted by PRS-RSRP measurement.

Requirement applicability related to UE processing capability N

- Requirements when duration of a single PRS resource exceeding UE processing capability - Option 1: If the time span of a DL PRS resource instance is greater than UE reported capability N, measurement requirements do not apply for this resource - Option 2: If the capability is smaller then what is configured, then the requirements are to be relaxed.

In RAN4#96-e, it was discussed that if the time span of a DL PRS resource instance is greater than the reported buffering capability N, measurement requirements do not apply. We think the proposal makes sense, as an instance of a single PRS resource should be buffered and processed by UE with one PRS occasion/period, and coherent combining of a PRS resource across more than one PRS occasions/periods is not typical UE implementation.

In RAN4#96-e, some companies commented that the issue is already addressed by the scaling factor LPRS,iN in the measurement period. However, it is noted that the issue here is concerned with duration or time span of a single PRS resource, while the scaling of the measurement period is due to aggregate duration of a PRS sample, which includes multiple PRS resources, exceeding the reported capability N.

As example is shown in Figure 2. The duration of each single resource is no larger than N, but the aggregate duration Lprs is larger than N. In this case, the measurement requirement would apply for resource 1 and resource 3, and the scaling factor in the measurement period LPRS,iN is 2.

Another issue related to UE processing capability N is resource 2. As resource 2 is across two sampling duration of N, it cannot be measured in one PRS sample. For the same reason that an instance of a single PRS resource should be buffered and processed by UE with one PRS occasion/period, the measurement requirement cannot apply for this resource, either.

Figure 2: sampling when configured PRS occasion duration K is larger than reported capability N

Proposal 5: The measurement requirements do not apply for a PRS resource if

- the time span of the PRS resource instance is greater than UE reported capability N, or

- the PRS resource is across two sampling duration of N within duration Lprs

Conclusions

In this paper we provided our views on the remaining issues for RSTD measurement requirements.

Proposal 1: RAN4 not to define separate requirements for the case when measurement gaps and processing time T do not have overlap between different positioning frequency layers in Rel-16.

Proposal 2: Calculation of PRS sample duration L is based the type (type 1 or type 2) UE reported.

Proposal 3: SSB collision is not accounted in PRS measurement period. The PRS measurement requirements apply PRS occasions are not dropped due to collision with SSB.

Proposal 4: RSTD measurement period is not impacted by PRS-RSRP measurement.

Proposal 5: The measurement requirements do not apply for a PRS resource if

- the time span of the PRS resource instance is greater than UE reported capability N, or

- the PRS resource is across two sampling duration of N within duration Lprs

Reference

- R4-2012264, CR to TS 38.133: PRS RSTD requirements, Intel Corporation

- R4-2012283, WF on requirements for RSTD and UE Rx-Tx time difference measurement, Huawei, HiSilicon