3GPP TSG-RAN4 Meeting #97-e	R4-2015786

# Electronic Meeting, 2 – 13 November, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draft | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to introduce CSI-RSRQ accuracy requirements and report mapping |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CSIRS_L3meas-Perf |  |  |  |  |  | Date: |  |  | 2020-09-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | CSI-RSRQ accuracy and report mapping need to be defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce CSI-RSRQ accuracy and report mapping |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Performance requirements for CSI-RS measurement are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.7.2, 10.1.8.2, 10.1.9.2, 10.1.10.2, 10.1.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

10.1.7.2	Intra-frequency CSI-RSRQ accuracy requirements in FR1

10.1.7.2.1	Absolute CSI-RSRQ Accuracy in FR1

Unless otherwise specified, the requirements for absolute accuracy of CSI-RSRQ in this clause apply to a cell on the same frequency as that of the serving cell in FR1.

The accuracy requirements in Table 10.1.7.2.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each relevant CSI-RS.

-	The time difference between the reference timing for measurement and the receive timing of the target CSI-RS resource is no larger than 3us

-	The bandwidth of the CSI-RS resource is no less than 48 PRBs

-	The resource density of the CSI-RS resource in frequency domain D=3

Table 10.1.7.2.1-1: CSI-RSRQ Intra frequency absolute accuracy in FR1

| Accuracy |  |  |  |  |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition |  |  | Extreme condition |  |  | CSI-RS Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  |  |  |  |  |  | NR operating band groups Note 3 | Minimum Io |  |  |  | Maximum Io |
| dB |  |  |  |  |  | dB |  | dBm / SCS |  |  | dBm/BW Channel | dBm/BW Channel |
| SCS (kHz) |  |  | SCS (kHz) |  |  |  |  | SCS (kHz) |  |  |  |  |
| 15 | 30 | 60 | 15 | 30 | 60 |  |  | 15 | 30 | 60 |  |  |
|  |  |  |  |  |  | -3 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | N/A | -70 |
|  |  |  |  |  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -70 |
|  |  |  |  |  |  |  | NR_TDD_FR1_C | -120 | -117 | -114 | N/A | -70 |
|  |  |  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 | N/A | -70 |
|  |  |  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -113 | N/A | -70 |
|  |  |  |  |  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | -112.5 | N/A | -70 |
|  |  |  |  |  |  |  | NR_FDD_FR1_G | -118 | -115 | -112 | N/A | -70 |
|  |  |  |  |  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | -111.5 | N/A | -70 |
|  |  |  |  |  |  | -6 | Note 2 | Note 2 | Note 2 | Note 2 | N/A | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |  |  |  |  |

<End of Change 1>

<Start of Change 2>

10.1.8.2	Intra-frequency CSI-RSRQ accuracy requirements in FR2

10.1.8.2.1	Absolute CSI-RSRQ Accuracy in FR2

Unless otherwise specified, the requirements for absolute accuracy of CSI-RSRQ in this clause apply to a cell on the same frequency as that of the serving cell in FR2.

The accuracy requirements in Table 10.1.8.2.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each relevant CSI-RS.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

-	The time difference between the reference timing for measurement and the receive timing of the target CSI-RS resource is no larger than 3us

-	The bandwidth of the CSI-RS resource is no less than 48 PRBs

-	The resource density of the CSI-RS resource in frequency domain D=3

Table 10.1.8.2.1-1: CSI-RSRQ Intra frequency absolute accuracy in FR2

| Accuracy |  |  |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition |  | Extreme condition |  | CSI-RS Ês/Iot Note 4 | Io Note 2 range |  |  |
|  |  |  |  |  | Minimum Io |  | Maximum Io |
| dB |  |  |  | dB | dBm / SCS Note 1 |  | dBm/BWChannel |
| SCS (kHz) |  | SCS (kHz) |  |  | SCS (kHz) |  |  |
| 60 | 120 | 60 | 120 |  | 60 | 120 |  |
|  |  |  |  | ≥-3 | Same value as CSI-RS_RP in Table B.2.x-2, according to UE Power class, operating band and angle of arrival |  | -50 |
|  |  |  |  | ≥-6 |  |  |  |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. Note 3: In the test cases, the CSI-RS Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |  |

<End of Change 2>

<Start of Change 3>

10.1.9.2	Inter-frequency CSI-RSRQ accuracy requirements in FR1

10.1.9.2.1	Aboslute Accuracy of CSI-RSRQ in FR1

The requirements for absolute accuracy of CSI-RSRQ in this clause apply to a cell on a frequency in FR1 that has different carrier frequency from the serving cell.

The accuracy requirements in Table 10.1.9.2.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for each relevant CSI-RS.

-	The time difference between the reference timing for measurement and the receive timing of the target CSI-RS resource is no larger than 3us

-	The bandwidth of the CSI-RS resource is no less than 48 PRBs

-	The resource density of the CSI-RS resource in frequency domain D=3

Table 10.1.9.2.1-1: CSI-RSRQ Inter frequency absolute accuracy in FR1

| Accuracy |  |  |  |  |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition |  |  | Extreme condition |  |  | CSI-RS Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  |  |  |  |  |  | NR operating band groups Note 3 | Minimum Io |  |  |  | Maximum Io |
| dB |  |  |  |  |  | dB |  | dBm / SCS |  |  | dBm/BW Channel | dBm/BW Channel |
| SCS (kHz) |  |  | SCS (kHz) |  |  |  |  | SCS (kHz) |  |  |  |  |
| 15 | 30 | 60 | 15 | 30 | 60 |  |  | 15 | 30 | 60 |  |  |
|  |  |  |  |  |  | -3 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | N/A | -70 |
|  |  |  |  |  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -70 |
|  |  |  |  |  |  |  | NR_TDD_FR1_C | -120 | -117 | -114 | N/A | -70 |
|  |  |  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 | N/A | -70 |
|  |  |  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -113 | N/A | -70 |
|  |  |  |  |  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | -112.5 | N/A | -70 |
|  |  |  |  |  |  |  | NR_FDD_FR1_G | -118 | -115 | -112 | N/A | -70 |
|  |  |  |  |  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | -111.5 | N/A | -70 |
|  |  |  |  |  |  | -6 | Note 2 | Note 2 | Note 2 | Note 2 | N/A | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |  |  |  |  |

10.1.9.2.2	Relative Accuracy of CSI-RSRQ in FR1

The relative accuracy of CSI-RSRQ in inter frequency case is defined as the RSRQ measured from one cell on a frequency in FR1 compared to the RSRQ measured from another cell on a different frequency in FR1.

The accuracy requirements in Table 10.1.9.2.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for each relevant CSI-RS.

-	|CSI-RS_RP1dBm - CSI-RS_RP2dBm| ≤ 27 dB

-	| Channel 1_Io -Channel 2_Io |  20 dB

-	The time difference between the reference timing for measurement and the receive timing of the target CSI-RS resource is no larger than 3us

-	The bandwidth of the CSI-RS resource is no less than 48 PRBs

-	The resource density of the CSI-RS resource in frequency domain D=3

Table 10.1.9.2.2-1: CSI-RSRQ Inter frequency relative accuracy in FR1

| Accuracy |  |  |  |  |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition |  |  | Extreme condition |  |  | CSI-RS Ês/Iot Note 4 | Io Note 1 range |  |  |  |  |  |
|  |  |  |  |  |  |  | NR operating band groups Note 3 | Minimum Io |  |  |  | Maximum Io |
| dB |  |  |  |  |  | dB |  | dBm / SCS |  |  | dBm/BW Channel | dBm/BW Channel |
| SCS (kHz) |  |  | SCS (kHz) |  |  |  |  | SCS (kHz) |  |  |  |  |
| 15 | 30 | 60 | 15 | 30 | 60 |  |  | 15 | 30 | 60 |  |  |
|  |  |  |  |  |  | -3 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | N/A | -70 |
|  |  |  |  |  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -70 |
|  |  |  |  |  |  |  | NR_TDD_FR1_C | -120 | -117 | -114 | N/A | -70 |
|  |  |  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 | N/A | -70 |
|  |  |  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -113 | N/A | -70 |
|  |  |  |  |  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | -112.5 | N/A | -70 |
|  |  |  |  |  |  |  | NR_FDD_FR1_G | -118 | -115 | -112 | N/A | -70 |
|  |  |  |  |  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | -111.5 | N/A | -70 |
|  |  |  |  |  |  | -6 | Note 2 | Note 2 | Note 2 | Note 2 | N/A | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 4: The parameter CSI-RS Ês/Iot is the minimum CSI-RS Ês/Iot of the pair of cells to which the requirement applies. |  |  |  |  |  |  |  |  |  |  |  |  |

<End of Change 3>

<Start of Change 4>

#### 10.1.10.2	Inter-frequency CSI-RSRQ accuracy requirements in FR2

##### 10.1.10.2.1	Aboslute Accuracy of CSI-RSRQ in FR2

The requirements for absolute accuracy of CSI-RSRQ in this clause apply to a cell on a frequency in FR2 that has different carrier frequency from the serving cell.

The accuracy requirements in Table 10.1.10.2.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for each relevant CSI-RS.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

-	The time difference between the reference timing for measurement and the receive timing of the target CSI-RS resource is no larger than 3us

-	The bandwidth of the CSI-RS resource is no less than 48 PRBs

-	The resource density of the CSI-RS resource in frequency domain D=3

Table 10.1.10.2.1-1: CSI-RSRQ Inter frequency absolute accuracy in FR2

| Accuracy |  |  |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition |  | Extreme condition |  | CSI-RS Ês/Iot | Io Note 2 range |  |  |
|  |  |  |  |  | Minimum Io |  | Maximum Io |
| dB |  |  |  | dB | dBm / SCS Note 1 |  | dBm/BWChannel |
| SCS (kHz) |  | SCS (kHz) |  |  | SCS (kHz) |  |  |
| 60 | 120 | 60 | 120 |  | 60 | 120 |  |
|  |  |  |  | ≥-3 | Same value as CSI-RS_RP in Table B.2.x-2, according to UE Power class, operating band and angle of arrival |  | -50 |
|  |  |  |  | ≥-4 |  |  |  |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. Note 3: In the test cases, the CSI-RS Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |  |

##### 10.1.10.2.2	Relative Accuracy of CSI-RSRQ in FR2

The relative accuracy of CSI-RSRQ in inter frequency case is defined as the RSRQ measured from one cell on a frequency in FR2 compared to the RSRQ measured from another cell on a different frequency in FR2.

The accuracy requirements in Table 10.1.10.2.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for each relevant CSI-RS.

-	|CSI-RS_RP1dBm - CSI-RS_RP2dBm|  27 dB

-	| Channel 1_Io -Channel 2_Io |  20 dB

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

-	The time difference between the reference timing for measurement and the receive timing of the target CSI-RS resource is no larger than 3us

-	The bandwidth of the CSI-RS resource is no less than 48 PRBs

-	The resource density of the CSI-RS resource in frequency domain D=3

Table 10.1.10.2.2-1: CSI-RSRQ Inter frequency relative accuracy in FR2

| Accuracy |  |  |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition |  | Extreme condition |  | CSI-RS Ês/Iot Note 4 | Io Note 2 range |  |  |
|  |  |  |  |  | Minimum Io |  | Maximum Io |
| dB |  |  |  | dB | dBm / SCS Note 1 |  | dBm/BWChannel |
| SCS (kHz) |  | SCS (kHz) |  |  | SCS (kHz) |  |  |
| 60 | 120 | 60 | 120 |  | 60 | 120 |  |
|  |  |  |  | ≥-3 | Same value as CSI-RS_RP in Table B.2.x-2, according to UE Power class, operating band and angle of arrival |  | -50 |
|  |  |  |  | ≥-4 |  |  |  |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. Note 3: In the test cases, the CSI-RS Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. Note 4: The parameter CSI-RS Ês/Iot is the minimum CSI-RS Ês/Iot of the pair of cells to which the requirement applies. |  |  |  |  |  |  |  |

<End of Change 4>

<Start of Change 5>

10.1.11	RSRQ report mapping

#### 10.1.11.1	SS-RSRQ measurement report mapping

The reporting range of SS-RSRQ is defined from -43 dB to 20 dB with 0.5 dB resolution. The reporting range of CSI-RSRQ is defined from -43 dB to 0 dB with 0.5 dB resolution. The mapping of measured quantity is defined in Table 10.1.11.1-1. The range in the signalling may be larger than the guaranteed accuracy range.

Table 10.1.11.1-1: SS-RSRQ and CSI-RSRQ measurement report mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| RSRQ_0 | RSRQ<-43 | dB |
| RSRQ_1 | -43≤RSRQ<-42.5 | dB |
| RSRQ_2 | -42.5≤RSRQ<-42 | dB |
| RSRQ_3 | -42≤RSRQ<-41.5 | dB |
| RSRQ_4 | -41.5≤RSRQ<-41 | dB |
| .. | .. | … |
| RSRQ_86 | -0.5≤RSRQ<0 |  |
| RSRQ_87 | 0≤RSRQ<0.5 |  |
| RSRQ_88 | 0.5≤RSRQ<1 |  |
| .. | .. | … |
| RSRQ_122 | 17.5≤RSRQ<18 | dB |
| RSRQ_123 | 18≤RSRQ<18.5 | dB |
| RSRQ_124 | 18.5≤RSRQ<19 | dB |
| RSRQ_125 | 19≤RSRQ<19.5 | dB |
| RSRQ_126 | 19.5≤RSRQ<20 | dB |
| RSRQ_127 | 20 ≤RSRQ | dB |
| Note 1: The values from RSRQ_88 to RSRQ_127 are not applicable for CSI-RSRQ. |  |  |

<End of Change 5>