3GPP TSG-RAN WG4 Meeting #97-e	R4-2015815

Electronic Meeting, 2 – 13 November 2020

Agenda Item:	7.5.4

Source:	Ericsson

Title:	UE demodulation requirements for WI on MR-DC and CA enhancements

Document for:	Approval

# 1	Introduction

3GPP RAN plenary #89-e agreed that the core part of WI Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements (LTE, NR) has completed [1][2]. This contribution discusses the impacts to UE demodulation and CSI reporting requirements due to this WI according to the WI summary [3].

# 2	Discussion

This section reviews the introduced functionalities with this WI and shows our view on the impact to UE demodulation and CSI reporting requirements.

Idle/inactive measurement reporting: Allows the network to assign UE to do measurements during IDLE or INACTIVE that the network can use for when the UE enters CONNECTED mode.

This functionality does not affect to UE demodulation because these functionalities are applicable for UEs in IDLE/INACTIVE states.

Direct SCG/SCell configuration: Allows the network to configure the UE to store the SCG/SCell configuration upon transition to INACTIVE state, so that it can be quickly restored upon transition back to CONNECTED.

This functionality does not affect to UE demodulation because these functionalities are applicable for UEs in INACTIVE state.

Dormant NR SCell: Introduces SCell dormancy. If configured, UE stops monitoring PDCCH on the dormant SCell, but continues performing RRM/CSI measurements, AGC and beam management.

This feature is discussed in RRM part, and it does not affect to UE demodulation performance.

Fast MCG link recovery: Supports fast recovery from MCG link failure by allowing the UE in MR-DC to send an MCG Failure Information message to the MN via the SCG upon the detection of a radio link failure on the MCG. Based on measurement information received in MCG Failure Information message the network can then determine the correct action to restore the MCG connection, e.g. change of PCell.

This functionality is the improvement of the procedure, and it does not affect to UE demodulation requirements.

Cross-carrier scheduling with different numerologies: Supports the cross-carrier scheduling in carrier aggregation, with different numerology on the scheduling and scheduled carriers.

This functionality is related to procedure and we don’t think it affects to the PDSCH CA demodulation performance discussed in RAN4 Rel-16 WI NR performance requirement enhancement.

Aperiodic CSI-RS triggering with different numerologies: Supports triggering Aperiodic CSI-RS with different numerology between CSI-RS and triggering PDCCH.

This functionality is related to procedure and we don’t think it affects to the CA CQI reporting tests discussed in RAN4 Rel-16 WI NR performance requirement enhancement.

CA with unaligned frame boundary: Supports the NR inter-band carrier aggregation with slot alignment, but with unaligned frame boundary and partial SFN alignment.

This functionality does not affect to the PDSCH CA demodulation performance discussed in RAN4 Rel-16 WI NR performance requirement enhancement.

Enhancements to single Tx switched uplink for EN-DC: This functionality does not affect to UE demodulation requirements because it is the UL-related improvements.

Support for asynchronous NR-DC: Allows the non-co-located deployments by relaxing synchronization requirements for gNBs involved in NR-DC operation.

RAN4 has defined PDSCH demodulation requirements for NR-DC for interworking in TS38.101-4 Clause 9. However it is specified for NR-DC between FR1 and FR2, and verifies FR2 cell only. This means the asynchronous NR-DC does not affect to the test case.

It is also our understanding the performance part of this WI does not intend to introduce new PDSCH demodulation requirements for NR-DC, such as new PDSCH CA demodulation requirements discussed in RAN4 Rel-16 WI NR performance requirement enhancement. According to our observation, we don’t think this WI requires to define new PDSCH demodulation or CSI reporting requirements. We therefore propose no new PDSCH demodulation and CSI reporting requirements are defined due to the WI on MR-DC and CA enhancements.

Proposal: No UE demodulation and CSI reporting requirements are defined due to WI on MR-DC and CA enhancements.

# 3	Summary

Proposal: No UE demodulation and CSI reporting requirements are defined due to WI on MR-DC and CA enhancements.

# 4	References

- RP-200791, “Revised WID: Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements”, Ericsson.

- RP-201532, “Status report for Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements (LTE, NR)”, Ericsson.

- RP-201531, “Summary of WI on Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements (LTE, NR)”, Ericsson.