3GPP TSG-RAN WG4 Meeting #97-e	R4-2015818

Electronic Meeting, 2 – 13 November 2020

Agenda Item:	7.1.6.9

Source:	Ericsson

Title:	Open issues on beam management for NR-U

Document for:	Discussion

# 1	Introduction

This contribution discusses the remining open issues for NR-U beam management according to WF [1].

Measurement period for L1-RSRP - RAN4 should wait for LS response from RAN1 on the UE behavior when UE cannot transmit HARQ-ACK for MAC CE deactivation for semi-persistent CSI reporting. RAN4 should continue the discussion in the RRM maintenance in case RAN4 does not receive the LS response from RAN1 during RAN4#96-e

# 2	Discussion

## 2.1	Introduction of new clause for L1-RSRP reporting under CCA in TS38.133

RAN4#96-e agreed CR to introduce L1-RSRP reporting under CCA in TS38.133 [2], where the CR adds only subclause 9.5.4A based on 9.5.4, which specifies the L1-RSRP measurement requirements. However we have identified 9.5.4A is not enough to specify the L1-RSRP reporting under CCA. For example, if RAN4 introduce new clause for L1-RSRP measurement accuracy requirements under CCA in clause 10.1, it should be referred by clauses 9.5.2 and 9.5.3. Moreover, if we look 9.5.5 and 9.5.6, it specifies the measurement restriction and scheduling availability considering SSB and CSI-RS based measurement in FR1 and FR2. However RAN4 has already agreed that the measurements resources for L1-RSRP in NR-U is SSB in FR1 only for Rel-16. In order to avoid the confusion, we propose to introduce new clause 9.5A in TS38.133 for L1-RSRP reporting under CCA. See [3] for the detailed updates. According to the CR guideline, the existing 9.5.4A should be set to ‘Void’.

Proposal 1: Introduce new clause 9.5A in TS38.133 for L1-RSRP reporting under CCA.

## 2.2	UE behavior when UE cannot transmit HARQ-ACK for MAC-CE deactivation of semi-persistent CSI reporting

The remaining open issue on the L1-RSRP measurements is the UE behavior when UE cannot transmit HARQ-ACK for MAC-CE deactivation of semi-persistent CSI reporting. On this issue RAN4 has sent LS to RAN1 [4] and RAN1 is discussing the UE/gNB behavior when UE cannot transmit HARQ-ACK for MAC CE deactivation for semi-persistence CSI reporting.

Once RAN4 receives the LS response from RAN1 during RAN4#97-e, RAN4 should restart the discussion and capture the UE behavior in TS38.133 if necessary.

Proposal 2: RAN4 should wait for LS response from RAN1 on the UE behavior when UE cannot transmit HARQ-ACK for MAC CE deactivation for semi-persistent CSI reporting. Once RAN4 receives the LS response from RAN1, RAN4 should restart the discussion and capture the UE behavior in TS38.133 if necessary.

# 4	Summary

Proposal 1: Introduce new clause 9.5A in TS38.133 for L1-RSRP reporting under CCA.

Proposal 2: RAN4 should wait for LS response from RAN1 on the UE behavior when UE cannot transmit HARQ-ACK for MAC CE deactivation for semi-persistent CSI reporting. Once RAN4 receives the LS response from RAN1, RAN4 should restart the discussion and capture the UE behavior in TS38.133 if necessary.

# References

- R4-2012274, “WF on NR-U RRM requirements – Part 2”, RAN4#96-e.

- R4-2008580, “CR: Introduction of L1-RSRP measurements with CCA”, Ericsson.

- R4-2015819, “CR: Beam management requirements with CCA”, Ericsson.

- R4-2008576, “Clarification of UE behavior after receiving the MAC CE deactivation command for semi-persistent CSI reporting in NR-U”, Nokia.