3GPP TSG-RAN WG4 #97-e	R4-2015867

Electronic meeting, 2nd – 13th November 2020

Source: 	Ericsson

Title:  	Base station ultra-low BLER results

Agenda Item:	7.8.1.1.3

Document for:	Information

# Results

During RAN4#94-e, simulation parameters for the BS ultra-low BLER requirement were resolved ‎[1]. We present proposed SNR levels at 10-5 BLER as following:

| Scenario | Ideal (dB) | Impairment (dB) |
| --- | --- | --- |
| 15kHz SCS, 5MHz, Mapping type A | -7.90 | -5.4 |
| 15kHz SCS, 5MHz, Mapping type B | -7.90 | -5.4 |
| 15kHz SCS, 10MHz, Mapping type A | -8.10 | -5.6 |
| 15kHz SCS, 10MHz, Mapping type B | -8.10 | -5.6 |
| 30kHz SCS, 10MHz, Mapping type A | -7.90 | -5.4 |
| 30kHz SCS, 10MHz, Mapping type B | -7.90 | -5.4 |
| 30kHz SCS, 40MHz, Mapping type A | -8.70 | -6.2 |
| 30kHz SCS, 40MHz, Mapping type B | -8.70 | -6.2 |

# Simulation assumptions

| Parameter |  | Value |
| --- | --- | --- |
| Frequency range |  | FR1 |
| Transform precoding |  | Disabled |
| Antenna configuration |  | 1x2, ULA low |
| PUSCH configuration | Mapping type | Type A and Type B |
|  | Starting symbol (S) | 0 |
|  | Length (L) | 14 |
|  | PUSCH aggregation factor | 1 |
| PUSCH DMRS configuration | DMRS Type | Type 1 |
|  | DM-RS duration | Single-symbol DM-RS |
|  | Number of additional DMRS | 1 |
| Number of HARQ Transmissions |  | 1 |
| PT-RS |  | Disabled |
| Propagation condition |  | AWGN |
| MCS Table |  | Table 3, MCS 5 |
| SCS and BW |  | 15 kHz with BW 5 and10 MHz 30 kHz with BW 10 and 40 MHz |
| Frequency domain resource |  | Full Bandwidth |
| TDD pattern |  | 15 kHz SCS: 3D1S1U, S=10:2:2 30 kHz SCS: 7D1S2U, S=6:4:4 |
| Testing metric |  | Target BLER: 10-5 |

The new proposed FRC parameters for FR1 PUSCH performance requirements, transform precoding disabled, Additional DM-RS position = pos1 and 1 transmission layer (QPSK, R=99/1024) are presented in the table below

| Subcarrier spacing (kHz) | 15 | 15 | 30 | 30 |
| --- | --- | --- | --- | --- |
| Allocated resource blocks | 25 | 52 | 24 | 106 |
| CP-OFDM Symbols per slot (Note 1) | 12 | 12 | 12 | 12 |
| Modulation | QPSK | QPSK | QPSK | QPSK |
| Code rate (Note 2) | 99/1024 | 99/1024 | 99/1024 | 99/1024 |
| Payload size (bits) | 704 | 1480 | 672 | 1480 |
| Transport block CRC (bits) | 16 | 16 | 16 | 16 |
| Code block CRC size (bits) | - | - | - | - |
| Number of code blocks - C | 1 | 1 | 1 | 1 |
| Code block size including CRC (bits) (Note 2) | 720 | 1496 | 688 | 1496 |
| Total number of bits per slot | 7200 | 14976 | 6912 | 30528 |
| Total symbols per slot | 3600 | 7488 | 3456 | 15264 |