3GPP TSG-RAN WG4 Meeting#97-e                	R4-2015884

E-meeting, November 2nd – 11th, 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on test cases for MD-DC EMR

Agenda item:	7.5.3.2

Document for:	Discussion

Introduction

Performance work is to start for the LTE_NR_DC_CA_RRM work and the part related to measurement for early measurement reporting. For this purpose, RAN4 need to agree on list of test cases for the feature. In this paper we list our view on the test cases needed.

Discussion

For a number of meetings, RAN4 has discussed the core requirements which were officially closed in last plenary. Although some core requirement aspects are still open and needs finalization, RAN4 need to begin the discussion related to test cases for early measurement reporting.

In this paper we give our view on the test cases that we see would be needed for the feature. Covering both new core requirements defined in 36.133 and 38.133.

Detected cell requirement during state transition and Idle mode

RAN4 defined new core requirements for detected cell requirement during state transition and Idle mode. Concerning test cases for detected cell requirement during state transition and Idle mode, we list following:

38.133: 3 different test cases. 2 covering inter-frequency and 1 for covering LTE inter-RAT cell.

- Detected cell status: 1 test case with release followed by connection setup immediately after release. Change of EMR cell Rxlevel before reporting. non-Overlapping carrier, FR1.

- Detected cell status: 1 test case with release followed by connection setup immediately after release. Change of EMR cell Rxlevel before reporting. Overlapping carrier, FR2.

- Detected cell status: 1 test case with release followed by connection setup immediately after release. Change of EMR cell Rxlevel before reporting. LTE Overlapping carrier

| Carrier | Frequency Range | Carrier type |
| --- | --- | --- |
| inter-frequency | FR1 | non-overlapping |
| inter-frequency | FR2 | overlapping |
| inter-RAT | N/A | overlapping |

36.133: For LTE we suggest 2 new test cases. Both covering NR inter-RAT cell of which one is overlapping in FR1 and another is non-overlapping in FR2.

- Detected cell status: 1 test case with release followed by connection setup immediately after release. Change of EMR cell Rxlevel before reporting. Overlapping carrier, FR1.

- Detected cell status: 1 test case with release followed by connection setup immediately after release. Change of EMR cell Rxlevel before reporting. non-Overlapping carrier, FR2.

| Carrier | Frequency Range | Carrier type |
| --- | --- | --- |
| inter-RAT | FR1 | overlapping |
| inter-RAT | FR2 | non-overlapping |

Measurements of inter-frequency CA/DC candidate cells

Similarly, RAN4 have defined new core requirements for measurements of inter-frequency CA/DC candidate cells. Concerning test cases for measurements of inter-frequency CA/DC candidate cells, we list following:

38.133: We suggest following test cases, based on the new core requirements for measurements and reporting of inter-frequency and RAT overlapping and non-overlapping carriers with and without beam level measurements:

- Measurements in idle mode, inter-frequency, FR1: 1 test case with release followed by connection setup, overlapping inter-frequency carrier, new cell, connection setup after time enough to perform cell detection and measurements.

- Measurements in idle mode, inter-frequency, FR1: 1 test case with release followed by connection setup, non-overlapping inter-frequency carrier, known cell, connection setup after a short time (good EMR cell conditions).

- Measurements in idle mode, inter-frequency, FR2: 1 test case with release followed by connection setup, overlapping inter-frequency carrier, known cell, connection setup after time enough to perform cell measurements (excluding beam level measurements).

- Measurements in idle mode, inter-frequency, FR2: 1 test case with release followed by connection setup, non-overlapping inter-frequency carrier, new cell, connection setup after a short time (good EMR cell conditions) (excluding beam level measurements).

- Measurements in idle mode, inter-frequency, FR2: 1 test case with release followed by connection setup, overlapping inter-frequency carrier, new cell, connection setup after time enough to perform cell detection, excluding beam level measurements and measurements.

- Measurements in idle mode, inter-frequency, FR2: 1 test case with release followed by connection setup, non-overlapping inter-frequency carrier, new cell, connection setup after a short time (good EMR cell conditions).

| Carrier | Frequency Range | Carrier type | Cell type | SSB Measurements |
| --- | --- | --- | --- | --- |
| inter-frequency | FR1 | overlapping | new cell | Yes |
| inter-frequency | FR1 | non-overlapping | known cell | No |
| inter-frequency | FR2 | overlapping | known cell | No |
| inter-frequency | FR2 | non-overlapping | known cell | Yes |
| inter-frequency | FR2 | overlapping | new cell | Yes |
| inter-frequency | FR2 | non-overlapping | new cell | No |

Measurements of E-UTRAN inter-RAT DC candidate cells

Based on the defined core requirements we suggest 2 test cases for this setup.

38.133: Test cases for Measurements of E-UTRAN inter-RAT DC candidate cells:

- Measurements in idle mode, inter-RAT: 1 test case with release followed by connection setup, overlapping inter-frequency carrier, known cell, connection setup after time enough to perform cell detection and measurements.

- Measurements in idle mode, inter-RAT: 1 test case with release followed by connection setup, non-overlapping inter-frequency carrier, new cell, connection setup after a short time (good EMR cell conditions).

| Carrier | Frequency Range | Carrier type | Cell type | SSB Measurements |
| --- | --- | --- | --- | --- |
| Inter-RAT | LTE | overlapping | known cell | N/A |
| Inter-RAT | LTE | non-overlapping | new cell | N/A |

Measurements of NR inter-RAT DC candidate cells

In LTE RAN4 defined new NR inter-RAT EMR core requirements in Rel-16. Hence, these test cses should be defined in 36.166 and cover test cases for Measurements of NR inter-RAT DC candidate cells.

36.133 we suggest following test cases based on the new core requirements for measurements and reporting of inter-RAT overlapping and non-overlapping NR carriers with and without beam level measurements:

- Measurements in idle mode, inter-RAT, FR1: 1 test case with release followed by connection setup, overlapping inter- RAT carrier, new cell, connection setup after time enough to perform cell detection and measurements.

- Measurements in idle mode, inter- RAT, FR1: 1 test case with release followed by connection setup, non-overlapping inter- RAT carrier, known cell, connection setup after a short time (good EMR cell conditions).

- Measurements in idle mode, inter- RAT, FR2: 1 test case with release followed by connection setup, overlapping inter- RAT carrier, known cell, connection setup after time enough to perform cell detection and measurements (no beam level measurements).

- Measurements in idle mode, inter- RAT, FR2: 1 test case with release followed by connection setup, non-overlapping inter- RAT carrier, new cell, connection setup after a short time (good EMR cell conditions) (no beam level measurements).

- Measurements in idle mode, inter- RAT, FR2: 1 test case with release followed by connection setup, overlapping inter- RAT carrier, new cell, connection setup after time enough to perform cell detection, beam level measurements and measurements.

- Measurements in idle mode, inter- RAT, FR2: 1 test case with release followed by connection setup, non-overlapping inter- RAT carrier, new cell, connection setup after a short time (good EMR cell conditions).

| Carrier | Frequency Range | Carrier type | Cell type | SSB Measurements |
| --- | --- | --- | --- | --- |
| Inter-RAT | FR1 | non-overlapping | new cell | Yes |
| Inter-RAT | FR1 | overlapping | known cell | No |
| Inter-RAT | FR2 | non-overlapping | known cell | No |
| Inter-RAT | FR2 | overlapping | known cell | Yes |
| Inter-RAT | FR2 | non-overlapping | new cell | Yes |
| Inter-RAT | FR2 | overlapping | new cell | No |

Based on the proposed list of test cases for the core requirements for MR-DC EMR we propose:

RAN4 to discuss and agree on a list of test cases for MR-DC EMR.

Additionally, RAN4 would need to introduce measurement performance test cases.

RAN4 introduces measurement performance test cases.

Conclusion

The performance work is to start for the LTE_NR_DC_CA_RRM work and the part related to measurement for early measurement reporting. In this paper we have listed our view on the test cases needed. It is proposed:

- RAN4 to discuss and agree on a list of test cases for MR-DC EMR.

- RAN4 introduces measurement performance test cases.

References