3GPP TGS-RAN WG4 Meeting #97e	R4-2015887
Electronic Meeting, November 2 – 13, 2020

Agenda item:	10.24.1

Source:	Intel Corporation

Title:	Views on UE RF requirements of new FWA with 23dBm max TRP

Document for:	Approval

# 1	Introduction

Discussions on the Rel-17 WI for a new FWA use case with stricter regulations have been ongoing in RAN4 and led to the following agreements for the RF requirement scope [1-2]:

Since then, discussions in RAN4 #95e and RAN4 #96e mainly focused on two things: the minimum peak EIRP and EIS requirements, and how the requirements will be introduced into the specifications [3-4]. For the budget evaluations needed to derive peak EIRP/EIS, the approved WF captured the following options for further discussion [5]:

While the above options were discussed during RAN4 #96e, no agreements were reached [4]. This left several open issues to address, including how to determine appropriate values for the minimum peak EIRP and minimum peak EIS requirements. In this paper we present our views on how we can define these two requirements.

# 2	Discussion

In the past several meetings, we have discussed the minimum peak EIRP and EIS requirement derivations. Overall, taking an average of the proposed values received the most support [4]. However, companies were split on the number of antenna elements to focus on. To examine how we may address this, we will first discuss the minimum peak EIRP requirement using a data driven approach. For reference, the table below summarizes the proposals thus far for this requirement.

Minimum peak EIRP

Table 1. Summary of minimum peak EIRP proposals

From Table 1 we see that even though we have three values based on 16-elements, only one value exceeds 30dBm. In fact, taking the average of all proposed values yields 28.83dBm (or 29dBm for n258, and 28.9dBm for n257). This average is very close to two of the numbers based on 16-elements. While our preference is still to define the minimum peak EIRP as 28.3dBm (as captured in [6]), we think using the average of all proposed values is a reasonable compromise for this requirement. At ~29 dBm, the average value is already significantly higher than the PC3 requirement (over 6dB increase). Furthermore, we note that the number of antenna elements is an implementation choice and using the overall average ensures neither implementation is precluded.

Observation 1: Based on the proposed data in Table 1, it is reasonable to use the average of all proposed values as a compromise for the minimum peak EIRP requirement. At ~29 dBm, the average value is over 6dB higher than the PC3 requirement.

Proposal 1: As a compromise, use 29dBm (average of all proposed values) for the n257/n258 minimum peak EIRP requirement of the new FWA with 23dBm max TRP.

Minimum peak EIS

Table 2. Summary of minimum peak EIS proposals

Like Table 1 in the preceding section, Table 2 summarizes the proposals for the minimum peak EIS requirement. Upon inspection we see the range in values proposed is less than 3.5dB. Averaging all the proposals listed leads to -92.6dBm. Once again, this value represents a reasonable compromise for the requirement

Proposal 2: As a compromise, use -92.6dBm (average of all proposed values) for the n257/n258 minimum peak EIS requirement of the new FWA with 23dBm max TRP.

Multi-band relaxation (MBR)

In addition to the two RF requirements discussed in the previous sections, multi-band relaxation is also an open issue. During the last RAN4 meeting, options for a 0.5dB relaxation and 0.7dB relaxation received the most support [4]. While we think both options are reasonable, we prefer to first agree on the minimum peak EIRP and EIS requirements before making this decision.

Observation 2: Both 0.5dB and 0.7dB are reasonable MBR options for the new FWA with 23dBm max TRP. However, we should first agree on the minimum peak EIRP and EIS requirements before choosing the relaxation value.

# 3	Conclusions

In this paper we discussed the proposals for the minimum peak EIRP and EIS requirements of the new FWA use case, and how we may reach a compromise on their definition. The following observations and proposals have been made:

Observation 1: Based on the proposed data in Table 1, it is reasonable to use the average of all proposed values as a compromise for the minimum peak EIRP requirement. At ~29 dBm, the average value is over 6dB higher than the PC3 requirement.

Proposal 1: As a compromise, use 29dBm (average of all proposed values) for the n257/n258 minimum peak EIRP requirement of the new FWA with 23dBm max TRP.

Proposal 2: As a compromise, use -92.6dBm (average of all proposed values) for the n257/n258 minimum peak EIS requirement of the new FWA with 23dBm max TRP.

Observation 2: Both 0.5dB and 0.7dB are reasonable MBR options for the new FWA with 23dBm max TRP. However, we should first agree on the minimum peak EIRP and EIS requirements before choosing the relaxation value.

# 4	References

- R4-2005713, “Email discussion summary for [94e Bis][32]NR_FR2_FWA_Bn257_Bn258, SoftBank, RAN4 #94e-Bis, April 2020

- R4-2005177, “WF on FR2 new FWA UE RF and RRM requirements,” Huawei, HiSilicon, SoftBank, RAN4 #94e-Bis, April 2020

- R4-2008964, “Email discussion summary for [95e][136] NR_FR2_FWA_Bn257_Bn258,” Softbank, RAN4 #95e, May 2020

- R4-2011870, “Email discussion summary for [96e][132] NR_FR2_FWA_Bn257_Bn258,” Softbank, RAN4 #96e, August 2020

- R4-2008921, “WF on FR2 FWA evaluations,” Qualcomm, RAN4 #95e, May 2020

- R4-2007110, “UE RF requirements for new FR2 FWA use case,” Intel, RAN4 #95e, May 2020