3GPP TGS-RAN WG4 Meeting #97e	R4-2015888
Electronic Meeting, November 2 – 13, 2020

Agenda item:	10.28.1

Source:	Intel Corporation

Title:	PC3 minimum peak EIRP and EIS requirements for band n262

Document for:	Discussion

# 1	Introduction

While FR2 extends from 24.25 to 52.6 GHz, current requirements are only defined up to 43.5 GHz for band n259 [1-2]. To ensure the recently auctioned 47 GHz band is captured in the specifications, a work item aiming to define requirements for the 47.2 to 48.2 GHz frequency range was approved during RAN #88e [3].

Discussions for the work item started in the RAN4 #96e meeting [4]. Three separate way forwards were approved, each addressing the following: UE RF requirements [5], testability considerations [6] and link budget parameters [7]. The latter urged companies to share their views on the link budget parameters needed to derive the minimum peak EIRP and EIS requirements [7].

  - To define the maximum output power and Refsens OTA requirements for n262, companies are encouraged to provide the link budget parameters for Tx and Rx for all power classes 1/2/3/4 in the next meeting according to the tables in the WF.

  - PC3 is considered as the highest priority

Additionally, the link budget WF captured [7] two tables with relevant budget parameters to facilitate the analysis and discussion of the minimum peak EIRP and EIS requirements.

In this paper, we discuss the budget calculations and provide derivations of the minimum peak EIRP and EIS requirements for the new FR2 band n262.

# 2	Discussion

Two performance items will be addressed in the sections below: the single-band minimum peak EIRP and minimum peak EIS requirements. To derive these budgets, we will first discuss relevant performance challenges for band n262.

## 2.1	UE RF requirements

Given that priority has been given to PC3 and this one is the most space-limited form factor, we will continue to use 4-elements in the budget derivations. Additionally, we need to consider the impact moving up 5 GHz in frequency (compared to band n259) will have on the design performance and losses. Overall, a lower minimum peak EIRP is expected. This stems from a more challenging design, a reduction in available device gain at higher frequencies, and higher losses from a frequency range that is more susceptible to material and integration losses. These differences are captured in the link budget parameters of the upcoming sections.

### 2.1.1 Minimum peak EIRP

Table 1 lists the parameters needed to derive the minimum peak EIRP. Compared to band n259, and as mentioned in the preceding section, the derived minimum peak EIRP value is lower. The end result is a minimum peak EIRP of 17dBm, which corresponds to a 1.7dB decrease compared to the PC3 requirement for band n259 (18.7 dBm).

Table 1. PC3 minimum peak EIRP evaluation for band n262

| Parameter | Unit | Freq. range 47.2 - 48.2 GHz |
| --- | --- | --- |
| Pout per element | dBm | 11 |
| # of antennas in array |  | 4 |
| Total conducted power per polarization | dBm | 17.0 |
| Avg. antenna element gain | dBi | 4.0 |
| Antenna roll-off loss vs freq. | dB | -2.5 |
| Realized antenna array gain | dBi | 7.5 |
| Polarization gain | dB | 2.80 |
| Mismatch and transmission line loss including load pull | dB | -3.50 |
| Beam forming loss (phase shifter and amplitude error) | dB | -0.50 |
| Finite beam table | dB | -0.25 |
| Beam forming loss (one beam table fits all) | dB | -0.25 |
| Form-factor integration losses | dB | -5.80 |
| Total implementation loss | dB | -10.30 |
| Peak EIRP (Minimum) | dBm | 17.0 |

Observation 1: The derived PC3 minimum peak EIRP value for band n262 is 17dBm. Compared to the PC3 requirement for band n259, this represents a reduction of 1.7dB.

Proposal 1: Define the PC3 minimum peak EIRP requirement of band n262 as 17dBm.

### 2.1.2 Minimum peak EIS

Table 2 details the parameters for the minimum peak EIS derivation. The resulting value is a less sensitive minimum peak EIS compared to band n259, again mostly due to the higher frequency (increased NF and overall losses). The derived result is -83.2dBm, which represents a 1.5dB change compared to the n259 requirement (-84.7 dBm).

Table 2. PC3 minimum peak EIS evaluation for band n262

| Parameter | Unit | Value |
| --- | --- | --- |
| Band number |  | n262 |
| Modulation |  | QPSK |
| Bandwidth | MHz | 50 |
| SNR | dB | -1 |
| Thermal noise - 10log[(k*T*BW)/1mW] | dBm | -97 |
| Noise figure | dB | 12 |
| Effective array gain - 10log(# ant.) + element gain + roll-off | dB | 7.5 |
| Total implementation loss | dB | 10.3 |
| Peak EIS (Minimum) | dBm | -83.2 |

Observation 2: The derived PC3 minimum peak EIS value for band n262 is -83.2dBm. Compared to the PC3 requirement for band n259, the difference is 1.5dB.

Proposal 2: Define the PC3 minimum peak EIS requirement of band n262 as -83.2dBm (for 50 MHz bandwidth).

# 3	Conclusions

In this paper we provided the link budget parameters for the new FR2 band n262 and derived its PC3 minimum peak EIRP and EIS values. The following observations and proposals have been made:

Observation 1: The derived PC3 minimum peak EIRP value for band n262 is 17dBm. Compared to the PC3 requirement for band n259, this represents a reduction of 1.7dB.

Proposal 1: Define the PC3 minimum peak EIRP requirement of band n262 as 17dBm.

Observation 2: The derived PC3 minimum peak EIS value for band n262 is -83.2dBm. Compared to the PC3 requirement for band n259, the difference is 1.5dB.

Proposal 2: Define the PC3 minimum peak EIS requirement of band n262 as -83.2dBm (for 50 MHz bandwidth).

# 4	References

- 3GPP TS38.101-2, version 16.5.0, October 2020

- 3GPP TS38.104, version 16.5.0, October 2020

- RP-201232, “New WID: introduction of NR 47 GHz band,” T-Mobile USA, Dish Network, RAN #88e, June 2020

- R4-2011873, “Email discussion summary for [96e][135] NR_47GHz_Band,” Nokia, RAN4 #96e, August 2020

- R4-2011818, “WF on UE RF requirement for 47 GHz band,” Qualcomm, RAN4 #96e, August 2020

- R4-2011819, “WF on UE testability above 43.5 GHz,” Rohde & Schwarz, Apple, RAN4 #96e, August 2020

- R4-2011817, “WF on link budget parameters for Tx/Rx of n262,” Huawei, HiSilicon, Apple, RAN4 #96e, August 2020