3GPP TSG-RAN WG4 Meeting #97-e	R4-2015897

# Electronic meeting, November 2nd – 13th, 2020

Source:		Ericsson

Title:			SI IMT parameters – DL simulation results

Agenda item:		11.1.3.2

Document for:		Discussion

# Introduction

ITU-R WP5D sent a LS ([1]) to 3GPP RAN requesting for the parameters in a set of frequency ranges. For the ranges below 6GHz, it was agreed in RAN#87-e meeting to answer as soon as possible considering 3GPP has already defined bands for those ranges. But for the ranges 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz, this is not the case for the frequency ranges 6.425-7.125GHz and 10.0-10.5GHz A new SI ([2]) has then been agreed in last RAN#87-e meeting to address ITU-R WP5D’s request.

In last RANe#96-e meeting, the first simulation results were shared but, due to the large spread, it was not possible to come to any conclusion. A Way Forward [4] was then agreed to progress on this topic, with a first calibration offline with the active companies so far on this topic.

This contribution is providing our UL simulation results based on this WF [4].

# Discussion

## Macro urban uncoordinated scenario

### Assumptions

The baseline we used was set based on current specifications, i.e. 45dB BS ACLR and 33dB UE ACS, ACIR would then be 32.7dB.

Based on the agreed simulation assumptions, we simulate with different values of ACIR and compared with the ideal situation when there is no interference in between adjacent networks to evaluate the throughput impacts of the various ACIR values.

Results are given in the following sub-sections.

### 6.425-7.125 GHz

#### Calibration

As agreed in the WF [4], we first provide our DL SINR cdf (Figure 1) to calibrate all companies’ simulations.

Figure 1: DL SINR cdf

#### Simulations

Following Table 1 and Figure 2 capture the throughput loss at 5%-tile and average for various ACIR values.

Figure 2: Throughput loss at average at 5%-tile an Average for 7GHz

| ACIR (dB) | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Throughput lost at Average | 2.7 | 2.4 | 2.1 | 1.8 | 1.6 | 1.4 | 1.2 | 1.0 | 0.9 | 0.8 | 0.7 |
| Throughput lost at 5%-tile | 13.1 | 10.6 | 10.0 | 8.3 | 6.6 | 6.1 | 5.5 | 4.7 | 3.1 | 2.2 | 2.1 |

Table 1: Throughput lost for different ACIR values at 7GHz

Based on our simulation results, the DL ACIR value for 7GHz should be ~30dB (a bit less).

### 10.0-10.5 GHz

#### ISD issue

The results were not conclusive at 10 GHz with 450m ISD, as it was already observed by some companies in last RAN4#96-e meeting. Following the agreed Way Forward ([5]), we made then UL and DL simulations with an ISD of 400m for 10 GHz.

#### Calibration

As agreed in the WF [4], we first provide our DL SINR cdf (Figure 3) to calibrate all companies’ simulations.

Figure 3: DL SINR cdf

#### Simulations

Following

| ACIR (dB) | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Throughput lost at Average | 2.1 | 1.9 | 1.7 | 1.4 | 1.3 | 1.1 | 0.9 | 0.8 | 0.7 | 0.6 | 0.5 |
| Throughput lost at 5%-tile | 11.8 | 10.0 | 9.6 | 9.4 | 7.8 | 6.3 | 4.8 | 4.7 | 4.2 | 3.9 | 3.5 |

Table 2 and Figure 4 capture the throughput loss at 5%-tile and average for various ACIR values.

Figure 4: Throughput loss at average at 5%-tile an Average for 10GHz

Following table captures the throughput loss at 5%-tile and average for various ACIR values.

| ACIR (dB) | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Throughput lost at Average | 2.1 | 1.9 | 1.7 | 1.4 | 1.3 | 1.1 | 0.9 | 0.8 | 0.7 | 0.6 | 0.5 |
| Throughput lost at 5%-tile | 11.8 | 10.0 | 9.6 | 9.4 | 7.8 | 6.3 | 4.8 | 4.7 | 4.2 | 3.9 | 3.5 |

Table 2: Throughput lost for different ACIR values at 10GHz

Based on our simulation results, the DL ACIR value for 10GHz should below  ~29dB.

## Observation from the 7-24GHz SI

During the SI on 7-24GHz, feasibility studies were done and some key parameters were captures in the associated TR ([3]). Among them, achievable ACLR values have been investigated and the outcomes are captured in section 5.5.2 of this TR. It should be noticed that, considering an acceptable PA efficiency for the targeted output power and available technology, BS ACLR should not exceed 38dB (at 8 GHz), which looks also relevant value from the coexistence simulation results.

Observation 1: According to feasibility studies in the context of the 7-24GHz SI, BS ACLR should not exceed 38dB at 8GHz.

## Indoor scenario

No antenna parameters were agreed, nor even discussed for indoor deployement. It was then not relevant to simulate this deployment scenarion. It should be further discussed then if indoor would need to be simulated and if it should be a considered scenario for those frequencies.

Observation 2: Antenna parameters for indoor were not discussed. Indoor scenario consideration would need further discussion.

# Conclusion

In this contribution, we provide our simulation results in DL for the coexience study related to the SI ITU-R IMT parameters. We also made following observation:

Observation 1: According to feasibility studies in the context of the 7-24GHz SI, BS ACLR should not exceed 38dB at 8GHz.

Observation 2: Antenna parameters for indoor were not discussed. Indoor scenario consideration would need further discussion.

# References

- RP-200042, Parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-23, LS from ITU-R WP5D

- RP-200513, Study on IMT parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz, Ericsson, Huawei.

- TR 38.820, NR, 7-24 GHz frequency range, v16.0.0

- R4-2011827, WF on the Simulations for the SI on 6.425-7.125GHz and 10.0-10.5GHz, Nokia

- R4-2011827, WF on Simulation Assumptions for the SI on 6.425-7.125GHz and 10.0-10.5GHz, Nokia