3GPP TSG-RAN WG4 Meeting #97-e		                                     R4-2015913

Electronic Meeting, 02 November – 13 November 2020

# Source:	Thales

# Title:	NTN use case scenarios and architectures

# Type:	discussion

# Document for:	Discussion

# Agenda Item:	12.8.2 Use cases and deployment scenarios

Release	Rel-17

# Introduction

The objective of this document is to establish working assumption for the scenarios and use cases to be considered by NTN RAN4 work for the definition of the generic and core requirements for NTN-NR.

RP-202120 was discussed at plenary meeting RAN#89-e, and the 3th proposal is:

“Proposal 3: The proponents of a RAN4 led “satellite” band specific WI are expected to reference all the relevant sources (including but are not limited to ITU-R Radio Regulations, relevant national regulations, pre-existing Harmonized Standards developed for example in ETSI, coexistence studies approved by regulatory bodies and/or 3GPP specifications) and contribute to the definition of the requirements that should be met. Moreover they shall clarify the use case scenarios and architectures (e.g. orbit, altitude, type of UE, duplex mode, channel bandwidth, SCS...) provided that the use case scenarios and architectures are within the scope of WID “NR-NTN-solutions” in order to define the requirements.”

In order to further clarify use cases to be taken into account for KPI and RAN4 core requirements definition, we therefore want to clarify SCS parameters and frequency bandwidths to be considered, together with some other essential parameters. Moreover, use cases and satellite deployments scenarios to be considered by RAN4 need to be further clarified.

# Use case scenarios and architectures

The objective of this section is to recall deployment scenarios and NTN essential system characteristics such as e.g. orbit, altitude, type of UE, duplex mode, channel bandwidth, SCS...) and show the relationship with TR 38.821.

## Possible parameters and use cases to be considered by for NR based Satellite networks

In NTN scenarios, and with respect to the MSS S-band proposed during RAN4#97-e as an exemplary frequency band in FR1, the NR frequency channel bandwidth can be 5, 10, 15, 20, MHz for Frequency Range FR1, as shown in Table 1.

Furthermore, a channel bandwidth is associated with various RF Requirements (e.g. out-of-band emission) which may reduce the system frequency bandwidth, with guard band ratio from 20% to 2% in FR1.

Table 1 All potentially supported FR1 NR NTN carrier bandwidths and transmission bandwidth configurations for max 30 MHz bandwidth

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz |
| --- | --- | --- | --- | --- | --- | --- |
| 15 | 25 RBs | 52 RBs | 79 RBs | 106 RBs | 133 RBs | 160 RBs |
|  | 4.50 MHz | 9.36 MHz | 14.22 MHz | 19.08 MHz | 23.94 MHz | 28.8 MHz |
|  | 242,5 kHz Minimum Guard Band | 312,5 kHz Minimum Guard Band | 382,5 kHz Minimum Guard Band | 452,5 kHz Minimum Guard Band | 522,5 kHz Minimum Guard Band | 592,5 kHz Minimum Guard Band |
| 30 | 11 RBs | 24 RBs | 38 RBs | 51 RBs | 65 RBs | 78 RBs |
|  | 3.96 MHz | 8.64 MHz | 13.68 MHz | 18.36 MHz | 23.40 MHz | 28.08 MHz |
|  | 505 kHz Minimum Guard Band | 665 kHz Minimum Guard Band | 645 kHz Minimum Guard Band | 805 kHz Minimum Guard Band | 785 KHz Minimum Guard Band | 945 kHz Minimum Guard Band |
| 60 | N/A | 11 RBs | 18 RBs | 24 RBs | 31 RBs | 38 RBs |
|  | N/A | 7.92 MHz | 12.96 MHz | 17.28 MHz | 22.32 MHz | 27.36 MHz |
|  | N/A | 1010 kHz Minimum Guard Band | 990 kHz Minimum Guard Band | 1330 kHz Minimum Guard Band | 1310 kHz Minimum Guard Band | 1290 kHz Minimum Guard Band |

Proposal 1: Consider SCS 15 & 30 kHz for FR1 exemplary band for RAN4 work.

Considering the supported NR NTN carrier bandwidths, and depending on the available system frequency bandwidth, RAN4 needs to identify selected scenarios for Fractional Frequency Reuse (FRF) Schemes.

From RAN1 work it seems that solutions with frequency reuse > 1 will be assumed in Rel-17.

Proposal 2: Consider frequency reuse schemes with frequency reuse > 1 for RAN4 work.

Moreover, currently frequency band n65 considers several restrictions in terms of BandWidth configurations (with respect to potential BW configuration per FR1), as presented in Table 2.

Table 2 Supported FR1 NR NTN carrier bandwidths and transmission bandwidth configuration for frequency band n65

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| --- | --- | --- | --- | --- |
| 15 | 25 RBs | 52 RBs | 79 RBs | 106 RBs |
|  | 4.50 MHz | 9.36 MHz | 14.22 MHz | 19.08 MHz |
|  | 242,5 kHz Minimum Guard Band | 312,5 kHz Minimum Guard Band | 382,5 kHz Minimum Guard Band | 452,5 kHz Minimum Guard Band |
| 30 | - | 24 RBs | 38 RBs | 51 RBs |
|  | - | 8.64 MHz | 13.68 MHz | 18.36 MHz |
|  | - | 665 kHz Minimum Guard Band | 645 kHz Minimum Guard Band | 805 kHz Minimum Guard Band |
| 60 | - | 11 RBs | 18 RBs | 24 RBs |
|  | - | 7.92 MHz | 12.96 MHz | 17.28 MHz |
|  | - | 1010 kHz Minimum Guard Band | 990 kHz Minimum Guard Band | 1330 kHz Minimum Guard Band |

Proposal 3: Consider exemplary frequency bandwidths of 5, 10, 15, 20 MHz for FR1 RAN4 work.

Considering the supported NR NTN carrier bandwidths, and depending on the available system frequency bandwidth, RAN4 needs to identify coexistence scenarios.

Proposal 4: RAN4 needs to identify coexistence scenarios in adjacent bands.

Moreover, the reference scenarios from TR 38.821 can be down-selected to those from Table 3 below.

Table 3 Reference satellite scenarios for Rel-17 work item “NR_NTN_solutions”

| Scenarios | C1.1 | C2.1 | A1 |
| --- | --- | --- | --- |
| Orbit | LEO @ 600 km altitude | LEO @ 600 km altitude | GEO @ 35,786 km altitude |
| Frequency band | FR1 | FR1 | FR1 |
| Beams generation | Earth fixed beams (Note 1) | Earth moving beams | Earth fixed beams |

Proposal 5: For exemplary band S, RAN4 should consider scenarios C1.1, C2.1 (LEO Earth Fixed Beams and Earth Moving Beams) and A1 (GEO).

Table x Reference satellite scenarios for Rel-17 work item “NR_NTN_solutions”

| Scenarios | C1.1 | C2.1 | A1 |
| --- | --- | --- | --- |
| Orbit | LEO @ 600 km altitude | LEO @ 600 km altitude | GEO @ 35,786 km altitude |
| Frequency band | FR1 | FR1 | FR1 |
| Beams generation | Earth fixed beams | Earth moving beams | Earth fixed beams |

Table 4: Reference scenario parameters

| Scenarios | GEO based non-terrestrial access network (Scenario A) | LEO based non-terrestrial access network (Scenario C) |
| --- | --- | --- |
| Orbit type | notional station keeping position fixed in terms of elevation/azimuth with respect to a given Earth point | circular orbiting around the Earth |
| Altitude | 35,786 km | 600 km |
| Spectrum (service link) | FR1 (e.g. S band) FR2 (TBD) |  |
| Max channel bandwidth capability (service link) | 30 MHz for FR1 400 MHz for FR2 (TBD) |  |
| Payload | Scenario A: Transparent (including radio frequency function only) | Scenario C: Transparent (including radio frequency function only) |
| Inter-Satellite link | No | No |
| Earth-fixed beams | Yes | Scenario C1: Yes (steerable beams), see Note 1 Scenario C2: No (the beams move with the satellite) |
| Max beam foot print size (edge to edge) regardless of the elevation angle | 3500 km (Note 5) | 1000 km |
| Min Elevation angle for both sat-gateway and user equipment | 10° for service link and 10° for feeder link | 10° for service link and 10° for feeder link |
| Max distance between satellite and user equipment at min elevation angle | 40,581 km | 1,932 km (600 km altitude) |
| Max Round Trip Delay (propagation delay only) | Scenario A: 541.46 ms (service and feeder links) | Scenario C: (transparent payload: service and feeder links) 25.77 ms (600km) |
| Max differential delay within a cell (Note 6) | 10.3 ms | 3.12 ms for 600km |
| Max Doppler shift (Earth fixed user equipment) | 0.93 ppm | 24 ppm (600km) |
| Max Doppler shift variation (Earth fixed user equipment) | 0.000045 ppm/s | 0.27 ppm/s (600km) |
| Maximum Delay variation as seen by the UE (Note 7) | Negligible | Up to +/- 48 µs/sec (600 km) |
| Service link | 3GPP defined New Radio |  |
| Note 1: Each satellite has the capability to steer beams towards fixed points on Earth using beamforming techniques. This is applicable for a period of time corresponding to the visibility time of the satellite, from the point of view of the ground cell. Note 2: Max delay variation within a beam (Earth fixed user equipment) is calculated based on minimum elevation angle for both gateway and user equipment. Note 3: Max differential delay within a beam is calculated based on maximum beam foot print diameter at nadir. Note 4: Speed of light used for delay calculation is 299792458 m/s. Note 5: The maximum beam foot print size for GEO is based on current state of the art GEO high throughput systems, assuming beam edge at low elevation. Note 6: The maximum differential delay at cell level has been computed considering the one at beam level for largest beam size. It does not preclude that cell may include more than one beam when beam size are small or medium size. However the cumulated differential delay of all beams within a cell will not exceed the maximum differential delay at cell level in the table above. Note 7: The delay variation measures how fast the round trip delay (function of UE-satellite-NTN gateway distance) varies over time when the satellite moves towards/away from the UE. It is expressed in µs/s and is negligible for GEO scenario. |  |  |

The NTN study results apply to GEO scenarios as well as all NGSO scenarios with circular orbit at altitude greater than or equal to 600 km.

## User Equipment

The User Equipment considered for the key reference scenario parameters can be found in Table 5 of this document.

Table 5 Reference satellite scenarios: User equipment types

| User equipment characteristics | Handheld | VSAT (Note 1) |
| --- | --- | --- |
| Antenna type | Omnidirectional antenna | Directive antenna (up to 60 cm equivalent aperture diameter) |
| Motion on the Earth | 500 km/h (e.g. on board a high speed train) | Up to 1200 km/h (e.g. aircraft mounted) |
| Antenna polarisation | Linear: +/-45°X-pol | Circular |
| Max transmit power | up to 200 mW (UE power class 3) | up to 20 W |
| Note 1: VSAT terminal characteristics could be implemented with phased array antenna. It may be used for ESIM (Earth Station in Motion) to be mounted on Moving platforms (e.g., aircrafts, vessels) or building |  |  |

Proposal 6: RAN4 should consider the following UE key reference scenario parameters:

Table y Reference satellite scenarios: User equipment types

| User equipment characteristics | Handheld | VSAT (Note 1) |
| --- | --- | --- |
| Antenna type | Omnidirectional antenna | Directive antenna (up to 60 cm equivalent aperture diameter) |
| Motion on the Earth | 500 km/h (e.g. on board a high speed train) | Up to 1200 km/h (e.g. aircraft mounted) |
| Antenna polarisation | Linear: +/-45°X-pol | Circular |
| Max transmit power | up to 200 mW (UE power class 3) | up to 20 W |
| Note 1: VSAT terminal characteristics could be implemented with phased array antenna. It may be used for ESIM (Earth Station in Motion) to be mounted on Moving platforms (e.g., aircrafts, vessels) or building |  |  |

UEs with GNSS capabilities are assumed in RAN1, and hence UEs may have the capability to determine their own position. Thanks to these capabilities, UEs may (or may not) pre-compensate timing and frequency offset.

However, both UE with and without the capability for pre-compensation of timing and frequency offset should be supported. The reasons are that processing capabilities (such as NR UE processing) and/or service availability (such as 5GC available services) may differ or may be decoupled.

Proposal 7: UE with GNSS capabilities are assumed for RAN4 work.

Proposal 8: UE with/without capability for timing and/or frequency pre-compensation should be supported in NTN WI and further considered by RAN4.

Proposal 9: RAN4 should follow RAN1 outcomes for the synchronization solutions to be considered.

## Satellite Parameters

In TR 38.821, see clause 6 for satellite parameters.

The following tables representing two sets of satellite parameters are considered as the baseline for system level simulator calibration in TR 38.821, and can be further considered by RAN4 work:

Table 6.1.1.1-1: Set-1 satellite parameters for system level simulator calibration

| Satellite orbit |  | GEO | LEO-600 |
| --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in TR 38.811 | Section 6.4.1 in TR 38.811 |
| Payload characteristics for DL transmissions |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | S-band (i.e. 2 GHz) | 22 m | 2 m |
| Satellite EIRP density |  | 59 dBW/MHz | 34 dBW/MHz |
| Satellite Tx max Gain |  | 51 dBi | 30 dBi |
| 3dB beamwidth |  | 0.4011 deg | 4.4127 deg |
| Satellite beam diameter (Note 2) |  | 250 km | 50 km |
| Payload characteristics for UL transmissions |  |  |  |
| Equivalent satellite antenna aperture (Note1) | S-band (i.e. 2 GHz) | 22 m | 2 m |
| G/T |  | 19 dB K-1 | 1.1 dB K-1 |
| Satellite Rx max Gain |  | 51 dBi | 30 dBi |
| Note 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of TR 38.811. Note 2: This beam size refers to the Nadir pointing of the satellite |  |  |  |

Table 6.1.1.1-2: Set-2 satellite parameters for system level simulator calibration

| Satellite orbit |  | GEO | LEO-600 |
| --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in TR 38.811 | Section 6.4.1 in TR 38.811 |
| Payload characteristics for DL transmissions |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | S-band (i.e. 2 GHz) | 12 m | 1 m |
| Satellite EIRP density |  | 53.5 dBW/MHz | 28 dBW/MHz |
| Satellite Tx max Gain |  | 45.5 dBi | 24 dBi |
| 3dB beamwidth |  | 0.7353 deg | 8.8320 deg |
| Satellite beam diameter (Note 2) |  | 450 km | 90 km |
| Payload characteristics for UL transmissions |  |  |  |
| Equivalent satellite antenna aperture (Note1) | S-band (i.e. 2 GHz) | 12 m | 1 m |
| G/T |  | 14 dB K-1 | -4.9 dB K-1 |
| Satellite Rx max Gain |  | 45.5 dBi | 24 dBi |
| Note 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of TR 38.811. Note 2: This beam size refers to the Nadir pointing of the satellite |  |  |  |

Proposal 10: RAN4 should use TR 38.821 assumptions for satellite parameters.

# Conclusions

Proposal 1: Consider SCS 15 & 30 kHz for FR1 exemplary band for RAN4 work.

Proposal 2: Consider frequency reuse schemes with frequency reuse > 1 for RAN4 work.

Proposal 3: Consider exemplary frequency bandwidths of 5, 10, 15, 20 MHz for FR1 RAN4 work.

Proposal 4: RAN4 needs to identify coexistence scenarios in adjacent bands.

Proposal 5: For exemplary band S, RAN4 should consider scenarios C1.1, C2.1 (LEO Earth Fixed Beams and Earth Moving Beams) and A1 (GEO).

Table x Reference satellite scenarios for Rel-17 work item “NR_NTN_solutions”

| Scenarios | C1.1 | C2.1 | A1 |
| --- | --- | --- | --- |
| Orbit | LEO @ 600 km altitude | LEO @ 600 km altitude | GEO @ 35,786 km altitude |
| Frequency band | FR1 | FR1 | FR1 |
| Beams generation | Earth fixed beams | Earth moving beams | Earth fixed beams |

Proposal 6: RAN4 should consider the following UE key reference scenario parameters:

Table y Reference satellite scenarios: User equipment types

| User equipment characteristics | Handheld | VSAT (Note 1) |
| --- | --- | --- |
| Antenna type | Omnidirectional antenna | Directive antenna (up to 60 cm equivalent aperture diameter) |
| Motion on the Earth | 500 km/h (e.g. on board a high speed train) | Up to 1200 km/h (e.g. aircraft mounted) |
| Antenna polarisation | Linear: +/-45°X-pol | Circular |
| Max transmit power | up to 200 mW (UE power class 3) | up to 20 W |
| Note 1: VSAT terminal characteristics could be implemented with phased array antenna. It may be used for ESIM (Earth Station in Motion) to be mounted on Moving platforms (e.g., aircrafts, vessels) or building |  |  |

Proposal 7: UE with GNSS capabilities are assumed for RAN4 work.

Proposal 8: UE with/without capability for timing and/or frequency pre-compensation should be supported in NTN WI and further considered by RAN4.

Proposal 9: RAN4 should follow RAN1 outcomes for the synchronization solutions to be considered.

Proposal 10: RAN4 should use TR 38.821 assumptions for satellite parameters.

END