3GPP TSG-RAN WG4 Meeting #97-e		                                     R4-2015946

Electronic Meeting, 02 November – 13 November 2020

# Source:	Thales

# Title:	NTN RRM & Demodulation KPIs

# Type:	discussion

# Document for:	discussion

# Agenda Item:	12.8.4 RRM requirements

Release	Rel-17

# Introduction

RP-202120 was discussed at plenary meeting RAN#89-e, and the first, third and fourth discussed proposals were:

“Proposal 1: In RAN4, the WID “NR-NTN-solutions” will define the generic and core requirements and consider at least one example satellite band. RAN4 to decide based on technical considerations which band(s) to consider as exemplary band for the WI “NR-NTN-solutions”, with at least the pre-condition that the intended usage is compliant with ITU-R Radio Regulations.”

“Proposal 3: The proponents of a RAN4 led “satellite” band specific WI are expected to reference all the relevant sources (including but are not limited to ITU-R Radio Regulations, relevant national regulations, pre-existing Harmonized Standards developed for example in ETSI, coexistence studies approved by regulatory bodies and/or 3GPP specifications) and contribute to the definition of the requirements that should be met. Moreover they shall clarify the use case scenarios and architectures (e.g. orbit, altitude, type of UE, duplex mode, channel bandwidth, SCS...) provided that the use case scenarios and architectures are within the scope of WID “NR-NTN-solutions” in order to define the requirements.”

“Proposal 4: Traditional 3GPP work for developing generic requirements, such as inter-carrier co-existence to decide ACLR etc. should be followed where possible but may have to be adapted for the satellite case. Adaptations if needed shall be defined by RAN4. Satellite bands introduced in 3GPP for NTN shall neither impact the existing specifications of nor cause degradation (in the sense of RAN4 co-existence studies) to present and future networks in 3GPP specified terrestrial bands.”

The discussed proposals were made with respect to references to be used (e.g., 3GPP, ETSI, ITU-R), and specific requirements to be considered.

The objective of this document is therefore to propose a framework for NTN core requirements and consider in particular the potential demodulation Key Performance Indicators (KPIs) & RRM aspects to be considered by NTN RAN4 work.

# Potential KPIs to be considered as demodulation KPIs:

# RAN1 is currently considering different potential methods for synchronization and timing advance. For example, one of the methods is currently considering UE pre-compensation using GNSS and potentially ephemeris data.

Proposal 1: RAN4 should use RAN1 framework when defining requirements for UE UL synchronization and TA mechanisms.

The GNSS performance has to be therefore considered when defining demodulation KPIs & RRM performance. For example, the Time Figure of Merit (TFOM) is a parameter that reflects the estimated error range values between the reference value (expressed in pulse per second, PPS or 1PPS, i.e. GNSS 1PPS, such as GPS 1PPS) and the UE system value (which is being aligned to the reference 1PPS). The estimated error is referred to as the 1PPS Phase error. TFOM values are ranges of these phase errors. The larger the phase error estimate, the larger the TFOM value will be. If this estimated error is too large, it could adversely affect the performance of the UE UL synchronization and TA pre-compensation.

For example, TFOM 4 is reported when the estimated phase error is any value between 100 ns to less than 1 µs of the offset between the selected 1PPS reference and the system’s 1PPS, which may provide sufficient accuracy for 5G NTN UE synchronization scenarios based on GNSS in FR1. On the other hand, TFOM 3 is reported when the estimated phase error is any value between 10 ns to less than 100 ns of the offset between the selected 1PPS reference and the system’s 1PPS, which may provide sufficient accuracy for 5G NTN UE synchronization scenarios based on GNSS in FR2.

Therefore, some GNSS accuracy requirements need to be considered when specifying (at least) demodulation KPIs and RRM requirements.

Proposal 2: For FR1 frequency range, GNSS-based UL synchronization and TA mechanisms using pre-compensation shall use a GNSS accuracy assumption of TFOM value 4, which considers an Estimated Time Error (ETE) between 100 ns and 1 µs.

Proposal 3: For FR2 frequency range, GNSS-based UL synchronization and TA mechanisms using pre-compensation shall use a GNSS accuracy assumption of TFOM value 3, which considers an Estimated Time Error (ETE) between 10 ns and 100 ns.

Furthermore, NTN deals with specific scenarios in terms of Doppler compensation & Timing Advance. These specific scenarios may impact the performance of the system, if not sufficient accurate estimation and/or (pre)compensation. Therefore, among (existent) parameters, for NTN scenarios we should also (at least) consider:

- Specific NTN achievable throughput for each modulation expressed in terms of % of max throughput;

- Requirements on satellite position/velocity accuracy estimate;

- Specific NTN handover KPIs (e.g. interruption time);

- Specific NTN testing configurations with NTN specific Doppler;

- Specific NTN timing accuracy;

- Specific NTN RSRP/RSRQ measurement accuracy;

- Etc.

Proposal 4: RAN4 should start considering a list of potential RRM and demodulation KPIs with respect to considered NTN use cases.

Proposal 5: RRM & demodulation KPIs may include (at least):

- Specific NTN requirements in terms of accuracy estimation for satellite position/velocity;

- Specific NTN requirements for handover KPIs (e.g. interruption time);

- Specific NTN testing configurations with NTN specific Doppler;

- Specific NTN requirements in terms of timing accuracy;

- Specific NTN requirements for RSRP/RSRQ measurement accuracy.

Proposal 6: RRM & demodulation KPIs may include (at least):

- Specific NTN requirements in terms of achievable throughput for each modulation expressed in terms of percentage of max throughput and per given modulation;

# Specific TS 38.133 list of requirements

TS 38.133 accuracy requirements are normally used by 3GPP to define test requirements to be met by test specification (e.g TS 38.533), together with some test tolerances. Some of the legacy 3GPP requirements have been selected in the table below. This concerns e.g. active states, inactive states, connected state, timing and signaling characteristics, and some of the accuracy requirements define some precision in terms of e.g. cell selection, cell reselection, measurement, interruption time, etc.

Table 1: Parameters related to SA mobility states

| SA Mobility States | Essential Parameter | Parameter Name | Parameter meaning, remark, or specific requirement |
| --- | --- | --- | --- |
| RRC_IDLE state mobility | Cell Re-selection | UE measurement capability | Capability of UE to monitor intra-frequency and inter-frequency (NTN or TN) carrier. |
|  |  | Measurement and evaluation of serving cell | The NTN UE shall be able to measure the SS-RSRP and SS-RSRQ level of the serving cell and evaluate the cell selection criterion. UE filtering of the SS-RSRP and SS-RSRQ measurements of the serving cell using at least X measurements, where X FFS. |
|  |  | Measurements of intra-frequency NR cells | The NTN UE shall be able to identify new intra-frequency cells and perform SS-RSRP and SS-RSRQ measurements of the identified intra-frequency cells without an explicit intra-frequency neighbor list containing physical layer cell identities. The NTN UE shall measure SS-RSRP and SS-RSRQ at least every Tmeasure,NR_Intra equivalent parameter. NTN UE filtering of the SS-RSRP and SS-RSRQ measurements of the serving cell using at least X measurements, where X FFS. |
|  |  | Measurements of inter-frequency NR cells | The NTN UE shall be able to identify new inter-frequency cells and perform SS-RSRP or SS-RSRQ measurements of identified inter-frequency cells if carrier frequency information is provided by the serving cell, even if no explicit neighbour list with physical layer cell identities is provided. If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the NTN UE shall search for inter-frequency layers of higher priority at least every (NTN equivalent) Thigher_priority_search. Conditions to perform equal priority reselection need to be defined. |
|  |  | Maximum interruption in paging reception | NTN UE shall perform the cell re-selection with minimum interruption in monitoring downlink channels for paging reception. At intra-frequency and inter-frequency cell re-selection, the NTN UE shall monitor the downlink of serving cell for paging reception until the NTN UE is capable to start monitoring downlink channels of the target intra-frequency and inter-frequency cell for paging reception. The interruption time shall not exceed a specific NTN defined value. |
|  |  | Minimum requirement at transitions | Tmeasure,NR_Intra, Tevaluate,NR_Intra, Tmeasure,NR_Inter, Tevaluate,NR_Inter |
|  |  | Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion | N/A for NTN, since it concerns: Measurements for UE fulfilling low mobility criterion Measurements for UE fulfilling not-at-cell edge criterion Measurements for UE fulfilling low mobility and not-at-cell edge criteria |
|  | Minimization of Drive Tests (MDT) | Measurement Requirements | FSS, potentially N/A for NTN |
|  |  | Requirements for Relative Time Stamp Accuracy |  |
|  |  | Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting | FSS, potentially N/A for NTN |
|  |  | Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting | FSS, potentially N/A for NTN |
| RRC_INACTIVE state mobility | Cell Re-selection | UE measurement capability | Same as the ones for IDLE. |
|  |  | Measurement and evaluation of serving cell | Same as the ones for IDLE. |
|  |  | Measurements of intra-frequency NR cells | Same as the ones for IDLE. |
|  |  | Measurements of inter-frequency NR cells | Same as the ones for IDLE. |
|  |  | Maximum interruption in paging reception | Same as the ones for IDLE. |
|  | Minimization of Drive Tests (MDT) | Measurement Requirements | Same as the ones for IDLE. |
|  |  | Requirements for Relative Time Stamp Accuracy | Same as the ones for IDLE. |
|  |  | Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting | Same as the ones for IDLE. |
|  |  | Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting | Same as the ones for IDLE. |
|  |  | Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting | Same as the ones for IDLE. |
| RRC_CONNECTED state mobility | Handover Parameters - NR Handover | NR FR1 - NR FR1 Handover | Specific-NTN definition for Handover delay Specific-NTN definition for Interruption time |
|  |  | NR FR2- NR FR1 Handover | N/A (or FFS) |
|  |  | NR FR2- NR FR2 Handover | Specific-NTN definition for Handover delay Specific-NTN definition for Interruption time |
|  |  | NR FR1- NR FR2 Handover | N/A (or FFS) |
|  | Handover Parameters - NR DAPS (Dual-Active Protocol Stack) Handover | NR FR1 - NR FR1 DAPS Handover | FFS |
|  |  | NR FR2- NR FR1 DAPS Handover | FFS |
|  |  | NR FR1- NR FR2 DAPS Handover | FFS |
|  | Handover Parameters - NR Conditional Handover | NR FR1 – NR FR1 conditional handover | Specific-NTN definition for Handover delay Specific-NTN definition for Measurement time Specific-NTN definition for Preparation time Specific-NTN definition for Interruption time |
|  |  | NR FR2 – NR FR1 conditional handover | FFS |
|  |  | NR FR2 – NR FR2 conditional handover | Specific-NTN definition for Handover delay Specific-NTN definition for Measurement time Specific-NTN definition for Preparation time Specific-NTN definition for Interruption time |
|  |  | NR FR1 – NR FR2 conditional handover | FFS |
|  | RRC Connection Mobility Control | SA: RRC Re-establishment | Specific-NTN UE Re-establishment delay requirement |
|  |  | Random access | Specific-NTN Requirements for 4-step RA type Specific-NTN Requirements for 2-step RA type |
|  |  | SA: RRC Connection Release with Redirection | Specific-NTN RRC connection release with redirection to NR |

Proposal 7: Down-scope from TS 38.133 Stand-Alone mobility states parameters related to Cell-Reselection, MDT, HO, CHO.

Table 2: Parameters related to Timing and Signaling Characteristics

| Essential Parameter | Parameter Name | Parameter meaning, remark, or specific requirement |
| --- | --- | --- |
| Timing Parameter | UE transmit timing |  |
|  | UE timer accuracy |  |
|  | Timing advance |  |
|  | Cell phase synchronization accuracy |  |
|  | Maximum Transmission Timing Difference |  |
|  | Maximum Receive Timing Difference |  |
|  | deriveSSB-IndexFromCell tolerance |  |
| Signalling characteristics Parameter | Radio Link Monitoring |  |
|  | Interruption |  |
|  | SCell Activation and Deactivation Delay |  |
|  | UE UL carrier RRC reconfiguration delay |  |
|  | Link Recovery Procedures |  |
|  | Active BWP switch delay |  |
|  | PSCell Change |  |
|  | Uplink spatial relation switch delay |  |
|  | UE-specific CBW change |  |

Proposal 8: Down-scope from TS 38.133 Timing and Signaling Characteristics parameters.

Table 3: Parameters related to Measurement Procedures and Measurement Performance Requirements

| Essential Parameter | Parameter Name | Parameter meaning, remark, or specific requirement |
| --- | --- | --- |
| Measurement procedure | General measurement requirement | Measurement gap |
|  |  | UE Measurement capability |
|  |  | Capabilities for Support of Event Triggering and Reporting Criteria |
|  |  | Carrier-specific scaling factor |
|  |  | Minimum requirement at transitions |
|  | NR intra-frequency measurements | Requirements applicability |
|  |  | Number of cells and number of SSB |
|  |  | Measurement Reporting Requirements |
|  |  | Intra-frequency measurements without measurement gaps |
|  |  | Intra-frequency measurements with measurement gaps |
|  | NR inter-frequency measurements | Requirements applicability |
|  |  | Number of cells and number of SSB |
|  |  | Inter-frequency measurement with measurement gaps |
|  |  | Inter-frequency measurements reporting requirements |
|  |  | Inter-frequency SFTD measurement requirements |
|  |  | Inter frequency measurements without measurement gaps |
|  | L1-RSRP measurements for Reporting | Requirements applicability |
|  |  | Measurement Reporting Requirements (Periodic, Semi-Persistent, Aperiodic) |
|  |  | L1-RSRP measurement requirements |
|  |  | Measurement restriction for CSI-RS and SSB for L1-RSRP measurement |
|  |  | Scheduling availability of UE during L1-RSRP measurement |
|  | Cross Link Interference measurements | SRS-RSRP measurements |
|  |  | CLI-RSSI measurements |
|  |  | Scheduling availability of UE during CLI measurements |
|  | L1-SINR measurements for Reporting | Requirements applicability |
|  |  | Measurement Reporting Requirements (Periodic, Semi-Persistent, Aperiodic) |
|  |  | L1-SINR measurement requirements |
|  |  | Measurement restriction for L1-SINR measurement |
|  |  | Scheduling availability of UE during L1-SINR measurement |
|  | NR measurements for positioning | RSTD measurements (Requirements Applicability, Measurement Capability, Measurement Reporting Requirements) |
|  |  | PRS-RSRP measurements (Requirements Applicability, Measurement Capability, Measurement Reporting Requirements) |
|  |  | UE Rx-Tx time difference measurements (Requirements Applicability, Measurement Capability, Measurement Reporting Requirements, Measurement Period Requirements) |
|  |  | E-CID measurements (Measurement Requirements, Measurement Reporting Delay) |
|  | CSI-RS based L3 measurements | Requirements applicability |
|  |  | CSI-RS based Inter-frequency measurements |
|  | NR measurements with autonomous gaps | FFS |
| Measurement Performance requirements (NR measurements) | Intra-frequency RSRP accuracy requirements for FR1 | Specific-NTN Absolute SS-RSRP Accuracy Specific-NTN Relative SS-RSRP Accuracy |
|  | Intra-frequency RSRP accuracy requirements for FR2 | Specific-NTN Absolute SS-RSRP Accuracy Specific-NTN Relative SS-RSRP Accuracy |
|  | Inter-frequency RSRP accuracy requirements for FR1 | Specific-NTN Absolute SS-RSRP Accuracy Specific-NTN Relative SS-RSRP Accuracy |
|  | Inter-frequency RSRP accuracy requirements for FR2 | Specific-NTN Absolute SS-RSRP Accuracy Specific-NTN Relative SS-RSRP Accuracy |
|  | RSRP Measurement Report Mapping |  |
|  | Intra-frequency RSRQ accuracy requirements for FR1 | Absolute SS-RSRQ Accuracy in FR1 |
|  | Intra-frequency RSRQ accuracy requirements for FR2 | Absolute SS-RSRQ Accuracy in FR2 |
|  | Inter-frequency RSRQ accuracy requirements for FR2 |  |
|  | RSRQ report mapping |  |
|  | Intra-frequency SINR accuracy requirements for FR1 |  |
|  | Intra-frequency SINR accuracy requirements for FR2 |  |
|  | Inter-frequency SINR accuracy requirements for FR1 |  |
|  | Inter-frequency SINR accuracy requirements for FR2 |  |
|  | SINR report mapping |  |
|  | Power Headroom |  |
|  | PCMAX,c,f |  |
|  | L1-RSRP accuracy requirements for FR1 |  |
|  | L1-RSRP accuracy requirements for FR2 |  |
|  | SFTD accuracy requirements |  |
|  | CLI measurement accuracy requirements |  |
|  | RSTD Measurements | Measurement accuracy requirements Report mapping |
|  | PRS-RSRP Measurements | Measurement accuracy requirements Report mapping |
|  | UE Rx-Tx Time Difference Measurements | Measurement accuracy requirements Report mapping |
|  | FR2 P-MPR report | Report mapping |
| .. |  |  |

Proposal 9: Down-scope from TS 38.133 with respect to Measurement Procedures and Measurement Performance Requirements parameters.

# Specific NTN UL accuracy requirements

# NTN UL timing accuracy for initial access

In order to compute its User-specific TA, the UE can rely on the following procedure:

- The UE can derive its position based on its GNSS capabilities;

- The UE can derive the real-time position of the satellite based on its last acquisition of satellite Position-Velocity(-Time) or PV(T) values within the SI and some basic propagator model;

- The real-time propagation delay T1 between the satellite and the UE can be estimated with basic geometric formulas;

- The User specific TA is given by two times T1.

At the end, to maintain the PRACH sequences orthogonality, the residual error committed when User specific TA acquisition is performed should accommodate the following requirement:

TAerror<minCP2,GP2,Minimal Relative Cyclic Shift Duration2  [s]

This requirement can be interpreted as a requirement related to the accuracy associated to the distance satellite-UE where c denotes the speed of light:

ϵ<minCP2,GP2,Minimal Relative Cyclic Shift Duration2 ×c2  [m]

We can notice that the accuracy targets become more challenging when short preamble formats are considered and/or when the Zero Correlation Zone is set closer to 1.

Finally, the implication of this requirement on the UE 3D positioning error ΔU and the satellite 3D positioning error ΔS is detailed below (see e.g. Figure 1), where the UE 3D positioning error is represented by ΔU and the satellite 3D positioning error is represented by ΔS.

Figure 1: Geometric representation

Assuming that the UE is positioned on point U, UE estimates its location on point U’, and that the satellite is positioned on point S, and the UE estimates the satellite position on point S’, one can compute d=U'S' and d'= US.

Moreover, in order to maintain the PRACH sequences orthogonality, the residual error committed when User specific TA acquisition is performed should accommodate the following requirement:

TAerror<minCP2,GP2,Minimal Relative Cyclic Shift Duration2  [s]

This requirement can be interpreted as a requirement related to the accuracy associated to the distance satellite-UE, which is:

ϵ= d'-d<minCP2,GP2,Minimal Relative Cyclic Shift Duration2 ×c2  [m]

One can further derive the following equations:

U'S'2=d2=U'U2+US'2+2U'UUS'cosU'U,US'

US'2=US2+SS'2+2USSS'cosUS,SS'

d2=U'U2+US2+SS'2+2USSS'cosUS,SS'+2U'UUS2+SS'2+2USSS'cosUS,SS'cosU'U,US'

d2=ΔU2+d'2+ΔS2+2d'ΔScosUS,SS'+2ΔUd'2+ΔS2+2d'ΔScosUS,SS'cosU'U,US'

Knowing that -1≤cosX,Y≤1, one can define an upper bound for d²:

d²≤ΔU2+d'2+ΔS2+2d'ΔS+2ΔUd'2+ΔS2+2d'ΔS

d2≤ΔU2+d'+ΔS2+2ΔUd'+ΔS

d2≤d'+ΔS+ΔU2

d ≤d'+ΔS+ΔU

As a consequence, we can derive the following equation:

ϵ= d'-d≤ΔS+ΔU

Then, the requirement

ϵ<minCP2,GP2,Minimal Relative Cyclic Shift Duration2 ×c2  [m]

can be guaranteed as long as the following condition related to both satellite and UE 3D positioning errors holds:

ΔS+ΔU<minCP2,GP2,Minimal Relative Cyclic Shift Duration2 ×c2  [m]

For this reason, the following requirement on the UE and satellite 3D positioning errors is proposed:

ΔS+ΔU<minCP2,GP2,Minimal Relative Cyclic Shift Duration2 ×c2  [m]

Proposal 10: The UE shall be able to acquire its User specific TA with an accuracy better than ±min(CP/2,GP/2,(Minimal Relative Cyclic Shift Duration)/2) depending on the PRACH format and configuration.

The NTN-specific PRACH used format (if different from TN), PRACH CP, PRACH GP, and zero-correlation zone parameters (and other similar) are for the time being FSS.

Proposal 11: The UE 3D positioning error ΔU and the satellite 3D positioning error ΔS shall accommodate the following requirement: ΔU+ΔS < c/2 * min(CP/2,GP/2,(Minimal Relative Cyclic Shift Duration)/2).

# NTN UL frequency synchronization requirement

The key requirement for UL frequency synchronisation is the frequency error of ±0.1 ppm w.r.t the frequency carrier expected by the gNB, observed over a period of 1 ms.

Currently, this budget of ±0.1 ppm includes all the residual frequency errors due to frequency tracking error and crystal oscillator drifts. We propose that the residual frequency error resulting of the imperfect satellite mobility pre-compensation at UE side shall be included as well in this budget. It means that a portion of this budget shall be dedicated to this specific contribution.

Finally, it shall be investigated whether the Doppler shifts pre-compensation conducted by the UE can match the requirements in terms of UL frequency error depending on the sub-budget allocated to this source of error.

Proposal 12: UE shall be able to compensate the frequency offset due to the satellite mobility when generating its UL carrier frequency.

Proposal 13: The UE modulated carrier frequency shall be accurate to within ±0.1 ppm as observed over a period of 1 ms by the gNB.

Proposal 14: The UE residual frequency error shall be sufficiently low such that it can be considered included in the tolerated frequency error of ±0.1 ppm already captured in the specification.

# Conclusions

Proposal 1: RAN4 should use RAN1 framework when defining requirements for UE UL synchronization and TA mechanisms.

Proposal 2: For FR1 frequency range, GNSS-based UL synchronization and TA mechanisms using pre-compensation shall use a GNSS accuracy assumption of TFOM value 4, which considers an Estimated Time Error (ETE) between 100 ns and 1 µs.

Proposal 3: For FR2 frequency range, GNSS-based UL synchronization and TA mechanisms using pre-compensation shall use a GNSS accuracy assumption of TFOM value 3, which considers an Estimated Time Error (ETE) between 10 ns and 100 ns.

Proposal 4: RAN4 should start considering a list of potential RRM and demodulation KPIs with respect to considered NTN use cases.

Proposal 5: RRM & demodulation KPIs may include (at least):

- Specific NTN requirements in terms of accuracy estimation for satellite position/velocity;

- Specific NTN requirements for handover KPIs (e.g. interruption time);

- Specific NTN testing configurations with NTN specific Doppler;

- Specific NTN requirements in terms of timing accuracy;

- Specific NTN requirements for RSRP/RSRQ measurement accuracy.

Proposal 6: RRM & demodulation KPIs may include (at least):

- Specific NTN requirements in terms of achievable throughput for each modulation expressed in terms of percentage of max throughput and per given modulation;

Proposal 7: Down-scope from TS 38.133 Stand-Alone mobility states parameters related to Cell-Reselection, MDT, HO, CHO.

Proposal 8: Down-scope from TS 38.133 Timing and Signaling Characteristics parameters.

Proposal 9: Down-scope from TS 38.133 with respect to Measurement Procedures and Measurement Performance Requirements parameters.

Proposal 10: The UE shall be able to acquire its User specific TA with an accuracy better than ±min(CP/2,GP/2,(Minimal Relative Cyclic Shift Duration)/2) depending on the PRACH format and configuration.

The NTN-specific PRACH used format (if different from TN), PRACH CP, PRACH GP, and zero-correlation zone parameters (and other similar) are for the time being FSS.

Proposal 11: The UE 3D positioning error ΔU and the satellite 3D positioning error ΔS shall accommodate the following requirement: ΔU+ΔS < c/2 * min(CP/2,GP/2,(Minimal Relative Cyclic Shift Duration)/2).

Proposal 12: UE shall be able to compensate the frequency offset due to the satellite mobility when generating its UL carrier frequency.

Proposal 13: The UE modulated carrier frequency shall be accurate to within ±0.1 ppm as observed over a period of 1 ms by the gNB.

Proposal 14: The UE residual frequency error shall be sufficiently low such that it can be considered included in the tolerated frequency error of ±0.1 ppm already captured in the specification.

END