3GPP TSG-RAN WG4 #97-e		                              	 R4-2016011

Online, November 2nd – 13th, 2020

Source: 	Skyworks Solutions, Inc.

Title: 	n8 35MHz A-MPR and MSD Measurements

Agenda Item:	10.22.3 - UE RF requirements [NR_FR1_35MHz_45MHz_BW-Core]

Document for:	Approval

# Introduction

A new WI to study the introduction of 35 and 45 MHz channel bandwidth (CBW) has been agreed [1] and study assumptions agreed in WF [2]. This contribution brings measurement data and makes A-MPR and MSD TP proposals.

# Discussion

## n8 35MHz Spectrum Context

Figure 1 summarizes the n8 spectrum for 35MHz downlink (DL) channel bandwidth (CBW) operation and 3 uplink (UL) CBW scenario:

- Case A (top): case of 20MHz UL CBW using the lowest n8 channel. For this case, no A-MPR is needed, but MSD may require evaluation;

- Case B (middle): case of 20MHz UL CBW using the highest n8 channel. For this case, the upper ACL shoulders overlap the DL n8 channel by approximately 10MHz. Impact on Reference Sensitivity (REFSENS) is expected; and

- Case C (bottom): case of 35MHz UL CBW. For this case, the upper ACL shoulders overlap the n8 DL channel by approximately 25MHz. Strong impact on REFSENS is expected. No other impact is expected as agreed in [2].

Figure 1: n8 35MHz DL CBW spectrum perspective for A) 20MHz uplink CBW at highest channel (top), B) 20MHz uplink CBW at lowest channel (middle), C) 35MHz uplink CBW(bottom).

This paper presents measurement results to evaluate the impact of cases A, B and C on maximum sensitivity degradation (MSD) and A-MPR. Broader perspective on specification impact can be found in [3].

## Power Amplifier Calibration and Assumptions

- PA calibration point is 20 MHz, 15 kHz, QPSK, DFT-S-OFDM, 100 RB at lower channel edge with 1 dB MPR;

- Post PA losses: 4dB;

- Power Class 3 (PC3) operation;

- Local Oscillator (LO) leakage: -28dBc;

- IQ Image rejection: -28dB;

- C-IM3: -60dBc, C-IM5:-70dBc;

- Power amplifier is tested against 2 different supply voltages to account for large back-offs;

- Transmit (Tx) to Antenna n8 filter rejection at 717MHz: 9dB based on [1]; and

- Duplexer Tx to receiver (Rx) Tx noise rejection in Rx band: 55dB.

## Measurement Results

### Dataset for MSD Proposal

Case A: 35MHz DL CBW MSD due to 20MHz UL CBW at highest channel

Tx noise level in 35MHz DL channel measured with 20MHz UL LCRB =25 is -36.0 dBm.

Case B: 35MHz DL CBW MSD due to 20MHz UL CBW at lowest channel

Figure 3 shows measured 20MHz UL CBW Tx noise level in 35MHz DL CBW vs LCRB for the case when the UL channel is closest to the DL band.

Figure 3: 20MHz UL CBW Transmitter noise level in 35MHz DL CBW vs LCRB – Case B.

Tx noise level reaches a minimum for LCRB ranging from approximately 25 to 40 RB. We therefore propose to adopt LCRB =25 for MSD evaluation, value at which the transmitter noise level is -15.3 dBm, i.e. approximately 21dB higher than in Case A. This is expected as ACL shoulders overlap significantly the DL CBW.

Case C: 35MHz DL CBW MSD due to 35MHz UL CBW.

The measured 35MHz UL CBW Tx noise level in 35MHz DL CBW is plotted in Figure 4.

Figure 4: 35MHz UL CBW Transmitter noise level in 35MHz DL CBW vs LCRB – Case C.

Tx noise level reaches a minimum for LCRB ranging from approximately 20 to 60 RB. At LCRB =25, the noise level is -10.9 dBm, i.e. approximately 25dB higher than in Case A.

The estimated MSD for each case is shown in Table 3 where no A-MPR is assumed, i.e., the PA total transmit power at 0dB back-off is 27dBm. For 35MHz CBW, NRB is assumed equal to 188,92,44RBs for SCS15,30 and 60kHz respectively.

Table 3: n8 35MHz MSD for case A,B,C

The MSD are approximately 3dB for Case A, 17dB for Case B and 22dB for Case C.

Observation 1: For 20MHz UL CBW, the 35MHz DL CBW MSD ranges from 3dB (best case) to 17dB in the worst case of UL channel located closest to the downlink band. For 35MHz UL CBW, the 35MHz DL CBW MSD is approximately 22dB.

Since MSD Case A adds no further degradation to the baseline 20MHz UL / 20MHz DL operation, we propose to retain only 2 MSD test points, those corresponding to Case B and Case C. For the introduction of MSD test points, we propose that the support of 35 MHz and 45 MHz Channel bandwidths is optional for Band n8 and should use a separate table for band specific requirements [3]. This applies to n8 35MHz REFSENS specifications and considering the optionality aspects of this configuration, we propose to minimize the impact of technical specifications by capturing MSD Test Points in a single table format – see Proposal 1.

Proposal 1:

Table 7.3.2-1: Two Antenna Port Reference Sensitivity QPSK PREFSENS for 35MHz Channel bandwidth.

| Operating band / SCS / Channel bandwidth / Channel allocations / REFSENS/ Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | Channel bandwidth (DL) (MHz) | Channel bandwidth (UL) (MHz) | FC (DL) (MHz) | FC (UL) (MHz) | UL allocation (LCRB) | REFSENS (dBm) | Duplex Mode |
| n8 | 15 | 35 | 20 | 942.5 | 905 | 252 | -70.6 | FDD |
|  | 30 |  |  |  |  | 102 | -70.7 |  |
|  | 60 |  |  |  |  |  |  |  |
| n8 | 15 | 35 | 35 | 942.5 | 897.5 | 252 | -66.3 | FDD |
|  | 30 |  |  |  |  | 102 | -66.4 |  |
|  | 60 |  |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 2: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1) |  |  |  |  |  |  |  |  |

# Conclusion

In this contribution, we make proposals n8 35MHz CBW MSD test points with Proposal 1.

Proposal 1:

Table 7.3.2-1: Two Antenna Port Reference Sensitivity QPSK PREFSENS for 35MHz Channel bandwidth.

| Operating band / SCS / Channel bandwidth / Channel allocations / REFSENS/ Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | Channel bandwidth (DL) (MHz) | Channel bandwidth (UL) (MHz) | FC (DL) (MHz) | FC (UL) (MHz) | UL allocation (LCRB) | REFSENS (dBm) | Duplex Mode |
| n8 | 15 | 35 | 20 | 942.5 | 905 | 252 | -70.6 | FDD |
|  | 30 |  |  |  |  | 102 | -70.7 |  |
|  | 60 |  |  |  |  |  |  |  |
| n8 | 15 | 35 | 35 | 942.5 | 897.5 | 252 | -66.3 | FDD |
|  | 30 |  |  |  |  | 102 | -66.4 |  |
|  | 60 |  |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 2: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1) |  |  |  |  |  |  |  |  |

# References

[1]		RP-201722 WID revision: Introduction of channel bandwidths 35MHz and 45 MHz Huawei, HiSilicon, RAN#89e.

[2] 	R4-2011793, WF on UE RF requirements, 	Skyworks Solutions Inc., ZTE, Huawei, Qualcomm, RAN4#96e.

[3] 	R4-2015800, Specification impact of additional 35 and 45MHz channel bandwidths, Skyworks Solutions, Inc., 3GPP TSG-RAN WG4 Meeting #97e.