3GPP TSG-RAN WG4 #97-e		                              	  R4-2016027

Online, November 2nd – 13th, 2020

Source: 	Skyworks Solutions, Inc.

Title: 	n7 35MHz A-MPR and MSD Measurements

Agenda Item:	10.22.3 - UE RF requirements [NR_FR1_35MHz_45MHz_BW-Core]

Document for:	Approval

# Introduction

A new WI to study the introduction of 35 and 45 MHz channel bandwidth (CBW) has been agreed [1] and study assumptions agreed in WF [2]. This contribution brings measurement data and makes A-MPR and MSD TP proposals for n7.

# Discussion

## Establishing 35MHz AMPR from interpolation

Since band n7 NS_46 A-MPR is already established for 30 and 40 MHz, it is feasible to derive the 35 MHz A-MPR A5-A8 regions from Table 6.2.3.17-1 while using values from extracted from Table 6.2.3.17-2 in 38.101-1 specification and reproduced below:

Table 6.2.3.17-1: A-MPR regions for NS_46

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 15 MHz | 2558.5 ≤ FC ≤ 2562.5 |  | Note 1 | A1 |
| 20 MHz | 2547 ≤ FC ≤ 2560 |  | Note 1 | A2 |
| 25 MHz | 2534.5 ≤ FC < 2557.5 |  | Note 1 | A3 |
| 30 MHz | 2515 ≤ FC ≤ 2555 | ≥0, <1.44 | >0 | A4 |
|  |  | ≥1.44, <13.5 | >max (0, 12*SCS*RBend -1.8) | A5 |
|  |  | ≥13.5, <19.8 | >11.52 | A6 |
|  |  | ≥19.8, <25.92 | >6.3 | A7 |
|  |  | ≥25.92 | >0 | A8 |
| 40 MHz | 2520 ≤ FC ≤ 2550 | ≥0, <4.14 | >0 | A4 |
|  |  | ≥4.14, <18 | >max (0, 12*SCS*RBend - 4.5) | A5 |
|  |  | ≥18, <25.74 | >13.5 | A6 |
|  |  | ≥25.74, <32.4 | >12.6 | A7 |
|  |  | ≥32.4 | >0 | A8 |
| 50 MHz | 2525 ≤ FC ≤ 2545 | ≥0, <9 | >0 | A4 |
|  |  | ≥9, <21.6 | >max (0, 12*SCS*RBend - 7.2) | A5 |
|  |  | ≥21.6, <31.5 | >18 | A6 |
|  |  | ≥31.5, <39.6 | >16.2 | A7 |
|  |  | ≥39.6 | >0 | A8 |
| 50 MHz | 2525 ≤ FC ≤ 2545 | ≥0, <9 | >0 | A4 |
|  |  | ≥9, <21.6 | >max (0, 12*SCS*RBend - 7.2) | A5 |
|  |  | ≥21.6, <31.5 | >18 | A6 |
|  |  | ≥31.5, <39.6 | >16.2 | A7 |
|  |  | ≥39.6 | >0 | A8 |
| NOTE 1: 9.72 MHz for DFT-s-OFDM, > 16.02 MHz for CP-OFDM. CP-OFDM threshold only applies for 20 and 25 MHz bandwidths. |  |  |  |  |

Table 6.2.3.17-2: A-MPR for NS_46

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 | A7 | A8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Outer | Outer | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 3.5 | 4 | 4.5 | 5 | 2 | 3.5 | 6 | 10 |
|  | QPSK | 3.5 | 4 | 4.5 | 5 | 2 | 3.5 | 6 | 10 |
|  | 16 QAM | 3.5 | 4 | 4.5 | 5 | 2 | 3.5 | 6 | 10 |
|  | 64 QAM | 3.5 | 4 | 4.5 | 5 |  | 3.5 | 6 | 10 |
|  | 256 QAM |  |  |  |  |  |  | 6 | 10 |
| CP-OFDM | QPSK | 3.5 | 5.5 | 6 | 5 | 3.5 | 5.5 | 7 | 11 |
|  | 16 QAM | 3.5 | 5.5 | 6 | 5 | 3.5 | 5.5 | 7 | 11 |
|  | 64 QAM | 3.5 | 5.5 | 6 | 5 | 3.5 | 5.5 | 7 | 11 |
|  | 256 QAM |  |  | 6 |  |  |  | 7 | 11 |

The interpolated values from the 30, 40 and 50 MHz data points are presented in Table 1

Table 1: proposed values for n7 35 MHz channel bandwidth NS_46 AMPR region definition

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 35 MHz | 2517.5 ≤ FC ≤ 2552.5 | ≥0, <[2.7] | >0 | A4 |
|  |  | ≥[2.7], <[15.84] | >max (0, 12*SCS*RBend –[3.06]) | A5 |
|  |  | ≥[15.84], <[22.68] | >[12.6] | A6 |
|  |  | ≥[22.68], <[28.8] | >[9.36] | A7 |
|  |  | ≥[28.8] | >0 | A8 |

Proposal 1: values in table 1 are adopted for NS_46 for the definition of A-MPR regions associated with 35 MHz channel bandwidth.

## Power Amplifier Calibration and Assumptions

- PA calibration point is 20 MHz, 15 kHz, QPSK, DFT-S-OFDM, 100 RB at lower channel edge with 1 dB MPR;

- Post PA losses: 4dB;

- Power Class 3 (PC3) operation;

- Local Oscillator (LO) leakage: -28dBc;

- IQ Image rejection: -28dB;

- C-IM3: -60dBc, C-IM5:-70dBc;

## MSD Measurement Results

Data is extracted from previous measurements reported in [4] for 30,40, and 50MHz channel BW. Recalling data from [4] where 4dB post-PA loss, PA is operated at 27dBm, with ACLR linearity calibrated at 1dB MPR. An example plot of the PA output spectrum at 27 dBm for 50MHz channel bandwidth is presented in Figure 2.

Figure 2: Example of spectrum observed at PA output port at 27 dBm for 50MHz CBW operation with C-IM3.

The measured Tx noise in Rx band power levels in each respective bandwidth are:

- 30 MHz: -55.1 dBm

- 40 MHz: -51.9 dBm

- 50 MHz: -37.3 dBm.

Resulting in MSD ranging from <0.5 dB for 30 and 40MHz and 5dB for 50MHz as shown in Table 2.

Table 2: n7 30,40,50MHz MSD [4]

Observation 1:

- At 30 MHz and 40 MHz CBW, MSD can be neglected. So 35MHz it is reasonable to propose no MSD for 35MHz operation.

- At 50 MHz, worst case MSD is 5 dB. After off line discussions in R4#92, it was agree to specify 6.2dB MSD.

Considering no MSD is agreed at 30 and 40MHz CBW, no MSD is proposed for 35MHz CBW.

Proposal 2:

Table 7.3.2-1: Two Antenna Port Reference Sensitivity QPSK PREFSENS

| Operating Band | SCS kHz | 35 MHz (dBm) | Duplex Mode |
| --- | --- | --- | --- |
| n71 | 15 | -89.2 | FDD |
|  | 30 | -89.3 |  |
|  | 60 | -89.4 |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. |  |  |  |

Table 7.3.2-3: Uplink Configuration for Reference Sensitivity (from 38.101-1)

| Operating Band | SCS kHz | 35 MHz (dBm) | Duplex Mode |
| --- | --- | --- | --- |
| n71 | 15 | 451 | FDD |
|  | 30 | 201 |  |
|  | 60 | 101 |  |
| NOTE 1: 1 Refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). |  |  |  |

# Conclusion

In this contribution, we make proposals n7 35-45MHz A-MPR and MSD test points with Proposal 1 and 2.

Proposal 1: values in table 1 are adopted for NS_46 for the definition of A-MPR regions associated with 35 MHz channel bandwidth.

Table 1: proposed values for n7 35 MHz channel bandwidth NS_46 AMPR region definition

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 35 MHz | 2517.5 ≤ FC ≤ 2552.5 | ≥0, <[2.7] | >0 | A4 |
|  |  | ≥[2.7], <[15.84] | >max (0, 12*SCS*RBend –[3.06]) | A5 |
|  |  | ≥[15.84], <[22.68] | >[12.6] | A6 |
|  |  | ≥[22.68], <[28.8] | >[9.36] | A7 |
|  |  | ≥[28.8] | >0 | A8 |

Proposal 2:

Table 7.3.2-1: Two Antenna Port Reference Sensitivity QPSK PREFSENS

| Operating Band | SCS kHz | 35 MHz (dBm) | Duplex Mode |
| --- | --- | --- | --- |
| n71 | 15 | -89.2 | FDD |
|  | 30 | -89.3 |  |
|  | 60 | -89.4 |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. |  |  |  |

Table 7.3.2-3: Uplink Configuration for Reference Sensitivity (from 38.101-1)

| Operating Band | SCS kHz | 35 MHz (dBm) | Duplex Mode |
| --- | --- | --- | --- |
| n71 | 15 | 451 | FDD |
|  | 30 | 201 |  |
|  | 60 | 101 |  |
| NOTE 1: 1 Refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). |  |  |  |

# References

[1]		RP-201722 WID revision: Introduction of channel bandwidths 35MHz and 45 MHz Huawei, HiSilicon, RAN#89e.

[2] 	R4-2011793, WF on UE RF requirements, 	Skyworks Solutions Inc., ZTE, Huawei, Qualcomm, RAN4#96e.

[3] 	R4-1909942, Experimental Measurements for n7 30,40,50 CBW REFSENS, Skyworks Solutions, Inc., 3GPP TSG-RAN WG4 Meeting #92.