3GPP TSG-RAN4 Meeting #97e	R4-2016052

Electronic Meeting, Nov. 2 - Nov.13, 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 38.133 CR on the test case of E-UTRAN-NR FR1 interruptions at NR SRS carrier switching |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_Enh-Perf |  |  |  |  |  | Date: |  |  | 2020-10-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The test case for E-UTRAN-NR FR1 interruptions at NR SRS carrier switching needs to be specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The test case for E-UTRAN-NR FR1 interruptions at NR SRS carrier switching is provided. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The performance of SRS carrier switching is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New sections A.4.5.2. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

#### A.4.5.2.x	E-UTRAN - NR FR1 interruptions at NR SRS carrier based switching in asynchronous EN-DC

#### A.4.5.2.x.1	Test Purpose and Environment

The purpose of this test is to verify that when a UE needs to transmit periodic SRS, the UE can perform carrier based switching to one carrier not configured for PUCCH/PUSCH transmission from a CC with PUCCH/PUSCH transmission. The test will verify the interruption requirements on E-UTRAN PCell and NR PSCell in clause 8.2.1.2.12. Supported test configurations are shown in table A.4.5.2.x.1-1.

The general test parameters and NR cell specific test parameters are given in Table A.4.5.2.x.1-2 and A 4.5.2.x.1-3 below. And the E-UTRAN cell specific test parameters can refer to Table A.3.7.2.1-1. In the test there are three cells: Cell1, Cell2 and Cell3. Cell1 is E-UTRAN PCell, Cell2 is NR PSCell in FR1 with PUCCH/PUSCH transmission, Cell3 is an activated NR SCell in FR1 which operates in downlink without PUCCH/PUSCH transmission. The UE is configured with the SRS carrier based switching between PSCell and SCell.

The test consists of two successive time periods, with duration of T1 and T2, respectively. During T1 the UE shall be continuously scheduled on PCell and PSCell. Immediately at the beginning of T2, a PDCCH with TPC-SRS-RNTI is sent to the UE to initiate NR SRS switching.

Table A.4.5.2.x.1-1: Interruptions at SRS carrier switching supported test configurations in FR1

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, PSCell FDD duplex mode, SCell TDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, PSCell TDD duplex mode, SCell TDD duplex mode |
| 3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, PSCell FDD duplex mode, SCell TDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, PSCell TDD duplex mode, SCell TDD duplex mode |
| 6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.4.5.2.x.1-2: General test parameters for E-UTRAN – NR FR1 interruptions at SRS carrier based switching in asynchronous EN-DC

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2, 3 | One is E-UTRAN RF channel and the other two are NR RF channels |
| Active PCell |  | Cell1 | PCell on E-UTRAN RF channel number 1. |
| Configured PSCell |  | Cell2 | Configured PSCell on NR RF channel number 2. |
| Configured SCell |  | Cell3 | Configured activated secondary cell on NR RF channel number 3. |
| CP length |  | Normal | Applicable to Cell1, Cell2 and Cell3. |
| DRX |  | OFF | Continuous monitoring of primary cell |
| Filter coefficient |  | 0 | L3 filtering is not used |
| T1 | s | 5 |  |
| T2 | ms | 40 | UE shall perform SRS switching during T2 |

Table A.4.5.2.x.1-3: NR Cell specific test parameters for E-UTRAN – NR FR1 interruptions at SRS carrier based switching in asynchronous EN-DC

| Parameter |  | Unit | Cell2 | Cell3 |
| --- | --- | --- | --- | --- |
| Frequency Range |  |  | FR1 | FR1 |
| Duplex mode | Config 1,4 |  | FDD | TDD |
|  | Config 2,3,5,6 |  | TDD | TDD |
| TDD configuration | Config 1,4 |  | Not Applicable | TDDConfig.1.1 |
|  | Config 2,5 |  | TDDConf.1.1 | TDDConfig.1.1 |
|  | Config 3,6 |  | TDDConf.2.1 | TDDConfig.2.1 |
| BWchannel | Config 1,2,4,5 | MHz | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  | Config 3,6 |  | 40: NRB,c = 106 | 40: NRB,c = 106 |
| DL Initial BWP configuration | Config 1-6 |  | DLBWP.0.1 | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1-6 |  | DLBWP.1.1 | DLBWP.1.1 |
| UL Initial BWP configuration | Config 1-6 |  | ULBWP.0.1 | - |
| UL dedicated BWP configuration | Config 1-6 |  | ULBWP.1.1 | - |
| PDSCH Reference measurement channel | Config 1,4 |  | SR.1.1 FDD | SR.1.1 TDD |
|  | Config 2,5 |  | SR.1.1 TDD | SR.1.1 TDD |
|  | Config 3,6 |  | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET Reference Channel | Config 1,4 |  | CR.1.1 FDD | CR.1.1 TDD |
|  | Config 2,5 |  | CR.1.1 TDD | CR.1.1 TDD |
|  | Config 3,6 |  | CR.2.1 TDD | CR.2.1 TDD |
| RMC CORESET Reference Channel | Config 1,4 |  | CCR.1.1 FDD | CCR.1.1 TDD |
|  | Config 2,5 |  | CCR.1.1 TDD | CCR.1.1 TDD |
|  | Config 3,6 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| OCNG Patterns |  |  | OP.1 | OP.1 |
| TRS configuration | Config 1,4 |  | TRS.1.1 FDD | TRS.1.1 TDD |
|  | Config 2,5 |  | TRS.1.1 TDD | TRS.1.1 TDD |
|  | Config 3,6 |  | TRS.1.2 TDD | TRS.1.2 TDD |
| SMTC configuration |  |  | SMTC.1 | SMTC.1 |
| SSB configuration | Config 1,2,4,5 |  | SSB.1 FR1 | SSB.1 FR1 |
|  | Config 3,6 |  | SSB.2 FR1 | SSB.2 FR1 |
| PDSCH/PDCCH subcarrier spacing | Config 1,2,4,5 | kHz | 15 kHz | 15 kHz |
|  | Config 3,6 |  | 30 kHz | 30 kHz |
| PUCCH/PUSCH subcarrier spacing | Config 1,2,4,5 | kHz | 15 kHz | - |
|  | Config 3,6 |  | 30 kHz | - |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| Note2 |  | dBm/15kHz | -104 | -104 |
| Note2 | Config 1,2,4,5 | dBm/SCS | -104 | -104 |
|  | Config 3,6 |  | -101 | -101 |
| SS-RSRPNote3 | Config 1,2,4,5 | dBm/SCS | -87 | -87 |
|  | Config 3,6 |  | -84 | -84 |
|  |  | dB | 17 | 17 |
|  |  | dB | 17 | 17 |
| IoNote3 | Config 1,2,4,5 | dBm/ 9.36MHz | -58.96 | -58.96 |
|  | Config 3,6 | dBm/ 38.16MHz | -52.86 | -52.86 |
| Time offset to Cell1 Note 5 |  | s | - | 3 |
| Propagation condition |  | - | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: Receive time difference between slot boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |  |

Table A.4.5.2.x.1-4: Sounding Reference Symbol Configuration for E-UTRAN – NR FR1 interruptions at SRS carrier based switching in asynchronous EN-DC

| Field |  | Value | Comment |
| --- | --- | --- | --- |
| c-SRS | Config 1,2,4,5 | 12 | Frequency hopping is disabled |
|  | Config 3,6 | 24 |  |
| b-SRS |  | 0 |  |
| b-hop |  | 0 |  |
| freqDomainPosition |  | 0 | Frequency domain position of SRS |
| freqDomainShift |  | 0 |  |
| groupOrSequenceHopping |  | neither | No group or sequence hopping |
| SRS-PeriodicityAndOffset |  | sl5=2 for SCS 15kHz sl5=4 for SCS 30kHz | Once every 5 slots |
| pathlossReferenceRS |  | ssb-Index=0 | SSB #0 is used for SRS path loss estimation |
| usage |  | Codebook | Codebook based UL transmission |
| startPosition |  | 0 | resourceMapping setting. SRS on last |
| nrofSymbols |  | n1 | symbol of slot, and 1symbols for SRS |
| repetitionFactor |  | n1 | without repetition. |
| combOffset-n2 |  | 0 | transmissionComb setting |
| cyclicShift-n2 |  | 0 |  |
| nrofSRS-Ports |  | port1 | Number of antenna ports used for SRS transmission |
| Note: For further information see clause 6.3.2 in TS 38.331 [2]. |  |  |  |

#### A.4.5.2.x.2	Test Requirements

The UE shall be continuously scheduled in E-UTRAN PCell and NR PSCell during the entire length of T1. During the time duration T1 the UE shall transmit at least 99.5% of ACK/NACK on NR PSCell.

Each interruption on NR PSCell during the switching from NR PSCell to NR SCell shall not exceed the value as defined in Table A.4.5.2.x.2-1 dependent on the applied SRS carrier switching time.

Table A4.5.2.x.2-1: Interruption length on NR active serving cells at NR SRS carrier switching (slot)

|  | NR Slot length | SRS carrier | Interruption length X1 (slots) |  |
| --- | --- | --- | --- | --- |
|  | (ms) of victim cell | switching time (us)Note 1 | Sub carrier spacing for agressor cell (kHz) |  |
|  |  |  | 15 | 30 |
| 0 | 1 | ≤ 200 | 2 | 2 |
|  |  | 300, 500 | 2 | 2 |
|  |  | 900 | 3 | 3 |
| 1 | 0.5 | ≤ 200 | 3 | 2 |
|  |  | 300, 500 | 3 | 3 |
|  |  | 900 | 4 | 4 |
| Note1: NR SRS carrier switching time is UE capability indicated by higher layer parameter SRS-SwitchingTimeNR. |  |  |  |  |

Each interruption on E-UTRAN PCell during the switching from NR PSCell to NR SCell shall not exceed the value as defined in Table A.4.5.2.x.2-2 dependent on the applied SRS carrier switching time.

Table 4.5.2.x.2-2: Interruption length on E-UTRAN active serving cells at NR SRS carrier switching

| NR SRS carrier switching time (us)note1 | Interruption length X1 (subframes) |
| --- | --- |
| ≤500 | 2 |
| 900 | 3 |
| Note1: NR SRS carrier switching time is UE capability indicated by higher layer parameter SRS-SwitchingTimeNR. |  |